Neodymium Iron Boron (NdFeB) Magnet Rev 1.0 SDS

Publisher: Allstar Magnetics

Source: https://resds.com/document/neodymium-iron-boron-ndfeb-magnet-en-5e27a101ff

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

Neodymium Iron Boron (NdFeB) Magnet

Section 1: Identification

1.1. Product Identifier

Product Form: Mixture

Product Name: : Neodymium Iron Boron (NdFeB) Magnet, AKA: Rare Earth Magnet

1.2. Intended Use of Product: Permanent magnet for various uses.

1.3. Name, Address, and Telephone of the Responsible Party:

Allstar Magnetics

15100 NE 65th Street, Suite 170

Vancouver, WA 98682

1-800-949-8950

https://allstarmagnetics.com

1.4 Emergency Telephone Number: Within USA and Canada 1-800-424-9300 or +1 703-527-3887 (collect calls accepted)

Section 2: Hazard(s) Identification

2.1. Classification of the Substance or Mixture Classification (GHS-US)

Not classified

2.2. Label Elements GHS-US Labeling

No labeling applicable

2.3. Other Hazards

Under normal conditions of use and handling in the solid form, harmful substances cannot be released, nor is the solid metal

piece considered flammable. Much of the information provided in this SDS is for situations of use in which hazardous exposures

may occur, such as in welding applications or for metals in powder form.

Magnets have very strong magnetic forces which make them attract to other magnets and other ferromagnetic materials such

as iron or steel. This may result in injury during handling of magnets.

Strong magnets may affect the operation of pacemakers or other implanted medical devices.

2.4. Unknown Acute Toxicity (GHS-US)

No data available

Section 3: Composition/ Information on Ingredients

3.1. Substance

Neodymium iron boron

3.2. Chemical Formula: Nd2Fe14B

Section 4: First-Aid Measures

4.1.     Description of First Aid Measures

First-aid Measures General:  If medical advice is needed, have product SDS at hand.

First-aid Measures After Inhalation:  If inhaled, move to fresh air and keep at rest in a position comfortable for breathing. Obtain

medical attention if breathing difficulty persists.

First-aid Measures After Skin Contact: Wash immediately with plenty of soap and water. Rinse with plenty of water. Remove

contaminated clothing. Wash contaminated clothing before reuse. Obtain medical attention if irritation develops or persists.

First-aid Measures After Eye Contact: Immediately rinse with water for a prolonged period (at least 15 minutes) while holding

the eyelids wide open. Remove contact lenses, if present and easy to do. Continue rinsing. Obtain medical attention if irritation

develops or persists.

First-aid Measures After Ingestion:  If swallowed, do not induce vomiting: seek medical advice immediately and show this SDS.

4.2.    Most important symptoms and effects, both acute and delayed

Symptoms/Injuries: The primary acute health hazard associated with this product would be the potential for exposure to fumes

during metal processing operations. Under normal conditions of use not expected to present a significant hazard. During

processing or physical alteration, flakes or powder cause irritation of the respiratory tract, eyes, skin, and are harmful. Molten

SDS NEO, Rev 1.0, 08Feb2024                                                       Page 1 of 6

Page 2

material may release toxic, and irritating fumes.

Symptoms/Injuries After Inhalation:  During processing, inhalation of fumes may cause lung inflammation and injury with

symptoms of chest pains, chills, cough, headache, and diarrhea. Inhalation of dusts and fumes can cause metal fume fever.

Symptoms can include a metallic or sweet taste in the mouth, sweating, shivering, headache, throat irritation, fever, chills,

thirstiness, muscle aches, nausea, vomiting, weakness, fatigue, and shortness of breath.

Symptoms/Injuries After Skin Contact: During metal processing, . Dust may cause irritation in skin folds or by contact in

combination with tight clothing.

Symptoms/Injuries After Eye Contact: Dust generated from material cutting may cause a slight irritation. Slivers may be

generated, which could cause mechanical irritation or injure the eye.

Symptoms/Injuries After Ingestion:  If a large quantity has been ingested: Gastrointestinal irritation.

Indication of Any Immediate Medical Attention and Special Treatment Needed

If medical advice is needed, have product SDS at hand.

Information for Doctors: Ingestion of multiple magnets can pose a serious risk. Consider consulting the algorithm presented in,

"Management of Ingested Magnets in Children," (Hussain et al., 2012). Strong magnetic fields found near neodymium magnets

can interfere with the operation of implanted medical devices such as pacemakers and ICDs.

Section 5: Fire-Fighting Measures

5.1. Extinguishing Media

Suitable Extinguishing Media: Earth, sand, Class D dry powder extinguishing agent, without oxygen compounds.

Unsuitable Extinguishing Media: Do not use Halon agents or water.

5.2. Special Hazards Arising From the Substance or Mixture

Fire Hazard: Dry powders or neodymium magnets will oxidize, smolder, and burn rapidly in the presences of air or oxygen.

Maintain powders in water slurry or in inert atmospheres of nitrogen or argon to prevent spontaneous combustion. Magnets

may spark on impact. Handle carefully in explosive atmospheres.

Explosion Hazard: Product itself is not explosive but if dust is generated, dust clouds suspended in air can be explosive.

Reactivity: Hazardous reactions will not occur under normal conditions.

5.3. Advice for Firefighters

Firefighting Instructions: Do not breathe fumes from fires or vapors from decomposition. Keep upwind.

Protection During Firefighting: Firefighters must use full-face, self-contained breathing apparatus with full protective clothing

to prevent contact with skin and eyes to protect against potential hazardous combustion and decomposition products.

Section 6: Accidental Release Measures

6.1. Personal Precautions, Protective Equipment and Emergency Procedures

6.1.1. For Non-emergency Personnel

Protective Equipment: Wear eye protection.

Emergency Procedures: Avoid creating or spreading dust. Eliminate ignition sources.

6.1.2. For Emergency Responders

Protective Equipment: Safety glasses.

Emergency Procedures: Ventilate area. Eliminate ignition sources. Evacuate unnecessary personnel.

6.2. Environmental Precautions

Contain any spills with dikes or absorbents to prevent migration and entry into sewers or streams.

6.3. Methods and Material for Containment and Cleaning Up

For Containment: Contain and collect as any solid.

Methods for Cleaning Up: Avoid generation of dust during clean-up of spills. Take up mechanically (sweeping, shoveling) and

collect in suitable container for disposal. Vacuum must be fitted with HEPA filter to prevent release of particulates during clean-

up. Use only non-sparking tools. Use explosion-proof equipment.

6.4. Reference to Other Sections

See heading 8, Exposure Controls and Personal Protection.

Section 7: Handling and Storage

7.1.  Precautions for Safe Handling

Large magnets can attract to one another. Strong attractive forces can cause injury. Impacts of magnets can eject chips or bits

of magnet material at speed - eye protection should be used. Strong magnetic fields may affect the operation of implanted

medical devices such as pacemakers and ICDs.

SDS NEO, Rev 1.0, 08Feb2024                                                       Page 2 of 6

Page 3

Additional Hazards When Processed: Do not handle until all safety precautions have been read and understood. Fine dust

dispersed in air may ignite. Dust deposits should not be allowed to accumulate on surfaces, as these may form an explosive

mixture if they are released into the atmosphere in sufficient concentration.

Precautions for Safe Handling: Do not breathe dust. Do not get in eyes, on skin, or on clothing. Avoid creating or spreading dust.

Always wash hands after handling the product. Do not eat, drink or smoke when using this product. Ensure there is adequate

ventilation. Wear recommended personal protective equipment.

Hygiene Measures: Handle in accordance with good industrial hygiene and safety procedures. Always wash your hands

immediately after handling this product, and once again before leaving the workplace. Wash contaminated clothing before

reuse. Do not eat, drink or smoke in areas where product is used. Emergency eye wash fountains and safety showers should be

available in the immediate vicinity of any potential exposure.

Information about protection against Explosions/Fires: No particular measures are required in the provided form.

7.2.  Conditions for Safe Storage, Including Any Incompatibilities

Storage Conditions: Store in dry location free of corrosive atmosphere. Keep away from magnetic objects such as iron, cobalt or

nickel and high energy magnetic fields.

Incompatible Materials: Avoid contact with: Strong acids. Mineral acids. Corrosive substances in contact with metals may

produce flammable hydrogen gas.

7.3.  Specific End Use(s) Cast and Sintered Permanent Magnets

Section 8: Exposure Controls/Personal Protection

8.1. Control Parameters

Exposure Limits: Neodymium

OSHA/PEL: No exposure limit established

ACGIH/TLV: No exposure limit established

8.2. Exposure Controls

Appropriate Engineering Controls: Handle in a humidity controlled atmosphere. Handle in an enclosed, controlled process

under dry argon when possible. Ensure adequate ventilation to maintain exposures below occupational limits. Whenever

possible the use of local exhaust ventilation or other engineering controls is the preferred method of controlling exposure to

airborne dust and fume to meet established occupational exposure limits. Use good housekeeping and sanitation practices.

Do not use tobacco or food in work area. Wash thoroughly before eating or smoking. Do not blow dust off clothing or skin

with compressed air.

Personal Protective Equipment: Not generally required. The use of personal protective equipment may be necessary

as conditions warrant.

Hand Protection: Wear impermeable gloves, protective work clothing as necessary.

Eye Protection: Safety glasses.

Respiratory Protection: If permissible levels are exceeded, use NIOSH approved dust respirator.

Section 9: Physical and Chemical Properties

9.1. Information on Basic Physical and Chemical Properties

Physical State                                                                      :  Solid

Color                                                                                   :  Silver gray metallic

Odor                                                                                   : Not determined

Odor Threshold                                                                   : Not determined

pH                                                                                      : N/A

Evaporation Rate                                                                : N/A

Melting Point                                                                      : Above 1021°C (2192 - 2642 °F)

Freezing Point                                                                     : No data available

Boiling Point                                                                       : 3074°C

Flash Point                                                                          : N/A

Auto-ignition Temperature                                                  : No data available

Decomposition Temperature                                               : No data available

Flammability (solid, gas)                                                     : No data available

Vapor Pressure                                                                   : No data available

SDS NEO, Rev 1.0, 08Feb2024                                                       Page 3 of 6

Page 4

Relative Vapor Density at 20 °C                                           : No data available

Relative Density                                                                  : 7.008 g/cc

Density                                                                               : 7.5 g/cc

Solubility                                                                             : Water: Insoluble

Partition Coefficient: N-octanol/water                                 : No data available

Viscosity                                                                              : N/A

Explosive Properties                                                            : None.

Oxidizing Properties                                                            : None.

9.2. Other Information No additional information available

Section 10: Stability and Reactivity

10.1 Reactivity: No data.

10.2 Chemical Stability: Product is stable under recommended storage conditions.

10.3 Possibility of Hazardous Reactions: Contact with acids may evolve hydrogen gas. Dusts are flammable. May react with

water under fire conditions liberating flammable hydrogen gas.

10.4 Conditions to Avoid: Avoid exposure of powdered magnet material to air, oxygen or halogenated hydrocarbons and to

elevated temperatures above 150° C. Do not use or store in conditions as follows: acidic, alkaline or electrically

conductive liquids, corrosive gases.

10.5 Incompatible Materials:  Fine powders are incompatible with air, oxygen, halogenated hydrocarbons with strong

oxidizers. Avoid acids and other oxidizing agents.

10.6 Hazardous Decomposition Products: Neodymium oxides, neodymium hydroxides, hydrogen.

Section 11: Toxicological Information

11.1. Information On Toxicological Effects

Acute Toxicity: Not classified.

Skin Corrosion/Irritation: Not classified

Serious Eye Damage/Irritation: Not classified.

Respiratory or Skin Sensitization: Not classified. Not classified.

Germ Cell Mutagenicity: Not classified

Carcinogenicity: Not classified.

Reproductive Toxicity: Not classified.

Specific Target Organ Toxicity (Single Exposure): Not classified Specific Target Organ Toxicity (Repeated Exposure): Not

classified. Aspiration Hazard: Not classified

Symptoms/Injuries After Inhalation: During processing, inhalation of fumes may cause lung inflammation and injury with

symptoms of chest pains, chills, cough, headache, and diarrhea. Inhalation of dusts and fumes can cause metal fume fever.

Symptoms can include a metallic or sweet taste in the mouth, sweating, shivering, headache, throat irritation, fever, chills,

thirstiness, muscle aches, nausea, vomiting, weakness, fatigue, and shortness of breath.

Symptoms/Injuries After Skin Contact: During metal processing, . Dust may cause irritation in skin folds or by contact in

combination with tight clothing.

Symptoms/Injuries After Eye Contact: Dust generated from material cutting may cause a slight irritation. Slivers may be

generated, which could cause mechanical irritation or injure the eye.

Symptoms/Injuries After Ingestion: If a large quantity has been ingested: Gastrointestinal irritation.

Inhalation of Nickel compounds has been shown in studies to provide an increased incidence of cancer of the nasal cavity,

lung and possibly larynx in nickel refinery workers. Copper: Overexposure to fumes may cause metal fume fever (chills, muscle

aches, nausea, fever, dry throat, cough, weakness, lassitude); metallic or sweet taste; discoloration of skin and hair. Tissue

damage of mucous membranes may follow chronic dust exposure.

Section 12: Ecological Information (non-mandatory)

12.1. Toxicity: No further relevant information available.

12.2.  Persistence and Degradability: No further relevant information available.

12.3. Bio accumulative Potential: No further relevant information available.

12.4. Mobility in Soil: No further relevant information available.

12.5. Other Adverse Effects

No additional information available

SDS NEO, Rev 1.0, 08Feb2024                                                       Page 4 of 6

Page 5

Section 13: Disposal Considerations (non-mandatory)

13.1. Waste treatment methods

Sewage Disposal Recommendations: Do not empty into drains; dispose of this material and its container in a safe way.

Waste Disposal Recommendations: Dispose of waste material in accordance with all local, regional, national, and international

regulations.

Section 14: Transport Information (non-mandatory)

Magnets can generate magnetic fields that may affect navigation equipment. Magnets are able to attract ferromagnetic

materials.

Land Transport: Not regulated for transport.

Transport by Sea: Not regulated for transport

Air Transport: Neo magnets may or may not require a hazardous material label. See the International Air Transport

Association’s (IATA) Dangerous Goods Regulations (DGR) and FAA Title 49, Part 173.21

Section 15: Regulatory Information (non-mandatory)

15.1 US Federal Regulations

Safety, health and environmental regulations/legislation specific for the substance or mixture

SARA Section 355 (extremely hazardous              None of the ingredients is listed

substances):

Section 313 (Specific toxic chemical listings)              7440-48-4                       Cobalt

7429-90-5                   Aluminum

TSCA (Toxic Substances Control Act):                           All ingredients are listed

Proposition 65 (chemicals known to cause cancer)        7440-48-4                       Cobalt

Chemicals known to cause reproductive toxicity          Females: None of the ingredients is listed

Males: None of the ingredients is listed

Chemicals known to cause developmental toxicity       None of the ingredients is listed

Cancerogenity categories

EPA (Environmental Protection Agency)               None of the ingredients is listed

TLV (Threshold Limit Value established by ACGIH)         7440-48-4                       Cobalt       A3

7429-90-5                   Aluminum    A4

MAK (German Maximum Workplace                     7440-48-4                       Cobalt       2

Concentration)

NIOSH-Ca (National Institute for Occupational          None of the ingredients is listed

Safety and Health)

OSHA-Ca (Occupational Safety & Health              None of the ingredients is listed

Administration

National regulations:

Other regulations, limitations and prohibitive regulations  Guidelines 67/548/ECC, 1999/45/EC 1272/2008/EG (CLP)

1907/2006/EG (REACH)

German Hazardous Substances

PLEASE NOTE:                                       Magnetized parts generate magnetic fields and are able to attract

magnetizable materials. This may result in injury during handling of

magnets. Electronic devices and measure tools may be changed in

calibration or damaged by the high magnetic field strength. Please

keep magnetized magnets away from computers, displays and

magnetic storage devices. Especially people with heart pacemakers

must keep away from magnetic

fields.

Chemical Safety Assessment                        VOID (for articles)

SDS NEO, Rev 1.0, 08Feb2024                                                       Page 5 of 6

Page 6

Section 16: Other Information

Revision Date                                                               : 09/02/2014

The information is based on our present knowledge. However, this shall not constitute a guarantee for any specific

product features and shall not establish a legally valid contractual relationship.

Relevant phrases (Wording of safety instructions      H22    Flammable solid

quoted <section 3> concerning pure substances     8       Catches fire spontaneously if exposed to

(powder).                                   H25      air In contact with water releases

0       flammable gas Cause skin irritation

H26    May cause an allergic skin reaction

1       Cause serious eye irritation

H31    May cause allergy or asthma symptoms or breathing

5          difficulties

H31

7

H31

9

H33

4

R11     Highly flammable

R15     Contact with water liberates extremely flammable

R17     gases Spontaneously flammable in air

R36/3    Irritating to eyes and skin

8      May cause sensitization by inhalation and skin contact

R42/4  May cause long-term adverse effects in aquatic

3 R53   environment.

Department issuing MSDS

Contact

Abbreviations and acronyms                     IMDG: International Maritime Code for Dangerous Goods

IATA : International Air Transport Association

ACGIH : American Conference of Governmental Industrial

Hygienists

LC60: Lethal Concentration, 50%

LD50: Lethal Dose, 50 %

Sources

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and

environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.

SDS NEO, Rev 1.0, 08Feb2024                                                       Page 6 of 6