Neodymium oxide 2656 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/neodymium-oxide-prochem-en-93a817835e

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1  PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Neodymium oxide

PRODUCT NO:       2656

CAS NO:             1313-97-9

SYNONYMS:        Neodymium (III) oxide, neodymium sesquioxide

MANUFACTURER:

ProChem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL 61109                                      TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2  HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Not a dangerous substance according to the GHS

HMIS CLASSIFICATION:

Health: 0            Flammability: 0                  Reactivity: 0

NFPA RATING:

Health: 0              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: May cause eye irritation.

SKIN CONTACT: May be harmful if absorbed through skin. May cause skin irritation.

INHALATION: May be harmful if inhaled. May cause respiratory tract irritation.

INGESTION: May be harmful if swallowed.

SECTION 3   COMPOSITION / INFORMATION ON INGREDIENTS

Formula:  Nd2O3

Molecular Weight:  336.48

CHEMICAL NAME                CAS#              %               ACGIH (TWA)            OSHA (PEL)

Neodymium oxide                         1313-97-9                    100               No data                   No data

NEODYMIUM OXIDE                                                                          PAGE 1

Page 2

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least

15 minutes. Get medical attention if irritation occurs.

SKIN EXPOSURE: Wash with soap and water. Get medical attention if irritation develops.

INHALATION: If inhaled, removed to fresh air.  If not breathing, give artificial respiration.  If breathing is difficult, give oxygen.

Get medical attention.

INGESTION: Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an

unconscious person. Loosen tight clothing, such as a collar, tie, belt or waistband. Get medical attention if symptoms appear.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: Not applicable

EXPLOSION LIMITS: Not applicable

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES:   If involved in a fire, fire fighters should be equipped with a NIOSH approved positive

pressure self-contained breathing apparatus and full protective clothing.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: None

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear personal protective equipment. Avoid dust formation. Ensure adequate ventilation.

ENVIRONMENTAL PRECAUTIONS: Do not let product enter drains.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Use appropriate tools to put the spilled solid in a

convenient waste disposal container. Finish cleaning by spreading water on the contaminated surface and dispose of according to

local and regional authority requirements.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Do not ingest or breathe dust. Keep away from incompatibles such as oxidizing agents,

acids, and moisture.

CONDITIONS FOR SAFE STORAGE: Keep in a tightly closed container, in a cool and well-ventilated area. Hygroscopic.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when

handling a chemicals substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

VENTILATION:  If possible, handle the material in an efficient fume hood.

RESPIRATOR:   If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9   PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Light blue solid

MELTING POINT: 2270° C

BOILING POINT: 3760° C

MOLECULAR WEIGHT: 336.48

ODOR: None

SPECIFIC GRAVITY: No data available

VAPOR PRESSURE: Not applicable

SOLUBILITY IN WATER: Insoluble

NEODYMIUM OXIDE                                                                          PAGE 2

Page 3

SECTION 10   STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: None

INCOMPATIBILITY: Strong oxidizing agents

DECOMPOSITION PRODUCTS: None

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: LD50 Oral – rat - > 5,000 mg/kg

CARCINOGENIC EFFECTS: No component of this product present at levels greater than or equal to 0.1% is identified as probable,

possible or confirmed human carcinogen.

MUTAGENIC EFFECTS: No data

TETRATOGENIC EFFECTS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA Inventory

SARA 302 COMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA (TITLE 313): This material does not contain any chemical components with known CAS numbers that exceed the threshold (De

Minimis) reporting levels established by SARA Title III, Section 313.

MASSACHUSETTS RIGHT TO KNOW COMPONENETS: No components are subject to the Massachusetts Right to Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Neodymium oxide

NEW JERSEY RIGHT TO KNOW COMPONENTS: Neodymium oxide

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals known to State of California to cause cancer,

birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc. makes

no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the product based

on the information.

DATE REVISED: 08/25

SDS DEPT.

NEODYMIUM OXIDE                                                                          PAGE 3