Neodymium, Powder 2620 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/neodymium-powder-prochem-en-075e5ceccf

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Neodymium, Powder

PRODUCT NUMBER:  2620

CAS NUMBER:        7440-00-8

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H228               Flammable solid.

H260                    In contact with water releases flammable gases which may ignite spontaneously.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces. – No smoking.

P240                Ground/bond container and receiving equipment.

P231+P232          Handle under inert gas. Protect from moisture.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P302+P350            IF ON SKIN: Gently wash with soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338      IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

P370+P378             In case of fire: Use dry sand, dry chemical, or alcohol-resistant foam for extinction.

P402+P404           Store in a dry place. Store in a closed container.

HMIS CLASSIFICATION:

Health: 1              Fire: 3                  Reactivity Hazard: 2

NFPA RATING:

Health: 1            Flammability: 3         Reactivity Hazard: 2

NEODYMIUM POWDER                                                                  PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: Inhalation of fine dust may be irritating to the nasal cavity, mucous membranes, and respiratory tract.

INGESTION: No specific information on the physiological effects of ingestion.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Nd

Molecular Weight: 144.24

CHEMICAL NAME                                           CAS#                               %

Neodymium Powder                                                     7440-00-8                                               100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with soap and plenty of water. Remove contaminated clothes if necessary. Seek

medical assistance if irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use sand, carbon dioxide, or powdered extinguishing agent. Never use water.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Neodymium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid dust

formation. Avoid breathing dust. Keep away from ignition sources.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Mix spill with vermiculite or sodium carbonate. Sweep up

and place in suitable container for proper disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Ensure adequate ventilation. Avoid contact

with skin and eyes. Handle under inert gas. Keep container tightly closed.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store away from ignition sources. Air and moisture

sensitive. Store away from air, water, moisture, oxidizing agents, halogens, and acid chlorides.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

NEODYMIUM POWDER                                                                  PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

VENTILATION: Provide local exhaust, preferably mechanical.  Handle under inert gas.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Gray powder

ODOR: None

MOLECULAR WEIGHT: 144.24

BOILING POINT: 3027° C

MELTING POINT: 1021° C

SPECIFIC GRAVITY: 7 gm/ml

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Heat, flames, and sparks. Extremes of temperature and direct sunlight. Exposure to moisture.

INCOMPATIBILITY: Strong acids, nitrogen, strong oxidizing agents, phosphorus acid, and halogens.

DECOMPOSITION PRODUCTS: Neodymium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is no considered to be a

human carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: QO8575000

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

DOT:   UN3089                                              IATA:  UN3089

Metal powders, flammable, n.o.s.                                    Metal powder, flammable, n.o.s.

(Neodymium)                                                (Neodymium)

CLASS 4.1                                          CLASS 4.1

PG II                                          PG II

Marine Pollutant: No

NEODYMIUM POWDER                                                                  PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

SARA 302/304: Not Listed

SARA 311/312: Fire Hazard, Acute Health Hazard

SARA (TITLE 313): Not listed

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Neodymium

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Neodymium

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 03/15

SDS DEPT.

NEODYMIUM POWDER                                                                  PAGE 4