Nickel-64 SDS

Publisher: Neonest AB

Source: https://resds.com/document/nickel-64-isotope-sds-ni-64-64ni-nickel-metal-powder-enriched-nickel-metal-powder-add8c27b6f

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Nickel Metal

Powder, Enriched Nickel Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Nickel Metal Powder, Enriched Nickel   https://www.buyisotope.com   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Nickel Metal Powder, Enriched Nickel.

Molecular weight:   58.71

Chemical formula:   Ni

Synonyms:   None.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Skin Sensitizer (Category 1)   H317

Carcinogenicity (Category 2)   H351

STOT RE (Category 1)   H372

Label elements:

Hazard pictograms:

Page 2

Safety Data Sheet for Nickel Metal

Powder, Enriched Nickel Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Nickel Metal Powder, Enriched Nickel   https://www.buyisotope.com   Page 2/9

Signal Words:   Danger.

H317 May cause an allergic reaction.

H351 Suspected of causing cancer.

H372 Causes damage to organs through prolonged or repeated exposure.

Precautionary Statements:

P201 Obtain special instructions before use.

P202 Do not handle until all safety precautions have been read and understood.

P270 Do not eat, drink or smoke when using this product.

P280 Wear protective gloves/protective clothing.

P308 + P313 IF exposed or concerned: Get medical advice/attention.

P403 + P233 Store in a well-ventilated place. Keep container tightly closed.

P405 Store locked up.

P501 Dispose of contents/container in accordance with local and national regulations.

Other hazards:   None known.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Molecular

weight

Concentration

% by weight

Classification

EC1272/2008 EC - No.

Nickel

7440-02-0

58.71   >99%

Skin Sens. 1   H317

Carc. 2   H351

STOT RE 1   H372 231-111-4

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

In case of inhalation:   If inhaled, remove to fresh air. If breathing becomes difficult, call a physician.

In case of skin contact:   In case of contact, immediately wash skin with soap and copious amounts of water.

In case of eye contact:   In case of contact with eyes, flush with copious amounts of water for at least 15 minutes.

Assure adequate flushing by separating the eyelids with fingers. Call a physician.

In case of ingestion:   If swallowed, wash out mouth with water, provided person is conscious. Call a physician.

Most important symptoms and effects, both acute and delayed:

Skin contact:   May cause skin irritation.

Eye contact:   May cause eye irritation.

Inhalation:   Material may be irritating to mucous membranes and upper respiratory tract; may be harmful if inhaled.

Ingestion:   May be harmful if swallowed.

Indication of any immediate medical attention and special treatment needed:   Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Page 3

Safety Data Sheet for Nickel Metal

Powder, Enriched Nickel Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Nickel Metal Powder, Enriched Nickel   https://www.buyisotope.com   Page 3/9

Section 5: Fire - Fighting Measures

Extinguisher media:

Suitable extinguisher media:   Dry chemical powder.

Unsuitable extinguishing media:   None known.

Special hazards arising from the mixture:   Emits toxic fumes under fire conditions. Flammable solid.

Advice for fire-fighters:   Wear self-contained breathing apparatus and protective clothing for firefighting.

Further information:   Standard procedure for chemical fires. Use extinguishing measures that are appropriate to local

circumstances and the surrounding environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions:   Wear self-contained breathing apparatus, rubber boots and heavy rubber gloves. Wear

disposable coveralls and discard them after use.

Environmental precautions:   Prevent product from entering drains. Prevent further leakage or spillage if safe to do so.

Local authorities should be advised if significant spillages cannot be contained.

Methods for containment and cleaning up:

Methods for cleaning up:   Sweep up, place in a bag and hold for waste disposal. Avoid raising dust. Ventilate area and

wash spill site after material pickup is complete.

Reference to other sections:

Treat recovered material as described in the section "Disposal considerations".

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling:   Avoid breathing dust. Avoid contact with eyes, skin and clothing. Avoid prolonged or

repeated exposure. Handle under nitrogen.

Hygiene measures:   Do not eat, drink or smoke when using this product.

Conditions for safe storage, including incompatibilities:

Requirements for storage areas and containers:   Suitable: Keep tightly closed. Keep away from heat, sparks and open

flame. Store in a cool, dry place. Store under nitrogen.

Section 8: Exposure Controls/ Personal Protection

Control parameters:

Exposure Limits   –   RTECS:

Country   Source   Type   Value

USA   MSHA Standard-air   TWA   1 mg/m 3

USA   OSHA   PEL   8H TWA 1 mg (Ni)/m 3

New Zealand   OEL

Remarks: check ACGIH TLV

USA   NIOSH   TWA   0.015 mg (Ni)/m 3

Page 4

Safety Data Sheet for Nickel Metal

Powder, Enriched Nickel Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Nickel Metal Powder, Enriched Nickel   https://www.buyisotope.com   Page 4/9

Exposure controls:

Appropriate engineering controls:   Mechanical exhaust required. Safety shower and eye bath. Use non-sparking tools.

Individual protection measures, such as personal protective equipment:

Eye/face protection:   Wear safety goggles.

Hand protection:   Wear chemical-resistant gloves.

Body protection:   Wear protective clothing as appropriate.

Respiratory protection:   Use respirators and components tested and approved under appropriate government

standards such as NIOSH (US) or CEN (EU). Where risk assessment shows air-purifying respirators are appropriate, use

a dust mask type N95 (US) or type P1 (EN 143) respirator.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Solid (powder).

Colour:   Grey.

Odour:   No data available.

Odour threshold:   No data available.

Molecular Weight:   58. 71

pH (concentration):   No data available.

Melting point/range (° C):   1453 °C

Boiling point/range (° C):   2732   ° C

Freezing point   (°C):   No data available.

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   No data available.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour   pressure (20 ° C):   No data available.

Vapour density:   No data available.

Relative densi ty (25 ° C):   8.9 g/cm 3

Water solubility (g/L)   at 20 ° C:   Insoluble.

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Reactivity:   No dangerous reaction known under conditions of normal use.

Chemical stability:   Stable under normal conditions.

Possibility of hazardous reactions:   No dangerous reaction known under conditions of normal use.

Conditions to avoid:   Moisture .

Page 5

Safety Data Sheet for Nickel Metal

Powder, Enriched Nickel Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Nickel Metal Powder, Enriched Nickel   https://www.buyisotope.com   Page 5/9

Incompatible materials:   Acids, oxidizing agents, sulphur.

Hazardous decomposition products:   Nickel/nickel oxides.

Section 11: Toxicological Information

Information on toxicological effects:

Toxicity data: Rat 250 mg/kg LD 50

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

May cause an allergic skin reaction.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

This product is or contains a component that has been reported to be possibly carcinogenic based on its IARC, ACGIH,

NTP, or EPA classification.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Causes damage to organs through prolonged or repeated exposure.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:   No data available.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   Not relevant.

Other adverse effects:   An environmental hazard cannot be excluded in the event of unprofessional handling or

disposal. Harmful to aquatic life, with long-lasting effects.

Section 13: Disposal Considerations

Waste treatment methods:   Contact a licensed professional waste disposal service to dispose of this material. Dissolve

or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and

scrubber. Observe all federal, state and local environmental regulations.

Section 14: Transport Information

Page 6

Safety Data Sheet for Nickel Metal

Powder, Enriched Nickel Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Nickel Metal Powder, Enriched Nickel   https://www.buyisotope.com   Page 6/9

DOT:

Proper Shipping Name:   METAL POWDERS, FLAMMABLE, N.O.S.

Hazard Class : 4.1

UN Number:   3089

Packing Group:   II

Hazard Labels:

IATA:

Proper Shipping Name:   METAL POWDERS, FLAMMABLE, N.O.S.

Hazard Class : 4.1

UN Number:   3089

Packing Group:   II

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Page 7

Safety Data Sheet for Nickel Metal

Powder, Enriched Nickel Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Nickel Metal Powder, Enriched Nickel   https://www.buyisotope.com   Page 7/9

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations:   The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

EU Additional Classification

Symbol of Danger:   F-Xn

Indication of Danger:   Highly Flammable. Harmful.

R:   10-40-43

Risk Statements:   Flammable. Limited evidence of a carcinogenic effect. May cause sensitization by skin contact.

S:   16-36/37

Safety Statements:   Keep away from sources of ignition - no smoking. Wear suitable protective clothing and gloves.

US Classification and Label Text

Indication of Danger:   Harmful. Flammable (USA) Highly Flammable (EU).

Risk Statements:   Limited evidence of a carcinogenic effect. May cause sensitization by skin contact.

Safety Statements:   Keep away from sources of ignition - no smoking. Take precautionary measures against static

discharges. In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. Wear

suitable protective clothing and gloves.

US Statements:   Possible Carcinogen (US). Target organ(s): Lungs, nose.   California Prop. 65 carcinogen.

United States Regulatory Information

SARA Listed:   Yes

De minimis:   0.1%

Notes:   This product is subject to SARA section 313 reporting requirements.

TSCA Inventory Item:   Yes

California Prop. 65 Components:   This product is or contains chemical(s) known to the state of California to cause

cancer. This product is or contains chemical(s) known to the state of California to cause cancer.

Canada Regulatory Information

WHMIS Classification:   This product has been classified in accordance with the hazard criteria of the CPR, and the SDS

contains all the information required by the CPR.

DSL:   Yes

NDSL:   No

Page 8

Safety Data Sheet for Nickel Metal

Powder, Enriched Nickel Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Nickel Metal Powder, Enriched Nickel   https://www.buyisotope.com   Page 8/9

National regulations:   Follow national regulation for work with chemical agents.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

Page 9

Safety Data Sheet for Nickel Metal

Powder, Enriched Nickel Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Nickel Metal Powder, Enriched Nickel   https://www.buyisotope.com   Page 9/9

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H317 May cause an allergic reaction.

H351 Suspected of causing cancer.

H372 Causes damage to organs through prolonged or repeated exposure.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.