Platinum-194 SDS

Publisher: Neonest AB

Source: https://resds.com/document/platinum-194-isotope-sds-pt-194-194pt-platinum-metal-foil-enriched-platinum-metal-foil-786e43f3ba

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Platinum

Metal foil, Enriched Platinum

Metal Foil

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Platinum Metal Foil, Enriched Platinum   https://www.buyisotope.com   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Platinum Metal Foil; Platinum.

Molecular weight:   195.08

Chemical formula:   Pt

Synonyms:   Barium salt.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Not classified as hazardous.   None required.

Label elements:

Hazard pictograms:   Not required.

Signal Words:   Not required.

Hazard Statements:   Not required.

Precautionary Statements:   None.

Other hazards:   None known.

Page 2

Safety Data Sheet for Platinum

Metal foil, Enriched Platinum

Metal Foil

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Platinum Metal Foil, Enriched Platinum   https://www.buyisotope.com   Page 2/9

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Molecular

weight

Concentration

% by weight

Classification

EC1272/2008 EC - No.

Platinum

7440-06-4

195.08   >99%   Not Classified.

231-116-1

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

In case of inhalation:   If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a

physician.

In case of skin contact:   Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:   Flush eyes with water as a precaution.

In case of ingestion:   Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth

with water. Consult a physician.

Most important symptoms and effects, both acute and delayed:

Inhalation:   May be harmful if inhaled. May cause respiratory tract irritation.

Eyes:   May cause eye irritation.

Skin contact:   May be harmful if absorbed through skin. May cause skin irritation.

Ingestion:   May be harmful if swallowed.

Indication of any immediate medical attention and special treatment needed:   Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Section 5: Fire - Fighting Measures

Extinguisher media:

Suitable extinguisher media:   Use water spray, alcohol - resistant foam, dry chemical or carbon dioxide .

Unsuitable extinguishing media:   None known.

Special hazards arising from the mixture:   Emits toxic fumes under fire conditions.

Advice for fire-fighters:   Wear self-contained breathing apparatus for firefighting if necessary.

Further information:   Standard procedure for chemical fires. Use extinguishing measures that are appropriate to local

circumstances and the surrounding environment.

Page 3

Safety Data Sheet for Platinum

Metal foil, Enriched Platinum

Metal Foil

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Platinum Metal Foil, Enriched Platinum   https://www.buyisotope.com   Page 3/9

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions:   Avoid dust formation. Ensure adequate ventilation. Remove all sources of ignition. Evacuate

personnel to safe areas.

Environmental precautions:   Prevent product from entering drains. Prevent further leakage or spillage if safe to do so.

Local authorities should be advised if significant spillages cannot be contained.

Methods for containment and cleaning up:

Methods for cleaning up:   Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-

brushing and place in container for disposal according to local regulations (see Section 13). Keep in suitable, closed

containers for disposal.

Reference to other sections:

Treat recovered material as described in the section "Disposal considerations".

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling:   Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places

where dust is formed. Take measures to prevent the build-up of electrostatic charge.

Hygiene measures:   Do not eat, drink or smoke when using this product.

Conditions for safe storage, including incompatibilities:

Requirements for storage areas and containers:   Store in cool place. Keep container tightly closed in a dry, well-

ventilated place.

Section 8: Exposure Controls/ Personal Protection

Control parameters:

Occupational exposure limits:   Contains no substances with occupational exposure limit values

Exposure controls:

Appropriate engineering controls:   A system of local or mechanical exhaust is recommended to keep dust levels low.

Individual protection measures, such as personal protective equipment:

Eye/face protection:   Safety glasses.

Hand protection:   The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the

standard EN 374 derived from it. Handle with gloves.

Body protection:   Choose body protection according to the amount and concentration of the dangerous substance at

the workplace.

Respiratory protection:   Where risk assessment shows air-purifying respirators are appropriate, use a full-face particle

respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the

respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components

tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Page 4

Safety Data Sheet for Platinum

Metal foil, Enriched Platinum

Metal Foil

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Platinum Metal Foil, Enriched Platinum   https://www.buyisotope.com   Page 4/9

Se ction 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Solid (foil).

Colour:   Grey / silver.

Odour:   No data available.

Odour threshold:   No data available.

Molecular Weight:   195.08

pH (concentration):   No data available.

Melting point/range (° C):   1772   °C

Boiling point/range (° C):   3827   °C

Freezing point   (°C):   No data available.

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   No data available.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour   pressure (20 ° C):   No data available

Vapour density:   No data available.

Relative densi ty (25 ° C):   21.45 g/cm 3

Water solubility (g/L ) at 20 ° C:   No data available.

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Reactivity:   No dangerous reaction known under conditions of normal use.

Chemical stability:   Stable under normal conditions.

Possibility of hazardous reactions:   No dangerous reaction known under conditions of normal use.

Conditions to avoid:   None known .

Incompatible materials:   Strong oxidizing agents, alcohols.

Hazardous decomposition products:   Platinum oxide.

Section 11: Toxicological Information

Information on toxicological effects:

Not classified based on available information.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Page 5

Safety Data Sheet for Platinum

Metal foil, Enriched Platinum

Metal Foil

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Platinum Metal Foil, Enriched Platinum   https://www.buyisotope.com   Page 5/9

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Carcinogenicity - Rat - Implant

Tumorigenic: Equivocal tumorigenic agent, by RTECS criteria. Tumorigenic: Tumours at site of application.

Carcinogenicity - Mouse - Implant

Tumorigenic: Equivocal tumorigenic agent, by RTECS criteria. Tumorigenic: Tumours at site of application.

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as a probable,

possible or confirmed human carcinogen by IARC.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:   No data available.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   Not relevant.

Other adverse effects:   No data available.

Section 13: Disposal Considerations

Waste treatment methods:   Burn in a chemical incinerator equipped with an afterburner and scrubber, but exert extra

care in igniting. Observe all federal, state and local environmental regulations. Contact a licensed professional waste

disposal service to dispose of this material.

Section 14: Transport Information

UN number:   Not regulated as a dangerous good.

UN proper shipping name:   Not regulated as a dangerous good.

Transport hazard class(es):   Not regulated as a dangerous good.

Packing group:   Not regulated as a dangerous good.

Environmental hazards:   Not regulated as a dangerous good.

Special precautions for user:   Not dangerous goods in the meaning of ADR/RID, ADN, IMDG-Code, ICAO/IATA-DGR.

Transport in bulk according to Annex II of Marpol and the IBC Code:   Not applicable.

Page 6

Safety Data Sheet for Platinum

Metal foil, Enriched Platinum

Metal Foil

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Platinum Metal Foil, Enriched Platinum   https://www.buyisotope.com   Page 6/9

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Page 7

Safety Data Sheet for Platinum

Metal foil, Enriched Platinum

Metal Foil

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Platinum Metal Foil, Enriched Platinum   https://www.buyisotope.com   Page 7/9

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations:   The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

United States

SARA Listed:   No

TSCA Inventory Item:   Yes

Canada

WHMIS Classification:   This product has been classified in accordance with the hazard criteria of the CPR, and the SDS

contains all the information required by the CPR.

DSL:   Yes

NDSL:   No

National regulations:   Follow national regulation for work with chemical agents.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

Page 8

Safety Data Sheet for Platinum

Metal foil, Enriched Platinum

Metal Foil

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Platinum Metal Foil, Enriched Platinum   https://www.buyisotope.com   Page 8/9

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water H azard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

None.

Revision information:

Page 9

Safety Data Sheet for Platinum

Metal foil, Enriched Platinum

Metal Foil

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 18-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Platinum Metal Foil, Enriched Platinum   https://www.buyisotope.com   Page 9/9

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.