Praseodymium-doped gadolinium oxysulfide GA-1967 SDS

Publisher: Lorad

Source: https://resds.com/document/praseodymium-doped-gadolinium-oxysulfide-sds-en-211654f671

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

LORADCHEMICAL.COM

SAFETY DATA SHEET

1. PRODUCT AND COMPANY IDENTIFICATION

Trade Name:                              Praseodymium Doped Gadolinium Oxysulﬁde

Chemical Formula:                         Gd2O2S:Pr

Manufacturer Item Number:                 GA-1967

Manufacturer:                               Lorad Chemical Corporation

1200 19th Street North

Saint Petersburg, Florida, 33713

United States of America

Telephone:                              +1 (727) 826–5511

Fax:                                    +1 (727) 826–5510

Emergency Contact:                           (800) 255–3924 (US & Canada)

+1 (813) 248–0573 (International)

2. HAZARD IDENTIFICATION

Signal Word:                               Warning

Pictograms:

Hazard Statements:                      H302 Harmful if swallowed.

H312 Harmful in contact with skin.

H315 Causes skin irritation.

H319 Causes serious eye irritation.

H333 May be harmful if inhaled.

H335 May cause respiratory irritation.

Precautionary Statements:                 P260 Do not breath dust / fume / gas / mist / vapors / spray.

P264 Wash skin thoroughly after handling.

P270 Do not eat, drink or smoke when using this product.

P271 Use only outdoors or in a well-ventilated area.

P280 Wear protective gloves / protective clothing / eye protection / face protection.

P405 Store locked up.

P362+363 Take oﬀ contaminated clothing. Wash contaminated clothing before reuse.

P301+312 IF SHALLOWED: Call a POISON CENTER or physician if you feel unwell.

P305+351+338 IF IN EYES: Rinse cautiously with water for several minutes. Remove

contact lenses, if present and easy to do. Continue rinsing.

P302+332+313 IF ON SKIN: Wash with plenty of soap and water. If skin irritation

occurs: get medical advice/attention.

P337+313 If eye irritation persists get medical advice / attention.

P403+233 Store in a well ventilated place. Keep container tightly closed.

P304+340 IF INHALED: Remove person to fresh air and keep comfortable for breathing.

P501 Dispose of contents / container in accordance with local / regional / national /

international regulations.

HMIS Health Ratings (0-4)

- Health:                                 2

- Flammability:                            1

- Physical:                               1

Page 1of 5

Page 2

LORADCHEMICAL.COM

SAFETY DATA SHEET

3. COMPOSITION

Additional Names:                           Gadolinium oxide sulﬁde, praseodymium doped

Percentage:                             99+ wt%

CAS #:                                     68609-42-7

EC #:                                      271-826-9

4. FIRST AID PROCEDURES

General Treatment                            Consult a physician. Show this SDS to the doctor in attendance. Move out of dangerous

area.

Special Treatment:                     No Data Available.

Important Symptoms:                          Ingestion may cause nausea, vomiting, diarrhea, and abdominal pains. Inhalation of

dust may result in lung injury. Skin contact may cause a rash, redness or dermatitis. Eye

contact may cause redness or irritation.

Inhalation:                                                              If breathing, move person into fresh air. If not breathing, give artiﬁcial respiration.

Consult a physician. If irritation or other symptoms occur, remove from exposure and

consult a physician immediately.

Ingestion:                                 Never give anything by mouth to an unconscious person. Rinse mouth with water.

Consult a physician.

Skin:                                        For skin contact, ﬂush with large amounts of soap and water while removing

contaminated clothing. Consult a physician.

Eyes:                                        Immediately ﬂush eyes with fresh water for at least 15 minutes while holding the eyelids

open. Remove contact lenses if worn. Consult a physician.

5. FIREFIGHTING MEASURES

Flammability:                         No Data Available.

Special Hazards from Substance:          May evolve hazardous Sulfur Oxide fumes in ﬁre.

Extinguishing Media:                     Use alcohol-resistant foam, dry chemical or carbon dioxide.

Special Fire Fighting Procedures:            Wear self-contained breathing apparatus for ﬁreﬁghting if necessary.

6. ACCIDENTAL RELEASE MEASURES

If Material is Released / Spilled:               Avoid dust formation. Avoid breathing vapors, mist, or gas. Ensure adequate ventilation.

Vacuum up spillage with vacuum equipped with appropriate ﬁlter to prevent airborne

dust. If dust is present, use NIOSH approved respirator. Collect in suitable container for

disposal. Dispose of waste in accordance with applicable federal, state, local, and

provincial environmental regulations.

Environmental Precautions:                   Prevent further leakage or spills if safe to do so. Do not allow to enter drains, sewers, or

watercourses.

7. HANDLING AND STORAGE

Handling Conditions:                         Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Provide

appropriate exhaust ventilation at places where dust is formed. For precautions see

Section 2.

Page 2of 5

Page 3

LORADCHEMICAL.COM

SAFETY DATA SHEET

Storage Conditions:                      Keep container tightly closed in a cool, dry, and well-ventilated place.

Work / Hygienic Maintenance:              Keep formation of airborne dusts to a minimum. Guard against dust accumulation. In

case of insuﬃcient ventilation, wear suitable respiratory equipment.

Ventilation:                                   Provide appropriate exhaust ventilation at places where dust is formed. In case of

insuﬃcient ventilation wear suitable respiratory equipment.

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Permissible Exposure Limits:             No Data Available.

Threshold Limit Value:                  No Data Available.

Special Equipment:                    No Data Available.

Respiratory Protection:                    Wear respirator if there is dust formation or high concentrations of material present. In

case of inadequate ventilation or risk of inhalation of dust, use suitable respiratory

equipment.

Protective Gloves:                          Handle with gloves. Gloves must be inspected prior to use. use proper glove removal

technique (without touching glove’s outer surface) to avoid skin contact with this product.

Dispose of contaminated gloves after use in accordance with applicable laws and good

laboratory practices. Wash and dry hands.

Eye Protection:                                Safety glasses and face shield. Use equipment for eye protection tested and approved

under government standards such as NIOSH (US) or EN 166(EU).

Body Protection:                         Wear appropriate protective clothing.

9. PHYSICAL AND CHEMICAL CHARACTERISTICS

Color:                                      White

Forms:                                 Powder

Odor:                                       Odorless

Water Solubility:                                 Insoluble

Boiling Point:                         No Data Available.

Melting Point:                              >1,100ºC (>2,012ºC) (decomposes)

Flash Point:                          No Data Available.

Autoignition Temperature:               No Data Available.

Density:                             No Data Available.

Molecular Weight:                           519.47 g/mol

10. REACTIVITY

Stability:                                      Stable under recommended storage conditions.

Reacts with:                                     Material slowly hydrolyzes with formation of hydrogen sulﬁde in moist air and / or

aqueous solutions.

Page 3of 5

Page 4

LORADCHEMICAL.COM

SAFETY DATA SHEET

Incompatible Conditions:                     Strong acids, strong oxidizers, and ﬂammable materials.

Hazardous Decomposition Products:           Sulfur oxides, Hydrogen sulﬁde

11. TOXICOLOGICAL INFORMATION

Potential Health Effects:

- Eyes                              May cause irritation.

- Skin                              May cause irritation.

- Ingestion                           May cause irritation, nausea, vomiting, diarrhea, abdominal pains.

-  Inhalation                          May cause serious irritation.

- Chronic                           No Data Available.

Signs and Symptoms:                   No Data Available.

Aggravate Medical Conditions:            No Data Available.

Median Lethal Dose:                    No Data Available.

Carcinogen:                                    This material has not been identiﬁed as a known or suspected carcinogen by NTP,

IARC, or OSHA.

12. ECOLOGICAL INFORMATION

Aquatic Toxicity:                      No Data Available.

Persistence and degradability:            No Data Available.

Bioaccumulative potential:               No Data Available.

Notes:                              Do not allow material to be released into the environment without proper government

permits. Do not allow undiluted product or large quantities to reach ground water, water

course, or sewage system. Avoid transfer into the environment.

13. DISPOSAL CONSIDERATIONS

Disposal:                                         Offer surplus and non-recyclable solutions to a licensed disposal company. Contact a

licensed professional waste disposal service to dispose of this material. Empty

containers should be taken to an appropriated waste handling site for recycling or

disposal. Dispose of in accordance with local, state, or national regulations.

14. TRANSPORTATION DATA

Hazardous:                              DoT: Not Dangerous Goods

IMDG: Not Dangerous Goods

IATA: Not Dangerous Goods

Pictogram:                              N/A

Hazard Class:                            N/A

Packing Group:                           N/A

UN Number:                             N/A

Proper Shipping Name:                    N/A

Page 4of 5

Page 5

LORADCHEMICAL.COM

SAFETY DATA SHEET

15. REGULATORY INFORMATION

SARA 302 Components                  No chemical in this material are subject to the reporting requirements of SARA Title III,

Section 302.

SARA 313 Components                        This material does not contain any chemical components with known CAS numbers that

exceed the threshold (de minimus) reporting levels established by SARA Title III, Section

313.

SARA 311/312 Hazards                       Acute Health Hazard, Chronic Health Hazard

Massachusetts Right to Know Components   No components are subject to Ma. Right to Know Act.

Pennsylvania Right to Know Components     Gadolinium Oxysulﬁde: Praseodymium Activated (CAS No. 68609-42-7)

New Jersey Right to Know Compoents        Gadolinium Oxysulﬁde: Praseodymium Activated (CAS No. 68609-42-7)

California Prop. 65 Components               This product does not contain any chemicals known to the State of California to cause

cancer, birth defects, or any other reproductive harm.

16. OTHER INFORMATION

Copyright 2017 Lorad Chemical Corporation. The above information is believed to be correct but does not purport to be all inclusive and

shall be used only as a guide. The information in this document is based on the present state of our knowledge and is applicable to the

product with regards to appropriate safety standards. It does not represent a guarantee of the properties of the product. Lorad Chemical

Corporation and its Afﬁliates shall not be held liable for any damage resulting from handling or from contact with the above product.

Revision Date: 08/01/2017

Page 5of 5