Praseodymium (III) Fluoride 2971 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/praseodymium-iii-fluoride-prochem-en-0fda313d48

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Praseodymium (III) Fluoride

PRODUCT NUMBER:  2971

CAS NUMBER:        13709-46-1

SYNONYMS:        Praseodymium trifluoride

MANUFACTURER:

ProChem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                    TOLL FREE:    800-795-8788

Rockford, IL 61109

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H301+H311+H331     Toxic if swallowed, in contact with skin, or if inhaled.

Precautionary Phrases

P261                Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling.

P270             Do not eat, drink, or smoke when using this product.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P301+P310            IF SWALLOWED: Immediately call a POISON CENTER or doctor/physician.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338     IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

P312                   Call a POISON CENTER or doctor/physician if you feel unwell.

P361               Remove/take off immediately all contaminated clothing.

P363             Wash contaminated clothing before reuse.

P403+P233           Store in a well-ventilated place. Keep container tightly closed.

PRASEODYMIUM (III) FLUORIDE                                                           PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 0

EYE CONTACT: Causes irritation of the eyes. May cause serious damage. Effects may not immediately appear.

SKIN CONTACT: Causes irritation, with redness and pain. Soluble fluoride may be corrosive. Effects may not appear immediately.

INHALATION:  Irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: May cause salivation, nausea, vomiting, diarrhea, and abdominal pain.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: PrF3

Molecular Weight: 197.9

CHEMICAL NAME                                           CAS#                               %

Praseodymium (III) fluoride                                               13709-46-1                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under eyelids. A

victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if irritation

persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial respiration.

Get medical attention if necessary.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or water spray. Fight large fires with water spray or alcohol

resistant foam.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen fluoride and praseodymium oxides.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid dust

formation.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with powdered sodium

bicarbonate, lime, or calcium carbonate, and swept up.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Handle

under dry protective gas. Ensure good ventilation.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Do not store together with acids. Store away from

moisture, water, and reducing agents. Hygroscopic. Protect from humidity and water.

PRASEODYMIUM (III) FLUORIDE                                                           PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS:

Component                 Exposure Limits              Basis                           Entity

Praseodymium (III) fluoride       2.5 mg/m3                 TLV                     ACGIH

2.5 mg/m3                 PEL                   OSHA

2.5 mg/m3                REL                     NIOSH

TLV: Threshold Limit Value over 8 hours of work.           REL: Recommended Exposure Limit

PEL: Permissible Exposure Limit                           BEI-U: Biological Exposure Indices in Urine

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Light blue to green powder

ODOR: None

MOLECULAR WEIGHT: 197.9

BOILING POINT: No data available

MELTING POINT: 1395°C

SPECIFIC GRAVITY: 6.2

VAPOR DENSITY: No data available

SOLUBILITY IN WATER: Slightly soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Moisture

INCOMPATIBILITY: Reducing agents, acids, water, and moisture.

DECOMPOSITION PRODUCTS: Hydrogen fluoride and praseodymium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: LC50 Inhalation – 4h - .51 mg/l. LD50 Dermal – 300 mg/kg.

CARCINOGENIC EFFECTS: No component of this product present at levels greater than or equal to 0.1% is identified as a

carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

PRASEODYMIUM (III) FLUORIDE                                                           PAGE 3

Page 4

SECTION 14  TRANSPORTATION DATA

UN3288

Toxic solid, inorganic, n.o.s.

(Praseodymium (III) fluoride)

CLASS 6.1

PG III

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Acute health Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

NEW JERSEY RIGHT TO KNOW COMPONENTS: Praseodymium (III) fluoride

RHODE ISLAND RIGHT TO KNOW COMPONENTS: Praseodymium (III) fluoride

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE REVISED: 02/26

SDS DEPT.

PRASEODYMIUM (III) FLUORIDE                                                           PAGE 4