Praseodymium (III, IV) Oxide 2979 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/praseodymium-iii-iv-oxide-prochem-en-f62ff24748

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Praseodymium (III, IV) Oxide

PRODUCT NUMBER:  2979

CAS NUMBER:        12037-29-5

SYNONYMS:         Hexapraseodymium undecaoxide

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Not a dangerous substance according to the GHS

HMIS CLASSIFICATION:

Health: 1            Flammability: 0                   Reactivity: 0

NFPA RATING:

Health: 1              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: May be irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: May be harmful if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Pr6O11

Molecular Weight: 1021.43

CHEMICAL NAME                         CAS#               %              ACGIH (TWA)            OSHA (PEL)

Praseodymium (III, IV) Oxide                         12037-29-5                   100               No data                   No data

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

PRASEODYMIUM (III, IV) OXIDE                                                                  PAGE 1

Page 2

SECTION 4   FIRST AID MEASURES (Cont.)

INHALATION: Remove victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty in

breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: Not applicable

EXPLOSION LIMITS: Not applicable

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: If involved in a fire, fire fighters should be equipped with a NIOSH approved

positive self-contained breathing apparatus, flame and chemical resistant clothing, hats, boots, and gloves.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: None available.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear protective equipment. Ensure adequate ventilation. Avoid breathing vapors, mist, or gas.

ENVIRONMENTAL PRECAUTIONS: Do not let enter drains. Do not allow to be released into environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with vermiculite or sodium

carbonate and swept up.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Provide appropriate exhaust ventilation at places where dust is formed.

CONDITIONS FOR SAFE STORAGE: Store material in a tightly sealed container.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when handling a chemical

substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

VENTILATION: Handle in an efficient chemical fume hood.

RESPIRATOR:  If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Black powder

MOLECULAR WEIGHT: 1021.43

BOILING POINT: 4200° C

MELTING POINT: 2500° C

SPECIFIC GRAVITY: 7.07 gm/ml

ODOR: Odorless

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES (Cont.)

VAPOR PRESSURE: Not applicable

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Air and moisture stable

HAZARDOUS POLYMERIZATION: No hazardous polymerization

CONDITIONS TO AVOID: None

INCOMPATIBILITY: Strong oxidizing agents

DECOMPOSITION PRODUCTS: None

PRASEODYMIUM (III, IV) OXIDE                                                                  PAGE 2

Page 3

SECTION 11 TOXICOLOGICAL DATA

RTECS DATA: Oral (rat); LD50: >5 gm/kg. Intraperitoneal (mouse); Cytogenetic analysis: 77 mg/kg.

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified as a confirmed

carcinogen by IARC, ACGIH, NTP, and OSHA.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not availabe

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 313 COMPONENTS: This material does not contain any chemical components with know CAS numbers that exceed the

threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

SARA 302 COMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA 311/312 HAZARDS: No SARA Hazards

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right To Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Dipraseodymium trioxide

NEW JERSEY RIGHT TO KNOW COMPONENTS: Dipraseodymium trioxide

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals known to State of California to cause cancer,

birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 04/14

PRASEODYMIUM (III, IV) OXIDE                                                                  PAGE 3