Praseodymium, powder 2953 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/praseodymium-powder-prochem-en-c695182516

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Praseodymium, powder

PRODUCT NUMBER:  2953

CAS NUMBER:        7440-10-0

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228               Flammable solid.

H260                    In contact with water releases flammable gases which may ignite spontaneously.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces.-No smoking.

P231+P232          Handle under inert gas. Protect from moisture.

P241              Use explosion-proof electrical/ventilating/lighting equipment.

P370+P378             In case of fire: Use dry sand, dry chemical or alcohol-resistant foam for extinction.

P402+P404           Store in a dry place. Store in a closed container.

HMIS CLASSIFICATION:

Health: 1              Fire: 3                  Reactivity Hazard: 2

NFPA RATING:

Health: 1            Flammability: 3         Reactivity Hazard: 2

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: Inhalation may be irritating to the nasal cavity, mucous membranes, and respiratory tract.

INGESTION: May be harmful if swallowed.

PRASEODYMIUM, POWDER                                                               PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Pr

Molecular Weight: 140.90

CHEMICAL NAME                                           CAS#                               %

Praseodymium                                                           7440-10-0                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under the

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Special powder for metal fires. Do not use water.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Praseodymium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Keep unprotected

persons away. Keep away from sources of ignition.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Cover powder with sodium carbonate of dolomite to reduce

formation of dust. Sweep up and dispose accordingly.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Ensure adequate ventilation. Handle under

dry protective gas. Protect against electrostatic charges.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store away from air, moisture, water, acids,

oxidizing agents, halogens, and acid chlorides. Store under dry inert gas. Air and moisture sensitive.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Grey to silver powder

ODOR: None

MOLECULAR WEIGHT: 140.90

BOILING POINT: 3020°C

MELTING POINT: 935°C

PRASEODYMIUM, POWDER                                                               PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES (Cont.)

SPECIFIC GRAVITY: 6.77gm/ml

SOLUBILITY: Reacts with water

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Contact with hot water. Mixtures of fine dust and air.

INCOMPATIBILITY: Water, alcohols, oxidizers, oxygen, halogens, halocarbons, and mineral acids.

DECOMPOSITION PRODUCTS: Praseodymium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

DOT:   UN3089                               IATA:  UN3089

Metal powders, flammable, n.o.s.                   Metal powder, flammable, n.o.s.

(Praseodymium)                                (Praseodymium)

CLASS 4.1                             CLASS 4.1

PGII                                            PGII

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Reactivity Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 10/14

PRASEODYMIUM, POWDER                                                               PAGE 3