Samarium Chips 3067 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/samarium-chips-prochem-en-654d6de979

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Samarium Chips

PRODUCT NUMBER:  3067

CAS NUMBER:        7440-19-9

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228               Flammable solid.

H261                    In contact with water released flammable gases.

H373             May cause damage to organs through prolonged or repeated exposure.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces. –No smoking.

P223              Keep away from any possible contact with water, because of violent reaction and possible flash fire.

P231+P232          Handle under inert gas. Protect from moisture.

P240                Ground/bond container and receiving equipment.

P241              Use explosion-proof electrical/ventilating/lighting/equipment.

P260             Do not breathe dust/fume/gas/mist/vapors/spray.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P335+P334          Crush off loose particles from skin. Immerse in cool water/wrap in wet bandages.

P370+P378             In case of fire: Use dry sand, dry chemical, or alcohol-resistant foam for extinction.

P402+P404           Store in a dry place. Store in a closed container.

HMIS CLASSIFICATION:

Health: 1            Chronic: *               Fire: 1                     Reactivity Hazard: 1

NFPA RATING:

Health: 1            Flammability: 1         Reactivity Hazard: 1       Special: W

SAMARIUM CHIPS                                                                     PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: May be irritating to the nasal cavity, mucous membranes, and respiratory tract.

INGESTION: May be irritating if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Sm

Molecular Weight: 150.36

CHEMICAL NAME                                           CAS#                               %

Samarium Chips                                                          7440-19-9                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Dry chemical or Class D metal fire agent. Do not use water.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Samarium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust. Remove all sources of ignition.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Pick up and arrange disposal without creating dust. Sweep

up and keep in suitable, closed containers for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

contact with skin and eyes. Avoid formation of dust. Take all measures to prevent the build up of electrostatic charge.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Never allow product in contact with water during

storage.

SAMARIUM CHIPS                                                                     PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substance with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Silver to Grey chips

ODOR: None

MOLECULAR WEIGHT: 150.36

BOILING POINT: 1778° C

MELTING POINT: 1074° C

SPECIFIC GRAVITY: 7.47 gm/ml

SOLUBILITY: Reacts with water

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Avoid moisture. Air sensitive. Exposure to moisture. Heat, flames, and sparks. Extremes of temperature

and direct sunlight. Exposure to moisture.

INCOMPATIBILITY: Halogens, strong acids, strong oxidizing agents, and water.

DECOMPOSITION PRODUCTS: Samarium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

DOT:   UN3089                                      IATA:  UN3089

Metal Powders, Flammable, n.o.s.                          Metal Powder, Flammable, n.o.s

(Samarium)                                            (Samarium)

Class 4.1                                                Class 4.1

PG III                                    PG III

SAMARIUM CHIPS                                                                     PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not listed

SARA 311/312: Chronic Health Hazard

SARA (TITLE 313): Not listed

PENNSYLVANIA RIGHT TO KNOW: Samarium

NEW JERSEY RIGHT TO KNOW: Samarium

CALIFORNIA PROP. 65: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 05/15

SDS DEPT.

SAMARIUM CHIPS                                                                     PAGE 4