Samarium (III) Oxide SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/samarium-iii-oxide-sds-prochem-en-74b16c4332

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1  PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Samarium (III) Oxide

PRODUCT NO:       3096

CAS NO:             12060-58-1

SYNONYMS:         Samarium sesquioxide

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2  HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Not a dangerous substance according to the GHS

HMIS CLASSIFICATION:

Health: 0            Flammability: 0                   Reactivity: 0

NFPA RATING:

Health: 0              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: May cause eye irritation.

SKIN CONTACT: May cause skin irritation.

INHALATION: May cause respiratory tract irritation.

INGESTION: May be harmful if swallowed.

SECTION 3   COMPOSITION / INFORMATION ON INGREDIENTS

Formula:  Sm2O3

Molecular Weight: 348.72

CHEMICAL NAME                  CAS#             %               ACGIH (TWA)            OSHA (PEL)

Samarium oxide                          12060-58-1                   100               No data                   No data

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Rinse opened eye for several minutes under running water. Consult a physician.

SKIN EXPOSURE: Immediately wash with water and soap and rinse thoroughly. Seek immediate medical advice.

INHALATION: Supply fresh air.  If required, provide artificial respiration. Keep patient warm. Immediately seek medical advice.

SAMARIUM (III) OXIDE                                                                         PAGE 1

Page 2

SECTION 4   FIRST AID MEASURES (Cont.)

INGESTION: Do NOT induce vomiting. Loosen tight clothing such as a collar, tie, belt, or waistband.  If victim is not breathing,

perform mouth-to-mouth resuscitation. Immediately seek medical attention.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable.

AUTO IGNITION TEMPERATURE: Not applicable

EXPLOSION LIMITS: Not applicable

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES:   If involved in a fire, fire fighters should be equipped with a NIOSH approved positive

pressure self-contained breathing apparatus and full protective clothing.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: None

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear approved personal protective equipment. Avoid breathing dust, vapors, mist, or gas. Ensure

adequate ventilation.

ENVIRONMENTAL PRECAUTIONS: Do not let product enter drains or allow to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Use shovel to put material in the proper container for

disposal. Finish cleaning by spreading water on the contaminated surface.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear personal protective equipment. Keep container tightly sealed. Do not breathe dust.

Ensure adequate ventilation.

CONDITIONS FOR SAFE STORAGE: Keep container tightly closed and store in a cool, dry, and well-ventilated area.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when handling a chemicals

substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

VENTILATION:  If possible, handle the material in an efficient fume hood.

RESPIRATOR:   If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9   PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Yellow to beige powder

MELTING POINT: 2335°C

BOILING POINT: No data available

MOLECULAR WEIGHT: 348.72

ODOR: No data available

SPECIFIC GRAVITY: 8.347 gm/ml

VAPOR PRESSURE: Not applicable

SOLUBILITY IN WATER: Insoluble

SECTION 10   STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: No hazardous polymerization

CONDITIONS TO AVOID: None

SAMARIUM (III) OXIDE                                                                         PAGE 2

Page 3

SECTION 10   STABILITY AND REACTIVITY (Cont.)

INCOMPATIBILITY: Strong oxidizing agents

DECOMPOSITION PRODUCTS: None

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Acute oral toxicity (LD50): 5000 mg/kg (Rat).

CARCINOGENIC EFFECTS: No component of this product present at levels greater than or equal to 0.1% is identified as probable,

possible or confirmed human carcinogen.

MUTAGENIC EFFECTS: No data

TETRATOGENIC EFFECTS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA Inventory

OSHA HAZARDS: No known OSHA Hazards

SARA 302 COMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA (TITLE 313): This material does not contain any chemical components with known CAS numbers that exceed the threshold (De

Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 HAZARDS: No SARA Hazards

MASSACHUSETTS RIGHT TO KNOW COMPONENETS: No components are subject to the Massachusetts Right to Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Samarium (III) oxide

NEW JERSEY RIGHT TO KNOW COMPONENTS: Samarium (III) oxide

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals known to State of California to cause cancer,

birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 05/14

SDS DEPT.

SAMARIUM (III) OXIDE                                                                         PAGE 3