Scandium 2,4-pentanedionate, hydrate 3151 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/scandium-2-4-pentanedionate-hydrate-prochem-en-cfc69cdce2

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Scandium 2,4-pentanedionate, hydrate

PRODUCT NUMBER:  3151

CAS NUMBER:        699012-88-9

SYNONYMS:        Scandium acetylacetonate hydrate

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H302                Harmful if swallowed

H315               Causes skin irritation

H319               Causes serious eye irritation

H335             May cause respiratory irritation

Precautionary Phrases

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P270             Do not eat, drink, or smoke when using this product.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/protective clothing/eye protection/face protection

P301+P312            IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338      IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 0

SCANDIUM 2, 4-PENTANEDIONATE                                                          PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: Causes irritation of the eyes.

SKIN CONTACT: Causes irritation of and redness of the skin.

INHALATION: Causes irritation to nose, throat, and respiratory tract.

INGESTION: May be harmful if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Sc(CH3COCHCOCH3)3  xH2O

Molecular Weight: 342.26 (anhy.)

CHEMICAL NAME                                           CAS#                               %

Scandium 2,4-pentanedionate                                            699012-88-9                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for several minutes. Assure flushing under eyelids.

A victim may need assistance in keeping their eyelids open. Get immediate medical attention.

SKIN EXPOSURE: Wash affected area with soap and copious amounts of water. Remove contaminated clothes if

necessary. Seek medical assistance if irritation, burns, or redness, persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Do NOT induce vomiting. Never give anything by mouth to an unconscious person.  If conscious, wash out mouth

with water. Get medical attention if necessary.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or water spray. Fight large fires with water spray or alcohol

resistance foam.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Carbon oxides and scandium oxides.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing it to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up avoiding raising dust and place in a well-marked

container for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Wash thoroughly after using. Keep container closed when not in use. Avoid formation of dusts and aerosols.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Air sensitive.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield

SCANDIUM 2, 4-PENTANEDIONATE                                                          PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Clear to yellow solid

ODOR: None

MOLECULAR WEIGHT: 324.26 (anhy.)

BOILING POINT: No data available

MELTING POINT: 174-186° C

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Moisture

INCOMPATIBILITY: Oxidizing agents, acids, water, and moisture.

DECOMPOSITION PRODUCTS: Carbon oxides and scandium oxides.

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good.

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

DSCL (EEC): Listed on the DSCL inventory

SARA 302/304: Not Listed

SARA 311/312: Acute Health Hazard, Chronic Health Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

SCANDIUM 2, 4-PENTANEDIONATE                                                          PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION (Cont.)

NEW JERSEY RIGHT TO KNOW COMPONENTS: Scandium (III) acetylacetonate hydrate

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Scandium (III) acetylacetonate hydrate

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 08/14

SCANDIUM 2, 4-PENTANEDIONATE                                                          PAGE 4