Scandium (III) fluoride SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/scandium-iii-fluoride-sds-prochem-en-4d398fe4b4

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

24 HR. EMERGENCY PHONE # CHEM-TREC 1-800-424-9300

Manufacturer: ProChem, Inc. Phone: (815) 398-1788

826 Roosevelt Road Fax: (815) 398-1810

Rockford, IL 61109 E-Mail: Prochem1 @aol.com

SECTION 1 Product Identification ;

CHEMICAL NAME: Scandium (III) fluoride

CAS #: 13709-47-2

FORMULA: ScF3

SYNONYM: Scandium trifluoride

SECTION 2 Composition and Information on Ingredients

INGREDIENT: CAS # % ACGIH (TWA) OSHA (PEL)

Title Compound 13709-47-2 100 2.5mg/m3(as F) 2.5mg/m3(as F)

SECTION 3 Hazards Identification

EMERGENCY OVERVIEW: Irritating to skin, eyes and respiratory tract.

PRIMARY ROUTES OF EXPOSURE: Contact with skin and eyes. inhalation of dust.

EYE CONTACT: Causes slight to mild irritation of the eyes.

SKIN CONTACT: Causes slight to mild irritation of the skin.

INHALATION: Irritating to the nose, mucous membranes and respiratory tract.

INGESTION: Toxicity reduced to the extent that this compound is relatively insoluble. In large amounts may

cause vomiting, abdominal pain and diarrhea.

CHRONIC HEALTH EFFECTS: Prolonged exposure to hydrolysable fluorine compounds can cause deteriora-

tion of bone and tooth structure.

ACUTE HEALTH EFFECTS: Irritating to skin, eyes and respiratory tract. ingestion of large amounts may cause

fluoride poisoning. Symptoms include nausea, vomiting, diarrhea, weakness, coma, and cardiovascular collapse.

SECTION 4 First Aid Measures

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure

flushing under eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medi-

cal attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical

assistance if irritation persists.

INHALATION: Remove the victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if

conscious). Induce vomiting only if directed by medical personnel.

Page 2

a

SECTION 5 Firefighting Measures

a

FLASH POINT: not applicable

AUTO IGNITION TEMPERATURE: none

EXPLOSION LIMITS: none

EXTINGUISHING MEDIUM: None. Material is non-flammable

SPECIAL FIREFIGHTING PROCEDURES: ff involved in a fire, fire fighters should be equipped with a NIOSH

approved positive pressure self-contained breathing apparatus and full protective clothing.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: none

UNUSUAL FIRE OR EXPLOSION HAZARDS: No unusual fire or explosion hazards.

SECTION 6 Accidental Release Measures

SPILL AND LEAD PROCEDURES: Small spills can be mixed with powdered sodium bicarbonate, lime, or cal-

cium carbonate and swept up.

SECTION 7 Handling and Storage

HANDLING AND STORAGE: Store solid in a tightly sealed container away from moisture. Handle under a dry at-

mosphere of air or nitrogen. Prolonged exposure to the atmosphere may degrade the product.

SECTION 8 Exposure Controls and Personal Protection

EYE PROTECTION: Always wear approved safety glasses w/side shields, or safety goggles, face shield when

handling a chemicals substance in the laboratory.

SKIN PROTECTION: Chemical-resistant.

VENTILATION: If possible, handle the material in an efficient fume hood.

RESPIRATOR: If in form of fine dust and ventilation is not available a respirator should be worn. The use of respi-

rators requires a Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9 Physical and Chemical Properties

COLOR AND FORM: white powder

MOLECULAR WEIGHT: 101.95

MELTING POINT (DEG. C.): none

BOILING POINT: no data

VAPOR PRESSURE: no data

SPECIFIC GRAVITY: nodata

SOLUBILITY IN WATER: insoluble

SECTION 10 Stability and Reactivity

STABILITY: hygroscopic

HAZARDOUS POLYMERIZATION: no hazardous polymerization

CONDITIONS TO AVOID: contact with moisture

Page 3

INCOMPATIBILITY: active metals

DECOMPOSITION PRODUCTS: none

SECTION 11 Toxicological Information

RTECS DATA: Oral (mammal-species unspecified); LO: >10 gm/kg. Intraperitoneal (mammal-species unspeci-

fied); LO: >1 0 gm/kg

MUTAGENIC EFFECTS: no data

TETRATOGENIC EFFECTS: no data

CARCINOGENIC EFFECTS: no data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12 Ecological Information

ECOLOGICAL INFORMATION: No information available

SECTION 13 Disposal Considerations

DISPOSAL: Dispose of in according to local state and federal regulations.

SECTION 14 Transportation Information

Non-hazardous Non-hazardous

SECTION 15 Regulatory Information

TSCA: Listed in the TSCA inventory

SARA (TITLE 313): Title compound not listed

SECTION 16 Other Information

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However,

ProChem, Inc. makes no representation, warranty, or guarantee of any kind with respect to the information in this

document or any use of the product based on the information.

DATE PREPARED: 4/06