Scandium (III) Nitrate, Tetrahydrate 3139 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/scandium-iii-nitrate-tetrahydrate-prochem-en-ea1921a0b4

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Scandium (III) Nitrate, Tetrahydrate

PRODUCT NUMBER:  3139

CAS NUMBER:        13465-60-6

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H272             May intensify fire; oxidizer.

H315               Causes skin irritation

H319               Causes serious eye irritation

H335             May cause respiratory irritation

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces

P220                 Keep/store away from clothing/combustible material

P280              Wear protective gloves/protective clothing/eye protection/face protection

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P305+P351+P338      IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

HMIS CLASSIFICATION:

Health: 1              Fire: 0                  Reactivity Hazard: 1

NFPA RATING:

Health: 1            Flammability: 0         Reactivity Hazard: 1       Specific Hazard: OX

EYE CONTACT: Causes eye irritation.

SKIN CONTACT: Causes skin irritation

INHALATION: Causes respiratory tract irritation.

INGESTION: May be harmful if swallowed

SCANDIUM NITRATE                                                                   PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Sc(NO3)3  4H2O

Molecular Weight: 303.03

CHEMICAL NAME                                           CAS#                               %

Scandium (III) Nitrate Tetrahydrate                                        13465-60-6                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water. Assure adequate flushing under eyelids. Seek

medical attention.

SKIN EXPOSURE: Was affected area with soap and water. Remove contaminated clothes if necessary. Seek medical

assistance if irritation or redness persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Do NOT induce vomiting. Never give anything by mouth to an unconscious person.  If conscious, wash out mouth

with water. Get medical attention if necessary.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use carbon dioxide, extinguishing powder, or water spray. Fight larger fires with water spray or alcohol

resistant foam.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Nitrogen oxides and scandium oxides.

This substance is an oxidizer and its heat of reaction with reducing agents or combustibles may cause ignition.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing it to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Avoid raising dust. Sweep up and place in a well-marked

container for proper disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Wash thoroughly after using. Keep container closed when not in use. Avoid formation of dusts and aerosols. Keep

away from sources of ignition – No smoking.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store away from flammable substances, reducing

agents, organic materials, and metal powders.  Hygroscopic

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SCANDIUM NITRATE                                                                   PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White solid

ODOR: No data available

ODOR THRESHOLD: No data available

MOLECULAR WEIGHT: 303.03

BOILING POINT: No data available

MELTING POINT: No data available

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Exposure to moisture (hygroscopic)

INCOMPATIBILITY: Strong reducing agents, strong acids, flammable substances, organic materials, and metal powders.

DECOMPOSITION PRODUCTS: Scandium oxides and nitrogen oxides.

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: According to ACGIH, NTP, and OSHA, No components of this product present at levels greater than or

equal to 0.1% is identified a carcinogen. According to IARC, this substance is probably carcinogenic to humans

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN1477

Nitrates, inorganic, n.o.s.

(Scandium (III) nitrate tetrahydrate)

CLASS 5.1

PG III

Marine Pollutant: No/

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

DSCL (EEC): Listed on the DSCL inventory

SARA 302/304: Not Listed

SARA 311/312: Reactivity Hazard, Chronic Health Hazard

SARA (TITLE 313): Not Listed

SCANDIUM NITRATE                                                                   PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

CALIFORNIA PROP. 65: Not Listed

WHMIS CANADA: Class C: Oxidizing materials. Class D-2B: Toxic material causing other toxic effects

NEW JERSEY RIGHT TO KNOW COMPONENTS: Scandium trinitrate

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Scandium trinitrate

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 08/14

SCANDIUM NITRATE                                                                   PAGE 4