Silicon-29 SDS

Publisher: Neonest AB

Source: https://resds.com/document/silicon-29-sds-si-29-ffa2de641f

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Silicon Dioxide,

Enriched Silicon Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 25-Aug-2019

Version: 1.1.1.

Section 1: Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name: Silicon Dioxide, Enriched in Silicon.

Molecular weight: 60.08

Chemical formula: SiO2

Synonyms: None.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112 – begär Giftinformation.

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to: Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:                                  Hazard statement:

Not classified as hazardous.                                       None required.

Label elements:

Hazard pictograms: Not required.

Signal Words: Not required.

Hazard Statements: Not required.

Precautionary Statements: None.

Other hazards: None known.

Safety Data Sheet (SDS) for Silicon Dioxide, Enriched in Silicon        https://www.buyisotope.com                    Page 1/8

Page 2

Safety Data Sheet for Silicon Dioxide,

Enriched Silicon Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 25-Aug-2019

Version: 1.1.1.

Section 3: Composition/Information on Ingredients

Substance/Mixture: Substance.

Ingredients:

CAS-No.             Molecular     Concentration               Classification

Substance name (IUPAC/EC)

EC-No.              weight        % by weight                EC1272/2008

7631-86-9

Silicon dioxide                                                    60. 08            >99%                  Not Classified.

231-545-4

For explanation of abbreviations see Section 16.

Section 4: First-Aid Measures

Description of first aid measures:

In case of inhalation: Remove to fresh air.

In case of skin contact: Wash affected area with mild soap and water.

In case of eye contact: Flush eyes with lukewarm water, lifting upper and lower eyelids, for at least 15 minutes. Seek

medical attention symptoms persist.

Most important symptoms and effects, both acute and delayed:

Inhalation: May cause pneumoconiosis, pulmonary fibrosis and silicosis.

Eyes: May cause abrasive irritation.

Skin contact: May cause abrasive irritation.

Indication of any immediate medical attention and special treatment needed: Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Section 5: Fire-Fighting Measures

Extinguisher media:

Suitable extinguisher media: Use suitable extinguishing media for any type of fire involving surrounding materials.

Unsuitable extinguishing media: None known.

Special hazards arising from the mixture: Fumes from fire are hazardous.

Advice for fire-fighters: Firefighters must wear full face, self-contained breathing apparatus with full protective

clothing to prevent contact with skin and eyes.

Further information: Standard procedure for chemical fires. Use extinguishing measures that are appropriate to local

circumstances and the surrounding environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions: Wear appropriate respiratory and protective equipment specified in section 8. Isolate spill area

and provide ventilation. Avoid dust formation. Avoid breathing vapours, mist or gas.

Environmental precautions: Prevent product from entering drains. Prevent further leakage or spillage if safe to do so.

Local authorities should be advised if significant spillages cannot be contained.

Safety Data Sheet (SDS) for Silicon Dioxide, Enriched in Silicon       https://www.buyisotope.com                       Page 2/8

Page 3

Safety Data Sheet for Silicon Dioxide,

Enriched Silicon Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 25-Aug-2019

Version: 1.1.1.

Methods for containment and cleaning up:

Methods for cleaning up: Vacuum up spill using a high efficiency particulate absolute (HEPA) air filter and place in a

closed container for proper disposal. Take care not to raise dust.

Reference to other sections:

Treat recovered material as described in the section "Disposal considerations".

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling: Wash thoroughly after handling. Avoid contact with skin and eyes. Avoid formation of dust

and aerosols. Provide appropriate exhaust ventilation at places where dust is formed.

Hygiene measures: Do not eat, drink or smoke when using this product.

Conditions for safe storage, including incompatibilities:

Requirements for storage areas and containers: Store in cool, dry well-ventilated area. Store in tightly sealed

container.

Section 8: Exposure Controls/Personal Protection

Control parameters:

Occupational exposure limits: Contains no substances with occupational exposure limit values.

Exposure controls:

Appropriate engineering controls: Implement engineering and work practice controls to reduce and maintain

concentration of exposure at low levels.

Individual protection measures, such as personal protective equipment:

Eye/face protection: Use safety glasses. If there is a potential for exposure to particles which could cause eye

discomfort, wear chemical goggles. Maintain eye wash fountain and quick-drench facilities in work area.

Hand protection: Use chemical resistant gloves. Examples of preferred glove barrier materials include: Butyl rubber,

Neoprene, Nitrile/butadiene rubber, Polyethylene, Ethyl vinyl alcohol laminate, polyvinyl alcohol, Polyvinyl chloride.

Body protection: Protective gear suitable to prevent contamination.

Respiratory protection: NIOSH-approved respirator.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form): Solid (powder).

Colour: Transparent to grey.

Odour: Odourless.

Odour threshold: No data available.

Molecular Weight: 60.08

pH (concentration): No data available.

Melting point/range (°C): 1716 - 1736 °C

Boiling point/range (°C): 2230 °C

Safety Data Sheet (SDS) for Silicon Dioxide, Enriched in Silicon   https://www.buyisotope.com                    Page 3/8

Page 4

Safety Data Sheet for Silicon Dioxide,

Enriched Silicon Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 25-Aug-2019

Version: 1.1.1.

Freezing point (°C): No data available.

Flash point (°C): No data available.

Evaporation rate: No data available.

Flammability (solid, gas): No data available.

Ignition temperature (°C): No data available.

Upper/lower flammability/explosive limits: No data available.

Vapour pressure (20 °C): 10 mm at 1732 °C

Vapour density: No data available.

Relative density (25 °C): 2.2

Water solubility (g/L) at 20 °C: Insoluble.

n-Octanol/Water partition coefficient: No data available.

Auto-ignition temperature: No data available.

Decomposition temperature: No data available.

Viscosity, dynamic (mPa s): No data available.

Explosive properties: The substance or mixture is not classified as explosive.

Oxidising properties: The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Reactivity: No dangerous reaction known under conditions of normal use.

Chemical stability: Stable under normal conditions.

Possibility of hazardous reactions: No dangerous reaction known under conditions of normal use.

Conditions to avoid: None known.

Incompatible materials: Strong oxidizing agents, strong basis, hydrofluoric acid, fluorine, oxygen difluoride, chlorine

trifluoride and manganese trioxide.

Hazardous decomposition products: On contact with hydrofluoric acid, silicon dioxide may emit silicon tetrafluoride.

Section 11: Toxicological Information

Information on toxicological effects:

Acute Toxicity:

LD50 Oral - Rat - 3,160 mg/kg

LD50 Oral - Rat - 7,500 mg/kg.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

IARC 3: Group 3: Not classifiable as to its carcinogenicity to humans (Silicon dioxide)

ACGIH: No component of this product present at levels greater than or equal to 0.1% is identified as a carcinogen or

potential carcinogen by ACGIH.

Safety Data Sheet (SDS) for Silicon Dioxide, Enriched in Silicon   https://www.buyisotope.com                   Page 4/8

Page 5

Safety Data Sheet for Silicon Dioxide,

Enriched Silicon Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 25-Aug-2019

Version: 1.1.1.

NTP: No component of this product present at levels greater than or equal to 0.1% is identified as a known or

anticipated carcinogen by NTP.

OSHA: No component of this product present at levels greater than or equal to 0.1% is identified as a carcinogen or

potential carcinogen by OSHA.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity – single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT) – repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity: No data available.

Persistence and degradability: No data available.

Bioaccumulative potential: No data available.

Mobility in soil: No data available.

Results of PBT& vPvB assessment: Not relevant.

Other adverse effects: No data available.

Section 13: Disposal Considerations

Waste treatment methods: Dispose of in accordance with local, state and federal regulations. Offer surplus and non-

recyclable solutions to a licensed disposal company. Contact a licensed professional waste disposal service to dispose

of this material.

Section 14: Transport Information

UN number: Not regulated as a dangerous good.

UN proper shipping name: Not regulated as a dangerous good.

Transport hazard class(es): Not regulated as a dangerous good.

Packing group: Not regulated as a dangerous good.

Environmental hazards: Not regulated as a dangerous good.

Special precautions for user: Not dangerous goods in the meaning of ADR/RID, ADN, IMDG-Code, ICAO/IATA-DGR.

Transport in bulk according to Annex II of Marpol and the IBC Code: Not applicable.

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Safety Data Sheet (SDS) for Silicon Dioxide, Enriched in Silicon   https://www.buyisotope.com                  Page 5/8

Page 6

Safety Data Sheet for Silicon Dioxide,

Enriched Silicon Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 25-Aug-2019

Version: 1.1.1.

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations: The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

OSHA Hazards: Target Organ Effect, Irritant.

SARA 302 Components: No chemicals in this material are subject to the reporting requirements of SARA Title III,

Section 302.

SARA 313 Components: This material does not contain any chemical components with known CAS numbers that

exceed the threshold (de minimis) reporting levels established by SARA Title III, Section 313.

Safety Data Sheet (SDS) for Silicon Dioxide, Enriched in Silicon   https://www.buyisotope.com                  Page 6/8

Page 7

Safety Data Sheet for Silicon Dioxide,

Enriched Silicon Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 25-Aug-2019

Version: 1.1.1.

SARA 311/312 Hazards: Acute Health Hazard, Chronic Health Hazard.

Massachusetts Right to Know Components: Silicon dioxide / CAS No. 7631-86-9 / Revision Date 1993-04-24

Pennsylvania Right to Know Components: Silicon dioxide / CAS No. 7631-86-9 / Revision Date 1993-04-24

New Jersey Right to Know Components: Silicon dioxide / CAS No. 7631-86-9 / Revision Date 1993-04-24

California Prop. 65 Components: This product does not contain any chemicals known to the State of California to

cause cancer, birth defects, or any other reproductive harm.

National regulations: Follow national regulation for work with chemical agents.

Chemical safety assessment: No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

Safety Data Sheet (SDS) for Silicon Dioxide, Enriched in Silicon   https://www.buyisotope.com                  Page 7/8

Page 8

Safety Data Sheet for Silicon Dioxide,

Enriched Silicon Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 25-Aug-2019

Version: 1.1.1.

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

None.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.

Safety Data Sheet (SDS) for Silicon Dioxide, Enriched in Silicon   https://www.buyisotope.com                  Page 8/8