Tellurium-123 SDS

Publisher: Neonest AB

Source: https://resds.com/document/tellurium-123-sds-te-123-ab061cb1bb

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Tellurium Oxide,

Enriched Tellurium Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Section 1: Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name: Tellurium Oxide, Enriched Tellurium Oxide.

Molecular weight: 159.6

Chemical formula: TeO2

Synonyms: Tellurium Dioxide.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112 – begär Giftinformation.

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to: Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:                              Hazard statement:

Not classified as hazardous.                                   None required.

Label elements:

Hazard pictograms: Not required.

Signal Words: Not required.

Hazard Statements: Not required.

Precautionary Statements: None.

Other hazards: None known.

Safety Data Sheet (SDS) for Tellurium Oxide, Enriched Tellurium Oxide   https://www.buyisotope.com                Page 1/9

Page 2

Safety Data Sheet for Tellurium Oxide,

Enriched Tellurium Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Section 3: Composition/Information on Ingredients

Substance/Mixture: Substance.

Ingredients:

CAS-No.            Molecular         Concentration               Classification

Substance name (IUPAC/EC)

EC-No.             weight            % by weight                EC1272/2008

Acute Tox. 4     H302

Skin Corr. 1A    H314

7446-07-3                                                   Skin Sens. 1A    H317

Eye Dam. 1       H318

Acute Tox. 4     H332

Tellurium dioxide                                                 159.6               >99%

Carc. 1A H350 H350

Repr. 1A         H360

231-193-1                                                   STOT RE 1        H372

Aquatic Acute 1 H400

Aquatic Chronic 2H411

For explanation of abbreviations see Section 16.

Section 4: First-Aid Measures

Description of first aid measures:

In case of inhalation: If breathed in, move person into fresh air. If not breathing, give artificial respiration. In case of

discomfort seek medical attention.

In case of skin contact: Wash off with soap and plenty of water for at least 15 minutes. In case of discomfort seek

medical attention.

In case of eye contact: Flush eyes thoroughly with water for 15 minutes. Remove contact lenses after the initial 1-2

minutes and continue flushing. In case of discomfort seek medical attention.

In case of ingestion: Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth

with water. In case of discomfort seek medical attention.

Most important symptoms and effects, both acute and delayed:

Inhalation: Inhalation of dust may produce irritation, headache, metallic taste and/or cough. Severe exposures may

produce shortness of breath, chest pain and flu-like symptoms: weakness, fever, headache, chills, sweating, nausea

and muscular pain.

Eyes: May cause irritation, redness, and pain.

Skin contact: May cause irritation, redness, and pain.

Ingestion: Symptoms can include severe nausea, vomiting, diarrhoea, abdominal pain, choking, dizziness and

salivation.

Indication of any immediate medical attention and special treatment needed: Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Section 5: Fire-Fighting Measures

Extinguisher media:

Safety Data Sheet (SDS) for Tellurium Oxide, Enriched Tellurium Oxide      https://www.buyisotope.com                    Page 2/9

Page 3

Safety Data Sheet for Tellurium Oxide,

Enriched Tellurium Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Suitable extinguisher media: Use any means suitable for extinguishing surrounding fire.

Unsuitable extinguishing media: None known.

Special hazards arising from the mixture: None known.

Advice for fire-fighters: Wear self-contained breathing apparatus and protective clothing for firefighting.

Further information: Standard procedure for chemical fires. Use extinguishing measures that are appropriate to local

circumstances and the surrounding environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions: Wear appropriate protective clothing. Avoid generation of dusts.

Environmental precautions: Prevent product from entering drains. Prevent further leakage or spillage if safe to do so.

Local authorities should be advised if significant spillages cannot be contained.

Methods for containment and cleaning up:

Methods for cleaning up: Avoid generation of dusts. Carefully take up dry product. Forward for disposal. Clean up

affected area.

Reference to other sections:

Treat recovered material as described in the section "Disposal considerations".

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling: Avoid generation of dusts. Change contaminated clothing. Wash hands after working with

substance.

Hygiene measures: Do not eat, drink or smoke when using this product.

Conditions for safe storage, including incompatibilities:

Requirements for storage areas and containers: Store at room temperature (15-25 °C recommended). Keep well

closed and protected from direct sunlight and moisture.

Section 8: Exposure Controls/Personal Protection

Control parameters:

Occupational exposure limits:

INHALATION Exposure

Long-term: (DNEL) 580 µg/m³

DERMAL Exposure

Long-term: (DNEL) 830 µg/kg bw/day

Exposure controls:

Appropriate engineering controls: A system of local and/or general exhaust is recommended to keep employee

exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control

the emissions of the contaminant at its source, preventing dispersion into the general work area.

Safety Data Sheet (SDS) for Tellurium Oxide, Enriched Tellurium Oxide   https://www.buyisotope.com               Page 3/9

Page 4

Safety Data Sheet for Tellurium Oxide,

Enriched Tellurium Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Individual protection measures, such as personal protective equipment:

Eye/face protection: Use safety glasses. If there is a potential for exposure to particles which could cause eye

discomfort, wear chemical goggles.

Hand protection: Use chemical resistant gloves. Examples of preferred glove barrier materials include: Butyl rubber,

Neoprene, Nitrile/butadiene rubber, Polyethylene, Ethyl vinyl alcohol laminate, polyvinyl alcohol, Polyvinyl chloride.

Body protection: Plastic apron, sleeves, boots (if handling large quantities).

Respiratory protection: If the exposure limit is exceeded and engineering controls are not feasible, a half-face high-

efficiency particulate respirator (NIOSH type N100 filter) may be worn for up to ten times the exposure limit or the

maximum use concentration specified by the appropriate regulatory agency or respirator supplier, whichever is

lowest. A full-face piece high efficiency particulate respirator (NIOSH type N100 filter) may be worn up to 50 times the

exposure limit, or the maximum use concentration specified by the appropriate regulatory agency or respirator

supplier, whichever is lowest. If oil particles (e.g. lubricants, cutting fluids, glycerine, etc.) are present, use a NIOSH

type R or P filter. For emergencies or instances where the exposure levels are not known, use a full-facepiece positive-

pressure, air-supplied respirator.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form): Solid.

Colour: White.

Odour: Odourless.

Odour threshold: No data available.

Molecular Weight: 159.6

pH (concentration): No data available.

Melting point/range (°C): 733 °C

Boiling point/range (°C): 1245 °C

Freezing point (°C): No data available.

Flash point (°C): No data available.

Evaporation rate: No data available.

Flammability (solid, gas): No data available.

Ignition temperature (°C): No data available.

Upper/lower flammability/explosive limits: No data available.

Vapour pressure (20 °C): No data available.

Vapour density: No data available.

Relative density (25 °C): 6.02

Water solubility (g/L) at 20 °C: Insoluble.

n-Octanol/Water partition coefficient: No data available.

Auto-ignition temperature: No data available.

Decomposition temperature: No data available.

Viscosity, dynamic (mPa s): No data available.

Explosive properties: The substance or mixture is not classified as explosive.

Oxidising properties: The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Reactivity: No dangerous reaction known under conditions of normal use.

Chemical stability: Stable under normal conditions.

Safety Data Sheet (SDS) for Tellurium Oxide, Enriched Tellurium Oxide   https://www.buyisotope.com                Page 4/9

Page 5

Safety Data Sheet for Tellurium Oxide,

Enriched Tellurium Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Possibility of hazardous reactions: No dangerous reaction known under conditions of normal use.

Conditions to avoid: None known.

Incompatible materials: Strong acids, strong oxidizing agents.

Hazardous decomposition products: None known.

Section 11: Toxicological Information

Information on toxicological effects:

Acute Toxicity:

LD50 Oral, Rat - > 5,000 mg/kg

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Causes severe skin burns and eye damage.

Serious eye damage/eye irritation:

Causes serious eye damage.

Respiratory or skin sensitisation:

May cause an allergic skin reaction.

Germ cell mutagenicity:

No data available.

Carcinogenicity:

May cause cancer.

Reproductive toxicity:

May damage fertility or the unborn child.

Specific target organ toxicity – single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT) – repeated exposure:

Causes damage to organs through prolonged or repeated exposure.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:

Toxicity to fish: LC50 (4 days) 37.1 - 100 mg/L

Short–term toxicity to aquatic invertebrates:

EC50 (48 h) 5.79 - 7.24 mg/L

NOEC (48 h) 3.64 - 4.55 mg/L

Persistence and degradability: No data available.

Bioaccumulative potential: No data available.

Mobility in soil: No data available.

Results of PBT& vPvB assessment: Not relevant.

Other adverse effects: Very toxic to aquatic life. Toxic to aquatic life with long lasting effects.

Safety Data Sheet (SDS) for Tellurium Oxide, Enriched Tellurium Oxide   https://www.buyisotope.com                Page 5/9

Page 6

Safety Data Sheet for Tellurium Oxide,

Enriched Tellurium Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Section 13: Disposal Considerations

Waste treatment methods: Chemical residues are generally classified as special waste, and as such are covered by

regulations which vary according to location. Contact your local waste disposal authority for advice, or pass to a

chemical disposal company.

Section 14: Transport Information

UN number: Not regulated as a dangerous good.

UN proper shipping name: Not regulated as a dangerous good.

Transport hazard class(es): Not regulated as a dangerous good.

Packing group: Not regulated as a dangerous good.

Environmental hazards: Not regulated as a dangerous good.

Special precautions for user: Not dangerous goods in the meaning of ADR/RID, ADN, IMDG-Code, ICAO/IATA-DGR.

Transport in bulk according to Annex II of Marpol and the IBC Code: Not applicable.

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Safety Data Sheet (SDS) for Tellurium Oxide, Enriched Tellurium Oxide   https://www.buyisotope.com               Page 6/9

Page 7

Safety Data Sheet for Tellurium Oxide,

Enriched Tellurium Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations: The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

OSHA Hazards: Carcinogen, Target Organ Effect, Teratogen, Mutagen.

SARA 302 Components: No chemicals in this material are subject to the reporting requirements of SARA Title III,

Section 302.

SARA 313 Components: This material does not contain any chemical components with known CAS numbers that

exceed the threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 Hazards: Acute Health Hazard, Chronic Health Hazard.

Massachusetts Right To Know Components: No components are subject to the Massachusetts Right to Know Act.

Pennsylvania Right To Know Components: Tellurium dioxide / CAS No. 7446-07-3

New Jersey Right To Know Components: Tellurium dioxide / CAS No. 7446-07-3

California Prop. 65 Components: This product does not contain any chemicals known to the State of California to

cause cancer, birth defects, or any other reproductive harm.

National regulations: Follow national regulation for work with chemical agents.

Chemical safety assessment: No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

Safety Data Sheet (SDS) for Tellurium Oxide, Enriched Tellurium Oxide   https://www.buyisotope.com               Page 7/9

Page 8

Safety Data Sheet for Tellurium Oxide,

Enriched Tellurium Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

Safety Data Sheet (SDS) for Tellurium Oxide, Enriched Tellurium Oxide   https://www.buyisotope.com               Page 8/9

Page 9

Safety Data Sheet for Tellurium Oxide,

Enriched Tellurium Oxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H302 Harmful if swallowed.

H314 Causes severe skin burns and eye damage.

H317 May cause an allergic skin reaction.

H318 Causes serious eye damage.

H332 Harmful if inhaled.

H350 May cause cancer.

H360 May damage fertility or the unborn child.

H372 Causes damage to organs through prolonged or repeated exposure.

H400 Very toxic to aquatic life.

H411 Toxic to aquatic life with long lasting effects.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.

Safety Data Sheet (SDS) for Tellurium Oxide, Enriched Tellurium Oxide   https://www.buyisotope.com               Page 9/9