Terbium (III) sulfate octahydrate 3493 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/terbium-iii-sulfate-octahydrate-prochem-en-fdda2f76a5

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Terbium (III) sulfate octahydrate

PRODUCT NUMBER:  3493

CAS NUMBER:        13842-67-6

MANUFACTURER:

ProChem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL 61109                                      TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315              Causes skin irritation.

H319              Causes serious eye irritation.

H335             May cause serious respiratory irritation.

Precautionary Phrases

P261                Avoid breathing dust/fume/gas/mist/vapors/spray.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338     IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

HMIS CLASSIFICATION:

Health: 0            Flammability: 0                  Reactivity: 0

NFPA RATING:

Health: 0              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: Causes serious irritation of the eyes.

SKIN CONTACT: Causes irritation of the skin.

INHALATION:  Irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: May be irritating if swallowed.

TERBIUM (III) SULFATE OCTAHYDRATE                                                             PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Tb2(SO4)3!8H2O

Molecular Weight: 750.16

CHEMICAL NAME                                           CAS#                               %

Terbium (III) sulfate octahydrate                                           13842-67-6                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under eyelids. A

victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if irritation

persists.

INHALATION: Remove victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty in breathing,

coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not applicable

AUTO IGNITION TEMPERATURE: Not applicable

EXPLOSION LIMITS: Not applicable

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: If involved in a fire, fire fighters should be equipped with a NIOSH approved

positive self-contained breathing apparatus, flame and chemical resistant clothing, hats, boots, and gloves.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Sulfur oxides and terbium oxides.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear approved personal protective equipment. Avoid dust formation. Avoid breathing dust, vapors, mist

or gas. Ensure adequate ventilation.

ENVIRONMENTAL PRECAUTIONS: Do not let product enter drains or allow to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and place in suitable container for proper

disposal. Avoid generation of dust.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear approved personal protective equipment. Avoid contact with skin and eyes. Avoid

formation of dust. Ensure adequate ventilation.

CONDITIONS FOR SAFE STORAGE: Store material in a tightly sealed container in a dry and well-ventilated area. Hygroscopic.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when handling a chemical

substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

VENTILATION: Handle in an efficient chemical fume hood.

RESPIRATOR:  If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White powder

MOLECULAR WEIGHT: 750.16

BOILING POINT: No data available

MELTING POINT: 360°C

TERBIUM (III) SULFATE OCTAHYDRATE                                                             PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES (Cont.)

SPECIFIC GRAVITY: No data available

VAPOR PRESSURE: No data

SOLUBILITY IN WATER: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Hygroscopic

HAZARDOUS POLYMERIZATION: No hazardous polymerization

CONDITIONS TO AVOID: Moisture

INCOMPATIBILITY: Strong oxidizing agents

DECOMPOSITION PRODUCTS: Sulfur oxides and terbium oxides

SECTION 11 TOXICOLOGICAL DATA

RTECS DATA: No information available in the RTECS files.

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified as a confirmed

carcinogen by IARC, ACGIH, NTP, and OSHA.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: No data available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 313 COMPONENTS: This material does not contain any chemical components with know CAS numbers that exceed the

threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

SARA 302 COMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA 311/312 HAZARDS: Acute Health Hazard

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right To Know Act.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE REVISED: 01/25

SDS DEPT.

TERBIUM (III) SULFATE OCTAHYDRATE                                                             PAGE 3