Terbium, powder, -40 Mesh 3457 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/terbium-powder-40-mesh-prochem-en-eb1a09ba47

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Terbium, powder, -40 Mesh

PRODUCT NUMBER:  3457

CAS NUMBER:        7440-27-9

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228               Flammable solid

H260                    In contact with water releases flammable gases which may ignite spontaneously.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces. No smoking.

P231+P232          Handle under inert gas. Protect from moisture.

P241              Use explosion-proof electrical/ventilating/lighting/equipment

P370+P378             In case of fire: Use for extinction: Special powder for metal fires.

P402+P404           Store in a dry place. Store in a closed container.

P501                Dispose of contents/container in accordance with loca/regional/national/international regulations.

HMIS CLASSIFICATION:

Health: 1              Fire: 3                  Reactivity Hazard: 2

NFPA RATING:

Health: 1            Flammability: 3         Reactivity Hazard: 2

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: Inhalation of fine dust may be irritating to the nasal cavity, mucous membranes, and respiratory tract.

INGESTION: No specific information on this physiological effects of ingestion.

TERBIUM, POWDER, -40 MESH                                                             PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Tb

Molecular Weight: 158.92

CHEMICAL NAME                                           CAS#                               %

Terbium                                                                 7440-27-9                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Dry chemical or class D metal fire agent.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Terbium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Keep away from

sources of ignition

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allow to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: In the case of a spill, the fine powder should be covered

with sodium carbonate or dolomite in order to reduce the formation of dust. Sweep up with mixture.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Keep container closed when not in use. Handle under dry protective gas. Protect against electrostatic charges.

CONDITIONS FOR SAFE STORAGE: Store material in a tightly sealed container in a cool dry place under argon. Handle in argon.

Store away from air, water, moisture, oxidizing agents, halogens, and acid chlorides.  Moisture and air sensititive.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves and a flame resistant suit.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Silver-gray powder

ODOR: None

MOLECULAR WEIGHT: 158.92

BOILING POINT: 3230° C

TERBIUM, POWDER, -40 MESH                                                             PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES (Cont.)

MELTING POINT: 1356° C

SPECIFIC GRAVITY: 8.234 gm/ml

VAPOR DENSITY: No data available

SOLUBILITY: Reacts with water.

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Contact with water releases flammable gases. Reacts with strong oxidizing agents.

CONDITIONS TO AVOID: Contact with hot water, mixtures of fine dust with the air.

INCOMPATIBILITY: Oxidizing agents, acids, air, halogens, acid chlorides, and water.

DECOMPOSITION PRODUCTS: Terbium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

DOT:  UN3089                                      IATA:  UN3089

Metal Powders, flammable, n.o.s.                           Metal Powder, flammable, n.o.s.

(Terbium)                                                 (Terbium)

Class 4.1                                                Class 4.1

PG II                                    PG II

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

DSCL (EEC): Listed on the DSCL inventory

SARA 302/304: Not Listed

SARA 311/312: Not available

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

WHMIS CANADA: Class B-4: Flammable Solid. Class B-6: Reactive Flammable material.

TERBIUM, POWDER, -40 MESH                                                             PAGE 3

Page 4

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 07/14

TERBIUM, POWDER, -40 MESH                                                             PAGE 4