Thulium (III) acetate hydrate 3590 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/thulium-iii-acetate-hydrate-prochem-en-3bca4f86f4

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Thulium (III) acetate hydrate

PRODUCT NUMBER:  3590

CAS NUMBER:       314041-04-8

MANUFACTURER:

ProChem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                    TOLL FREE:    800-795-8788

Rockford, IL 61109

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315                    Causes skin irritation.

H319                    Causes serious eye irritation.

H335                  May cause respiratory irritation.

Precautionary Phrases

P261                       Avoid breathing dust/fume/gas/mist/vapors/spray.

P264                  Wash face, hands, and any exposed skin thoroughly after handling.

P271                   Use only outdoors or in a well-ventilated area.

P280                   Wear protective gloves/protective clothing/eye protection/face protection.

P302 + P352                  IF ON SKIN: Wash with plenty of soap and water.

P304 + P340 + P312          IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. Call

a POISON CENTER or doctor/physician if you feel unwell.

P305 + P351 + P338          IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and

easy to do. Continue rinsing.

P403 + P233                 Store in a well-ventilated place. Keep container tightly closed.

THULIUM (III) ACETATE                                                                         PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

EYE CONTACT: Causes serious eye irritation.

SKIN CONTACT: Causes skin irritation.

INHALATION: May be irritation to the nose, mucous membranes, and respiratory tract.

INGESTION: May cause respiratory irritation.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula:  Tm(OOCCH3)3 · XH2O

Molecular Weight: 346.07 (anhy)

CHEMICAL NAME                                         CAS#                               %

Thulium (III) acetate hydrate                                              314041-04-8                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Check for and remove contact lenses. In case of contact, immediately flush eyes with plenty of water for at

least 15 minutes. Get medical attention.

SKIN EXPOSURE: In case of contact, immediately flush skin with plenty of water.   If skin irritation persists, call a physician.

INHALATION: Remove to fresh air.  If not breathing, give artificial respiration. Get medical attention.

INGESTION: Clean out mouth with water. Drink plenty of water afterwards. Get medical attention if symptoms occur.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not applicable

AUTO IGNITION TEMPERATURE: Not applicable

EXPLOSION LIMITS: None

EXTINGUISHING MEDIUM: Carbon dioxide, powder, water spray. In case of major fire and large quantities: Evacuate area. Fight fire

remotely due to the risk of explosion.

SPECIAL FIRE FIGHTING PROCEDURES: Keep product and empty container away from heat and sources of ignition.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Carbon monoxide, carbon dioxide, and metal oxides.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Always wear appropriate protective equipment. Ensure adequate ventilation. Avoid dust formation.

ENVIRONMENTAL PRECAUTIONS: Not available

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and shovel into suitable containers for proper

disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Do not breathe dust. Wear suitable protective clothing. Avoid dust formation.

CONDITIONS FOR SAFE STORAGE: Store under an inert atmosphere. Keep container tightly closed in a dry and well-ventilated

area. Protect from moisture.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses w/side shields, or safety goggles, face shield when handling a chemical

substance in the laboratory.

SKIN PROTECTION: Chemical-resistant gloves.

VENTILATION: Use process enclosures, local exhaust ventilation, or other engineering controls to keep airborne levels below

recommended exposure limits.  If user operations generate dust, fume or mist, use ventilation to keep exposure to airborne

contaminants below the exposure limit.

RESPIRATOR:  If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

THULIUM (III) ACETATE                                                                         PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Off-white solid

MOLECULAR WEIGHT: 346.07 (anhy)

MELTING POINT: No data available

SPECIFIC GRAVITY: No data available

BOILING POINT: No data available

VAPOR PRESSURE: No data available

SOLUBILITY IN WATER: No data available

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Exposure to moisture and water.

INCOMPATIBILITY: Oxidizing agents

DECOMPOSITION PRODUCTS: Carbon monoxide, carbon dioxide, and metal oxides.

SECTION 11 TOXICOLOGICAL DATA

CARCINOGENIC EFFECTS: No component of this product presents at levels ≥ 0.1% is identified as probable, possible or confirmed

carcinogen by IARC.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

DEVELOPMENTAL TOXICITY: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local state and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

CALIFORNIA PROP 65: The product does not contain any Proposition 65 chemicals.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc. makes

no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the product based

on the information.

DATE PREPARED: 06/22

SDS DEPT.

THULIUM (III) ACETATE                                                                         PAGE 3