Titanium-49 SDS

Publisher: Neonest AB

Source: https://resds.com/document/titanium-49-isotope-sds-ti-49-49ti-titanium-dioxide-enriched-titanium-dioxide-49b642bdac

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Titanium Dioxide,

Enriched Titanium Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Titanium Dioxide, Enriched Titanium   https://www.buyisotope.com   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Titanium Dioxide, Enriched Titanium.

Molecular weight:   79.87

Chemical formula:   TiO 2

Synonyms:   Titanium (IV) Oxide, Titania.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Not classified as hazardous.   None required.

Label elements:

Hazard pictograms:   Not required.

Signal Words:   Not required.

Hazard Statements:   Not required.

Precautionary Statements:   None.

Other hazards:   None known.

Page 2

Safety Data Sheet for Titanium Dioxide,

Enriched Titanium Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Titanium Dioxide, Enriched Titanium   https://www.buyisotope.com   Page 2/9

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Molecular

weight

Concentration

% by weight

Classification

EC1272/2008 EC - No.

Titanium dioxide

13463-67-7

79.87   >99%   Not Classified.

236-675-5

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

In case of inhalation:   Remove to fresh air. Get medical attention for any breathing difficulty.

In case of skin contact:   Immediately flush skin with plenty of soap and water for at least 15 minutes. Remove

contaminated clothing and shoes. Wash clothing before reuse. Thoroughly clean shoes before reuse. Get medical

attention if irritation develops.

In case of eye contact:   Immediately flush eyes with plenty of water for at least 15 minutes, lifting upper and lower

eyelids occasionally. Get medical attention if irritation persists.

In case of ingestion:   Not expected to require first aid measures. If large amounts were swallowed, give water to drink

and get medical advice.

Most important symptoms and effects, both acute and delayed:

Inhalation:   May cause mild irritation to the respiratory tract.

Eyes:   May cause mild irritation, possible reddening.

Skin contact:   May cause mild irritation and redness.

Ingestion:   Not expected to be a health hazard exposure route.

Indication of any immediate medical attention and special treatment needed:   Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Section 5: Fire - Fighting Measures

Extinguisher media:

Suitable extinguisher media:   Use any means suitable for extinguishing surrounding fire.

Unsuitable extinguishing media:   None known.

Special hazards arising from the mixture:   None known.

Advice for fire-fighters:   In the event of a fire, wear full protective clothing and NIOSH-approved self-contained

breathing apparatus with full facepiece operated in the pressure-demand or other positive-pressure mode.

Further information:   Standard procedure for chemical fires. Use extinguishing measures that are appropriate to local

circumstances and the surrounding environment.

Page 3

Safety Data Sheet for Titanium Dioxide,

Enriched Titanium Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Titanium Dioxide, Enriched Titanium   https://www.buyisotope.com   Page 3/9

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions:   Wear appropriate personal protective equipment as specified in Section 8. Ventilate area of

leak or spill. Avoid dust formation. Avoid breathing vapours, mist or gas.

Environmental precautions:   Prevent product from entering drains. Prevent further leakage or spillage if safe to do so.

Local authorities should be advised if significant spillages cannot be contained.

Methods for containment and cleaning up:

Methods for cleaning up:   Sweep up and containerize for reclamation or disposal. Vacuuming or wet sweeping may be

used to avoid dust dispersal.

Reference to other sections:

Treat recovered material as described in the section "Disposal considerations".

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling:   Further processing of solid materials may result in the formation of combustible dusts. The

potential for combustible dust formation should be taken into consideration before additional processing occurs.

Provide appropriate exhaust ventilation at places where dust is formed.

Hygiene measures:   Do not eat, drink or smoke when using this product.

Conditions for safe storage, including incompatibilities:

Requirements for storage areas and containers:   Keep in a tightly closed container, stored in a cool, dry, ventilated

area. Protect against physical damage. Containers of this material may be hazardous when empty since they retain

product residues (dust, solids); observe all warnings and precautions listed for the product.

Section 8: Exposure Controls/ Personal Protection

Control parameters:

Occupational exposure limits:

Airborne Exposure Limits

OSHA Permissible Exposure Limit (PEL): 15 mg/m 3   (TWA)

ACGIH Threshold Limit Value (TLV): 10 mg/m 3   (TWA), A4 - Not classifiable as a human carcinogen.

Exposure controls:

Appropriate engineering controls:   A system of local and/or general exhaust is recommended to keep employee

exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control

the emissions of the contaminant at its source, preventing dispersion of it into the general work area.

Individual protection measures, such as personal protective equipment:

Eye/face protection:   Use chemical safety goggles and/or full-face shield where dusting or splashing of solutions is

possible. Maintain eye wash fountain and quick drench facilities in work area.

Hand protection:   Use chemical resistant gloves. Examples of preferred glove barrier materials include: Butyl rubber,

Neoprene, Nitrile/butadiene rubber, Polyethylene, Ethyl vinyl alcohol laminate, polyvinyl alcohol, Polyvinyl chloride.

Page 4

Safety Data Sheet for Titanium Dioxide,

Enriched Titanium Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Titanium Dioxide, Enriched Titanium   https://www.buyisotope.com   Page 4/9

Body protection:   Wear protective gloves and clean body-covering clothing.

Respiratory protection:   If the exposure limit is exceeded, a half-face dust/mist respirator may be worn for up to ten

times the exposure limit or the maximum-use concentration specified by the appropriate regulatory agency or

respirator supplier, whichever is lowest. A full-facepiece dust/mist respirator may be worn up to 50 times the

exposure limit, or the maximum-use concentration specified by the appropriate regulatory agency, or respirator

supplier, whichever is lowest. For emergencies or instances where the exposure levels are not known, use a full-

facepiece positive-pressure, air-supplied respirator.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Solid (powder).

Colour:   White.

Odour:   Odourless.

Odour threshold:   No data available.

Molecular Weight:   79.87

pH (concentration):   6-7

Melting point/range (° C):   1855   °C

Boiling point/range (° C):   2500-3000   °C

Freezing point   (°C):   No data available.

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   No data available.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour   pressure (20 ° C):   No data available

Vapour density:   No data available.

Relative densi ty (25 ° C):   No data available.

Water solubility (g/L ) at 20 ° C:   Insoluble in water.

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Reactivity:   No dangerous reaction known under conditions of normal use.

Chemical stability:   Stable under normal conditions.

Possibility of hazardous reactions:   No dangerous reaction known under conditions of normal use.

Conditions to avoid:   D ust generation .

Incompatible materials:   For Titanium Dioxide: A violent reaction with lithium occurs around 200   ° C (392   ° F) with a

flash of light; the temperature can reach 900   ° C. Violent or incandescent reaction may also occur with other metals

such as aluminium, calcium, magnesium, potassium, sodium or zinc

Hazardous decomposition products:   None known.

Page 5

Safety Data Sheet for Titanium Dioxide,

Enriched Titanium Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Titanium Dioxide, Enriched Titanium   https://www.buyisotope.com   Page 5/9

Section 11: Toxicological Information

Information on toxicological effects:

Not classified based on available information.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

NIOSH: NIOSH considers this substance to be a potential occupational carcinogen.

NTP: This substance is not known to be and is not anticipated to be a carcinogen.

IARC: Category 3.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:

Toxicity to Fish: LC 50   Other Fish, 1,000 mg/l, 96 hours.

Toxicity to Daphnia and Other Aquatic Invertebrates:

EC50   Daphnia magna , > 1,000 mg/l, 48 hours.

EC0   Daphnia magna,   1,000 mg/l, 48 hours.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   Not relevant.

Other adverse effects:   No data available.

Section 13: Disposal Considerations

Waste treatment methods:   Whatever cannot be saved for recovery or recycling should be managed in an appropriate

and approved waste disposal facility. Processing, use or contamination of this product may change the waste

management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of

unused product in accordance with local and national requirements.

Page 6

Safety Data Sheet for Titanium Dioxide,

Enriched Titanium Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Titanium Dioxide, Enriched Titanium   https://www.buyisotope.com   Page 6/9

Section 14: Transport Informa tion

UN number:   Not regulated as a dangerous good.

UN proper shipping name:   Not regulated as a dangerous good.

Transport hazard class(es):   Not regulated as a dangerous good.

Packing group:   Not regulated as a dangerous good.

Environmental hazards:   Not regulated as a dangerous good.

Special precautions for user:   Not dangerous goods in the meaning of ADR/RID, ADN, IMDG-Code, ICAO/IATA-DGR.

Transport in bulk according to Annex II of Marpol and the IBC Code:   Not applicable.

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Page 7

Safety Data Sheet for Titanium Dioxide,

Enriched Titanium Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Titanium Dioxide, Enriched Titanium   https://www.buyisotope.com   Page 7/9

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations:   The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

SARA 302 Components:   No chemicals in this material are subject to the reporting requirements of SARA Title III,

Section 302.

SARA 313 Components : This material does not contain any chemical components with known CAS numbers that

exceed the threshold (de minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 Hazards:   Chronic Health Hazard.

Massachusetts Right to Know Components:   Titanium dioxide, nanoparticles range in size from 1 to 150 nm CAS No.

13463-67-7 / Revision Date 1994-04-01

Pennsylvania Right to Know Components:   Titanium dioxide, nanoparticles range in size from 1 to 150 nm CAS No.

13463-67-7 / Revision Date 1994-04-01

New Jersey Right to Know Components:   Titanium dioxide, nanoparticles range in size from 1 to 150 nm CAS No.

13463-67-7 / Revision Date 1994-04-01

California Prop. 65 Components:   WARNING! This product contains a chemical known to the State of California to

cause cancer. Titanium dioxide, nanoparticles range in size from 1 to 150 nm.

National regulations:   Follow national regulation for work with chemical agents.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

Page 8

Safety Data Sheet for Titanium Dioxide,

Enriched Titanium Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Titanium Dioxide, Enriched Titanium   https://www.buyisotope.com   Page 8/9

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

Page 9

Safety Data Sheet for Titanium Dioxide,

Enriched Titanium Dioxide

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 24-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Titanium Dioxide, Enriched Titanium   https://www.buyisotope.com   Page 9/9

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

None.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.