Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III) GD8151 SDS

Publisher: Ereztech

Source: https://resds.com/document/tris-2-2-6-6-tetramethyl-3-5-heptanedionato-gadolinium-iii-sds-en-f136f21faf

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Ereztech GD8151  Page 1 of 12   Revision: 1.10

Date of Issue:  4/18/2020

EREZTECH LLC

11555 Medlock Bridge Road, Suite 100, Johns Creek, GA 30097, USA

T: +1.888.658.1221 F: 1.678.619.2020

E: info@ereztech.com  W: https://ereztech.com

Product Name: Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Product Type: Solid

CAS Number: 14768-15-1

Product Number: GD8151

Product Manufacturer: Ereztech LLC

11555 Medlock Bridge Road, Suite 100

Johns Creek, GA  30097

Product Information: (888) 658-1221

In Case of an Emergency: CHEMTREC: 1-800-424-9300 (USA);

+1 703-527-3887 (International); CCN836180

*** Contact manufacturer for all non-emergency calls.

Appearance/Odor: Off-white crystals, odor not determined.

Classification: SKIN CORROSION/IRRITATION - Category 2, H315

SERIOUS EYE DAMAGE/IRRITATION - Category 2A, H319

SPECIFIC ORGAN TOXICITY, SINGLE EXPOSURE; RESPIRATORY

TRACT IRRITATION - Category 3, H335

GHS Label Elements

Hazard Pictograms:

Signal Word: WARNING

Hazard Statements: H315:  Causes skin irritation.

H319:  Causes serious eye irritation.

H335:  May cause respiratory irritation.

Precautionary Statements

Prevention: P261:  Avoid breathing fumes/gas/mist/vapors/spray.

P264:  Wash exposed skin thoroughly after handling.

P271:  Use only outdoors or in a well ventilated area.

Section 2.  Hazards Identification

Section 1.  Identification

Page 2

Ereztech GD8151  Page 2 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

Prevention (cont.): P280:  Wear protective gloves/ protective clothing/ eye protection/

face protection.

Response: P302 + P352:  IF ON SKIN: Wash with plenty of soap and water.

P304 + P340:  IF INHALED: Remove victim to fresh air and keep at

rest in a position comfortable for breathing.

P305 + P351 + P338:  IF IN EYES: Rinse cautiously with water for

several minutes.  Remove contact lenses, if present and easy to do.

Continue rinsing.

P312:  Call a POISON CENTER or doctor/physician if you feel

unwell.

P332 + P313:  If skin irritation occurs: Get medical

advice/attention.

P337 + P313:  If eye irritation persists: Get medical

advice/attention.

P362:  Take off contaminated clothing and wash before reuse.

Storage: P403 + P233:  Store in a well ventilated place.  Keep container

tightly closed.

P405:  Store locked up.

Disposal: P501:  Dispose of contents/ container to an approved wasted

disposal plant.

General: None.

OSHA/HCS Status: This material is considered hazardous by the OSHA Hazard

Communication Standard (29 CFR 1910.1200).

Hazards Not Otherwise None identified.

Classified (HNOC):

Substances

Formula : C33H57GdO6

Molecular Weight : 707.05

CAS-No.  : 14768-15-1

EC-No.  : 238-834-4

Ingredient Name % CAS Number

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)  /g149 98 14768-15-1

There are no additional ingredients present which, within the current knowledge of the supplier

and in the concentrations applicable, are classified as hazardous to health or the environment

and hence require reporting in this section.

Occupational exposure limits, if available, are listed in Section 8.

Section 3.  Composition/Information on Ingredients

Section 2.  Hazards Identification

Page 3

Ereztech GD8151  Page 3 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

Description of Necessary First Aid Measures

General Advice: Move out of dangerous area.  Call a POISON CENTER or doctor/physician

immediately if symptoms develop or if you feel unwell.  Show this safety data

sheet to the doctor in attendance.

Eye Contact:  Immediately flush eyes with plenty of water, occasionally lifting the upper and

lower eyelids.  Check for and remove any contact lenses.  Continue rinsing.  Seek

medical attention if eye irritation develops and persists.

Skin Contact:  Take off contaminated clothing and shoes immediately.  Wash off contaminated

skin with soap and plenty of water.  Seek medical attention if irritation develops

and persists, if symptoms develop or if you feel unwell.

Inhalation:  Remove victim to fresh air and keep at rest in a position comfortable for

breathing.  If not breathing, if breathing is irregular or if respiratory arrest

occurs, provide artificial respiration or oxygen by trained personnel.  It may be

dangerous to the person providing aid to give mouth-to-mouth resuscitation.  If

unconscious, place in recovery position and get medical attention immediately.

Maintain an open airway. Loosen tight clothing such as a collar, tie, belt or

waistband.  Call a physician or POISON CONTROL CENTER if symptoms develop

or if you feel unwell.

Ingestion:  Rinse mouth.  Do NOT induce vomiting.  Remove dentures if any. If vomiting

occurs, the head should be kept low so that vomit does not enter the lungs.

Never give anything by mouth to an unconscious person. If unconscious, place in

recovery position and get medical attention immediately.  Maintain an open

airway.  Loosen tight clothing such as a collar, tie, belt or waistband.  Call a

physician or POISON CONTROL CENTER if symptoms develop or if you feel

unwell.

Most Important Symptoms/Effects, Acute and Delayed Potential Acute Health Effects

Eye Contact: Symptoms may include stinging, tearing, redness, swelling and blurred vision.

Inhalation:  Product may be irritating to respiratory system.  Symptoms may include

coughing, sneezing with phlegm production, sore throat, nausea, headache,

vomiting.

Skin Contact:  Symptoms may include an itching or burning sensation, reddening, swelling and

blistering with tissue necrosis.

Ingestion:  Product may be expected to be irritating to mucous membranes.  Symptoms may

include cramping, localized pain, headache, nausea and vomiting.

Indication of Immediate Medical Attention and Special Treatment Needed, If Necessary

Notes to Physician:  Treat symptomatically.

Specific Treatments:  No specific treatment.

Protection of First Responders:  No action taken shall be taken involving any personal risk

without suitable training.  It may be dangerous to the

person providing aid to give mouth-to-mouth resuscitation.

See Toxicological Information (Section 11)

Section 4.  First Aid Measures

Page 4

Ereztech GD8151  Page 4 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

General Hazards:  None identified.

Suitable Extinguishing Media:  Use water spray, alcohol resistant foam, dry chemical or carbon

dioxide (CO2).

Unsuitable Extinguishing Media:  None identified.

Unusual Fire and Explosion None identified.

Hazards:

Product of Combustion:  Decomposition products may include irritating fumes, organic

acid vapors, carbon oxides (COx) and gadolinium oxides.

Protection of Firefighters:  Promptly isolate the scene by removing all persons from the

vicinity of the incident if there is a fire.  No action shall be

taken involving any personal risk or without suitable training.

Fire-fighters should wear appropriate protection equipment and

self-contained breathing apparatus (SCBA) with a full face-

piece operated in a positive pressure mode.

Personal Precautions, Protective Equipment and Emergency Procedures

For Non-Emergency Personnel:  No action shall be taken involving any personal risk or without

suitable training.  Evacuate surrounding areas.  Keep

unnecessary and unprotected personnel from entering.  Do not

touch or walk through spilled material.  Avoid inhalation of

vapors or mist.  Provide adequate ventilation.  Wear respiratory

protection.  Put on appropriate personal protective equipment.

For Emergency Responders:  If specialized clothing is required to deal with the spillage, take

note of any information in Section 8 on suitable and unsuitable

materials. See also the information in "For Non-Emergency

Personnel".

Environmental Precautions:  Do not allow dispersal of spilled material and contact with

waterways, drains and sewers. Inform the relevant authorities

if the product has caused environmental pollution (sewers,

waterways or air).

Methods for Containment

General: Move containers from spill area if safe to do so.  Avoid the

formation and inhalation of dusts and aerosols.

Small Spill: Contain and collect spillage with a dry binding material (e.g.

sand, earth, vermiculite or diatomaceous earth) and place in

dry, sealed container for disposal according to local regulations

(see Section 13). Dispose of via a licensed waste disposal

contractor.

Section 6.  Accidental Release Measures

Section 5.  Fire Fighting Measures

Page 5

Ereztech GD8151  Page 5 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

Large Spill:  Approach release from upwind. Prevent entry into sewers,

water courses, basements or confined areas. Contain and

collect spillage with a dry binding material (e.g. sand, earth,

vermiculite or diatomaceous earth) and place in dry, sealed

container for disposal according to local regulations (see

Section 13). Dispose of via a licensed waste disposal

contractor.  Contaminated absorbent material may pose the

same hazard as the spilled product.

Note: see Section 1 for emergency contact information and

Section 13 for waste disposal.

Precautions:  Product is moisture sensitive; handle under a dry, inert gas.

Nitrogen with less than 5 ppm each of moisture and oxygen is

recommended.  Avoid formation and inhalation of dusts and

aerosols.  Keep container tightly sealed.  Avoid contact with

skin, eyes and clothing.  Do not ingest.  Avoid prolonged

exposure.  Ensure adequate ventilation.

Protective Measures:  Put on appropriate personal protective equipment (see Section

8). Do not ingest. Avoid contact with eyes, skin and clothing.

Avoid breathing dust. Keep in the original container kept tightly

closed when not in use. Empty containers retain product

residue and can be hazardous.  Do not reuse container.

General Occupational Hygiene:  Eating, drinking and smoking should be prohibited in areas

where this material is handled, stored and processed. Workers

should wash hands and face before eating, drinking and

smoking. Remove contaminated clothing and protective

equipment before entering eating areas. See also Section 8 for

additional information on hygiene measures.

Safe Storage Conditions:  Product is moisture sensitive; store under an inert gas.

Nitrogen with less than 5 ppm each of moisture and oxygen is

recommended. Store in original container protected from direct

sunlight in a dry and well-ventilated area, away from

incompatible materials and food and drink.  Keep container

tightly closed and sealed until ready for use.  Store locked up.

Section 6.  Accidental Release Measures

Section 7.  Handling and Storage

Page 6

Ereztech GD8151  Page 6 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

Introductory Remarks: These recommendations provide general guidance for handling

this product. Because work environments and material handling

practices vary, safety procedures should be developed for each

intended application. While developing safe handling

procedures, do not overlook the need to clean equipment and

conduct regular repairs. Waste from these procedures should

be handled in accordance with Section 13.

Occupational Exposure Limits: No exposure limits noted for this material.

Engineering Controls:  Properly operating chemical fume hood designed for hazardous

chemicals and having an average face velocity of at least 100

feet per minute.  Provide an eyewash/shower station.

Environmental Exposure Controls: Emissions from ventilation or work process equipment should

be checked to ensure they comply with the requirements of

environmental protection legislation. In some cases, fume

scrubbers, filters or engineering modifications to the process

equipment will be necessary to reduce emissions to acceptable

levels.

Individual Protection Measures

Hygiene Measures:  Wash hands, forearms and face thoroughly after handling

chemical products, before eating, smoking and using the

lavatory and at the end of the working period.  Remove all

soiled and contaminated clothing immediately.  Do not inhale

dusts produced by product.  Avoid contact with eyes and skin.

Ensure that eyewash stations and safety showers are close to

the workstation/g182s location.

Eye/Face Protection:  Safety eyewear complying with an approved standard should

be used when a risk assessment indicates this is necessary to

avoid exposure to dusts. If contact is possible, the following

protection should be worn, unless the assessment indicates a

higher degree of protection: chemical splash goggles,

faceshield (8-inch minimum).  Refer to 29 CFR 1910.133, ANSI

Z87.1, or European Standard EN166.

Skin Protection

Hand Protection:  Chemical-resistant, impervious gloves complying with an

approved standard should be worn at all times when handling

chemical products if a risk assessment indicates this is

necessary. Considering the parameters specified by the glove

manufacturer, check during use that the gloves are still

retaining their protective properties.

Section 8.  Exposure Controls/Personal Protection

Page 7

Ereztech GD8151  Page 7 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

Hand Protection (cont.):  It should be noted that the time to breakthrough for any glove

material may be different for different glove manufacturers. In

the case of mixtures, consisting of several substances, the

protection time of the gloves cannot be accurately estimated. If

contact is possible, the following protection should be worn,

unless the assessment indicates a higher degree of protection:

Chemical-resistant gloves.

Gloves must be inspected prior to use. Use proper glove

removal technique (without touching glove's outer surface) to

avoid skin contact with this product. Dispose of contaminated

gloves after use in accordance with applicable laws and good

laboratory practices. Wash and dry hands.  For full contact,

wear Neoprene or nitrile rubber gloves.

Other Skin Protection:  Appropriate footwear and any additional skin protection

measures should be selected based on the task being

performed and the risks involved and should be approved by a

specialist before handling this product.

Respiratory Protection:  Where risk assessment shows air-purifying respirators are

appropriate use a full-face respirator with multipurpose

combination (US) or type ABEK (EN 14387) respirator

cartridges as a backup to engineering controls. If the respirator

is the sole means of protection, use a full-face supplied air

respirator. Use respirators and components tested and

approved under appropriate government standards such as

NIOSH (US) or CEN (EU).

Physical State:  Solid, crystals.

Color: Off-white.

Odor:  Not determined.

Odor Threshold:  No data available.

pH:  No data available.

Melting Point:  182-184 /g131C (360-363 /g131F).

Boiling Point:  No data available.

Flash Point:  No data available.

Auto-ignition temperature:  No data available.

Specific Gravity:  No data available.

Vapor Pressure: No data available.

Water Solubility: Insoluble.  Product hydrolyzes slowly when exposed to water.

Section 9.  Physical and Chemical Properties

Section 8.  Exposure Controls/Personal Protection

Page 8

Ereztech GD8151  Page 8 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

Reactivity:  Product hydrolyzes slowly when exposed to water.

Chemical Stability:  This product is stable when stored under a dry, inert

atmosphere and away from heat. Nitrogen containing less

than 5 ppm each moisture and air is recommended.  This

product is not sensitive to impact.

Conditions to Avoid:  Exposure to water/moisture.

Incompatible Materials: Water, compounds containing active hydrogen (alcohols,

acids) and strong oxidizing agents.

Hazardous Decomposition Products:  Under normal conditions of storage and use, hazardous

decomposition products should not be produced.

Hazardous decomposition products formed under fire

conditions: irritating fumes, organic acid vapors, carbon

oxides (CO

x) and gadolinium oxides.  In the event of a

fire: see section 5.

Possibility of Hazardous Reactions:  Under normal conditions of storage and use, hazardous

reactions will not occur. Hazardous reactions or instability

may occur under certain conditions of storage or use.

Information on Toxicological Effects

Acute Toxicity : No specific data available.

Irritation/Corrosion : Product causes skin irritation and serious eye irritation.

Sensitization : No specific data available.

Germ Cell Mutagenicity : No effects known.

Carcinogenity

IARC : No component of this product present at levels greater

than 0.1% is identified as probable, possible or

confirmed human carcinogen by IARC.

ACGIH : No component of this product present at levels greater

than 0.1% is identified as probable, possible or

confirmed human carcinogen by ACGIH.

NTP : No component of this product present at levels greater

than 0.1% is identified as probable, possible or

confirmed human carcinogen by NTP.

OSHA : No component of this product present at levels greater

than 0.1% is identified as probable, possible or

confirmed human carcinogen by OSHA.

Reproductive Toxicity : No specific data available.

Teratogenicity : No specific data available.

Section 11.  Toxicological Information

Section 10.  Stability and Reactivity

Page 9

Ereztech GD8151  Page 9 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

Specific Target Organ Toxicity : Respiratory tract irritation.

(Single Exposure)

Specific Target Organ Toxicity : No specific data available.

(Repeated Exposure)

Aspiration Hazard : No specific data available.

Information on the Likely : Common routes of exposure: inhalation (failure to

Routes of Exposure prevent dust formation), dermal (failure to use skin

protection), eye (failure to use safety eyewear). Less

common: ingestion (failure to employ recommended

hygiene measures (e.g. smoking after handling product

without washing hands or using hand protection).

Additional Information : To the best of our knowledge, the chemical, physical

and toxicological properties of this product have not

been thoroughly investigated.

Numerical Measures of Toxicity

Toxicity to Fish : No specific data available.

Toxicity to Daphnia and Other : No specific data available.

Aquatic Invertebrates

Toxicity to Algae : No specific data available.

Persistence and Degradability

Biodegradability : No specific data available.

Bioaccumulative Potential : No specific data available.

Mobility in Soil : No specific data available.

Other Adverse Effects : An environmental hazard cannot be excluded in the

event of unprofessional handling or disposal.

Waste Treatment Methods

Product Dispose of in accordance with local, state, and federal

regulations.  Refer to 40 CFR 260-299 for complete waste

disposal regulations. Consult your local, state, or federal agency

before disposing of any chemicals.

Contaminated Packaging Empty containers retain product residue (dust) and can be

dangerous. Dispose of as unused product.

Section 11.  Toxicological Information

Section 13.  Disposal Considerations

Section 12.  Ecological Information

Page 10

Ereztech GD8151  Page 10 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

DOT IMDG IATA

UN Number  Not regulated  Not regulated  Not regulated

UN Proper Shipping Name  -  -  -

Transport Hazard Classes  -  -  -

Packing Group  -  -  -

Environmental Hazards  -  -  -

Additional Information  -  -  -

Special Precautions for User : Transport within user/g182s premises: always transport in closed

containers that are upright and secure. Ensure that persons

transporting the product know what to do in the event of an

accident or spillage.

Transporting in Bulk According :   Not applicable.

to Annex II of MARPOL 73/78

and the IBC Code

TSCA (Toxic Substance Control Act):

This product is not listed on the U.S. Toxic Substances Control Act Chemical Inventory (TSCA

Inventory).  Use of this product is restricted to research and development only.  This product must

be used under the supervision of a technically qualified individual as defined by the TSCA.  This

product must not be used for commercial purposes or in formulations for commercial purposes.

SARA 302 Components

No chemicals in this material are subject to the reporting requirements of SARA Title III, Section

302.

SARA 313 Components

This material does not contain any chemical components with known CAS numbers that exceed the

threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 Hazards

Acute Health Hazard (Skin corrosion or irritation; Serious eye damage or eye irritation; Specific

Target Organ Toxicity (STOT), single exposure: respiratory irritation).

Massachusetts Right to Know Components

No components are subject to the Massachusetts Right to Know Act.

Pennsylvania Right to Know Components

No components are subject to the Pennsylvania Right to Know Act.

New Jersey Right to Know Components

No components are subject to the New Jersey Right to Know Act.

California Proposition 65 Components

This product does not contain any chemicals known to State of California to cause cancer, birth

defects, or any other reproductive harm.

Section 14.  Transport Information

Section 15.  Regulatory Information

Page 11

Ereztech GD8151  Page 11 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

National Fire Protection Association (U.S.A.)

Reprinted with permission from NFPA 704-2001, Identification of the Hazards of Materials for

Emergency Response Copyright /g1391997, National Fire Protection Association, Quincy, MA 02269.

This reprinted material is not the complete and official position of the National Fire Protection

Association, on the referenced subject which is represented only by the standard in its entirety.

Copyright /g1392001, National Fire Protection Association, Quincy, MA 02269. This warning system is

intended to be interpreted and applied only by properly trained individuals to identify fire, health

and reactivity hazards of chemicals. The user is referred to certain limited number of chemicals

with recommended classifications in NFPA 49 and NFPA 325, which would be used as a guideline

only.  Whether the chemicals are classified by NFPA or not, anyone using the 704 systems to

classify chemicals does so at their own risk.

HMIS Rating

HEALTH 2

FLAMMABILITY 0

PHYSICAL HAZARD 0

History

Date of Printing : 4/18/2020

Date of Issue/Date of Revision : 4/18/2020

Date of Previous Issue : 7/16/18

References : None available

Abbreviations and Acronyms

ACGIH: American Conference of Governmental Industrial Hygienists.

ATE: Acute Toxicity Estimate

CAS: Chemical Abstracts Service (division of the American Chemical Society).

DOT: US Department of Transportation.

GHS: Globally Harmonized System of Classification and Labeling of Chemicals.

HMIS: Hazardous Materials Identification System.

HNOC: Hazards Not Otherwise Classified.

IARC: International Agency for Research on Cancer.

IATA: International Air Transport Association.

Section 16.  Other Information

Page 12

Ereztech GD8151  Page 12 of 12   Revision: 1.10

Date of Issue:  4/18/2020

Tris(2,2,6,6-tetramethyl-3,5-heptanedionato)gadolinium(III)

Safety Data Sheet

Abbreviations and Acronyms (cont.)

IDLH: Immediately Dangerous to Life or Health (US National Institute for Occupation Health and

Safety (NIOSH)).

IMDG: International Maritime Code for Dangerous Goods.

NFPA: National Fire Protection Association.

NIOSH: National Institute of Occupational Safety and Health.

NTP: National Toxicology Program.

OSHA: Occupational Safety and Health Administration.

PEL:  Permissible Exposure Limits.

REL:  Recommended Exposure Limits.

SARA: Superfund Amendments and Reauthorization Act.

STOT: Specific Target Organ Toxicity.

TLV:  Threshold Limit Values (ACGIH).

TWA:  Time Weighted Average.

VOC: Volatile Organic Compound.

Disclaimer

The information herein is believed to be accurate and is presented in good faith; however, no

warranties or representations are made by Ereztech LLC regarding the accuracy or

completeness of the information. Ereztech LLC shall not be liable for any damages resulting

from the handling, or from the contact with the above product.

Final determination of suitability of any material is the sole responsibility of the user. All

materials may present unknown hazards and should be used with caution. Although certain

hazards are described herein, we cannot guarantee that these are the only hazards that exist.

Section 16.  Other Information