Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III) ALFAA44381 SDS

Publisher: Avocado Research Chemicals Ltd. (Part of Thermo Fisher Scientific)

Source: https://resds.com/document/tris-n-n-bis-trimethylsilyl-amide-lanthanum-iii-sds-avocado-research-chemicals-ltd-part-of-thermo-fisher-en-dad0792392

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Page 1 / 8             SAFETY DATA SHEET

Revision Date 31-Jul-2026

Version 6

ALFAA44381

Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)

SECTION 1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

产品说明:产品说明:产品说明:产品说明:                   三[N,N-双(三甲基硅烷)胺]镧(III)三[N,N-双(三甲基硅烷)胺]镧(III)三[N,N-双(三甲基硅烷)胺]镧(III)三[N,N-双(三甲基硅烷)胺]镧(III)

Product Description:                Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)

Cat No. :                        44381

CAS No                           35788-99-9

Molecular Formula              C18 H54 LaN3 Si6

Supplier                         Avocado Research Chemicals Ltd.

(Part of Thermo Fisher Scientific)

Shore Road, Heysham

Lancashire, LA3 2XY,

United Kingdom

Office Tel: +44 (0) 1524 850506

Office Fax: +44 (0) 1524 850608

Emergency Telephone Number     For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

E-mail address                    begel.sdsdesk@thermofisher.com

Recommended Use                 Laboratory chemicals.

Uses advised against           No Information available

SECTION 2. HAZARD IDENTIFICATION

Physical State                      Appearance                          Odor

Solid                    No information available                        Odorless

Emergency Overview

Flammable solid. In contact with water releases flammable gas. Causes severe skin burns and eye damage. Reacts violently with

water. Moisture sensitive.

Classification of the substance or mixture

Flammable solids.                                                                Category 2

Substances/mixtures which, in contact with water, emit flammable gases                 Category 2

Skin Corrosion/Irritation                                                           Category 1 B

Serious Eye Damage/Eye Irritation                                                  Category 1

Label Elements

______________________________________________________________________________________________

Page 1

Page 2

ALFAA44381               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 2 / 8

Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)

______________________________________________________________________________________________

Signal Word                    Danger

Hazard Statements

H228 - Flammable solid

H261 - In contact with water releases flammable gases

H314 - Causes severe skin burns and eye damage

Precautionary Statements

Prevention

P210 - Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking

P231 + P232 - Handle and store contents under inert gas. Protect from moisture

P240 - Ground and bond container and receiving equipment

P264 - Wash face, hands and any exposed skin thoroughly after handling

P271 - Use only outdoors or in a well-ventilated area

P280 - Wear protective gloves/protective clothing/eye protection/face protection

Response

P301 + P330 + P331 - IF SWALLOWED: rinse mouth. Do NOT induce vomiting

P303 + P361 + P353 - IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower

P304 + P340 - IF INHALED: Remove person to fresh air and keep comfortable for breathing

P305 + P351 + P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing

P310 - Immediately call a POISON CENTER or doctor

P370 + P378 - In case of fire: Use dry sand, dry chemical or alcohol-resistant foam to extinguish

P302 + P335 + P334 - IF ON SKIN: Brush off loose particles from skin. Immerse in cool water

P362 + P364 - Take off contaminated clothing and wash it before reuse

Storage

P402 + P404 - Store in a dry place. Store in a closed container

Disposal

P501 - Dispose of contents/ container to an approved waste disposal plant

Physical and Chemical Hazards

Combustible material. In contact with water releases flammable gas. Reacts violently with water.

Health Hazards

Corrosive. Causes skin and eye burns.

Environmental hazards

Contains no substances known to be hazardous to the environment or not degradable in waste water treatment plants. Reacts

violently with water.

This product does not contain any known or suspected endocrine disruptors.

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

Component                              CAS No                Weight %

Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)                          35788-99-9                <=100

SECTION 4. FIRST AID MEASURES

General Advice

Show this safety data sheet to the doctor in attendance. Immediate medical attention is required.

Eye Contact

Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Immediate medical attention is required.

Keep eye wide open while rinsing.

Skin Contact

Wash off immediately with soap and plenty of water while removing all contaminated clothes and shoes. Call a physician

immediately.

Inhalation

Remove to fresh air. If not breathing, give artificial respiration. Call a physician or poison control center immediately. Do not use

______________________________________________________________________________________________

Page 2

Page 3

ALFAA44381               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 3 / 8

Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)

______________________________________________________________________________________________

mouth-to-mouth method if victim ingested or inhaled the substance; give artificial respiration with the aid of a pocket mask

equipped with a one-way valve or other proper respiratory medical device.

Ingestion

Immediate medical attention is required. Do NOT induce vomiting. Drink plenty of water. Never give anything by mouth to an

unconscious person.

Most important symptoms and effects

Causes burns by all exposure routes. Product is a corrosive material. Use of gastric lavage or emesis is contraindicated. Possible

perforation of stomach or esophagus should be investigated: Ingestion causes severe swelling, severe damage to the delicate

tissue and danger of perforation

Self-Protection of the First Aider

Ensure that medical personnel are aware of the material(s) involved, take precautions to protect themselves and prevent spread of

contamination.

Notes to Physician

Treat symptomatically.

SECTION 5. FIRE-FIGHTING MEASURES

Suitable Extinguishing Media

Dry sand. Carbon dioxide (CO 2). Powder. Do not use water or foam. CO 2, dry chemical, dry sand, alcohol-resistant foam.

Extinguishing media which must not be used for safety reasons

Water.

Specific Hazards Arising from the Chemical

The product causes burns of eyes, skin and mucous membranes. Reacts violently with water.

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear. Thermal decomposition can lead to release of irritating gases and vapors.

SECTION 6. ACCIDENTAL RELEASE MEASURES

Personal Precautions

Use personal protective equipment as required. Evacuate personnel to safe areas. Avoid contact with skin, eyes or clothing.

Environmental Precautions

Should not be released into the environment. Do not allow material to contaminate ground water system. See Section 12 for

additional Ecological Information. Do not flush into surface water or sanitary sewer system.

Methods for Containment and Clean Up

Sweep up and shovel into suitable containers for disposal. Avoid dust formation. Do not expose spill to water.

Refer to protective measures listed in Sections 8 and 13.

SECTION 7. HANDLING AND STORAGE

Handling

Wear personal protective equipment/face protection. Do not get in eyes, on skin, or on clothing. Use only under a chemical fume

hood. Do not breathe dust. Do not ingest. If swallowed then seek immediate medical assistance. Do not allow contact with water.

Storage

Corrosives area. Keep away from water or moist air. Keep containers tightly closed in a dry, cool and well-ventilated place.

______________________________________________________________________________________________

Page 3

Page 4

ALFAA44381               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 4 / 8

Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)

______________________________________________________________________________________________

Specific Use(s)

Use in laboratories

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Control Parameters

Monitoring methods

BS EN 14042:2003 Title Identifier: Workplace atmospheres. Guide for the application and use of procedures for the assessment of

exposure to chemical and biological agents. MDHS14/3 General methods for sampling and gravimetric analysis of respirable and

inhalable dust MDHS 91 Metals and metalloids in workplace air by X-ray fluorescence spectrometry MDHS 99 Metals in air by

ICP-AES

Exposure Controls

Engineering Measures

Ensure that eyewash stations and safety showers are close to the workstation location. Wherever possible, engineering control

measures such as the isolation or enclosure of the process, the introduction of process or equipment changes to minimise release

or contact, and the use of properly designed ventilation systems, should be adopted to control hazardous materials at source.

Personal protective equipment

Eye Protection                Goggles (European standard - EN 166)

Hand Protection                  Protective gloves

Glove material     Breakthrough time  Glove thickness   EU standard             Glove comments

Nitrile rubber      See manufacturers               -          EN 374             (minimum requirement)

Neoprene        recommendations

Natural rubber

PVC

Inspect gloves before use.

Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves.

(Refer to manufacturer/supplier for information)

Ensure gloves are suitable for the task: Chemical compatability, Dexterity, Operational conditions, User susceptibility, e.g.

sensitisation effects, also take into consideration the specific local conditions under which the product is used, such as the danger

of cuts, abrasion.

Remove gloves with care avoiding skin contamination.

Skin and body protection       Long sleeved clothing

Respiratory Protection       When workers are facing concentrations above the exposure limit they must use

appropriate certified respirators.

To protect the wearer, respiratory protective equipment must be the correct fit and be used

and maintained properly

Large scale/emergency use    Use a NIOSH/MSHA or European Standard EN 136 approved respirator if exposure limits

are exceeded or if irritation or other symptoms are experienced

Recommended Filter type: Particulates filter conforming to EN 143

Small scale/Laboratory use     Use a NIOSH/MSHA or European Standard EN 149:2001 approved respirator if exposure

limits are exceeded or if irritation or other symptoms are experienced.

Recommended half mask:- Particle filtering: EN149:2001

When RPE is used a face piece Fit Test should be conducted

Hygiene Measures                Handle in accordance with good industrial hygiene and safety practice.

Environmental exposure controls  No information available.

SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

______________________________________________________________________________________________

Page 4

Page 5

ALFAA44381               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 5 / 8

Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)

______________________________________________________________________________________________

Appearance

Physical State                        Solid

Odor                              Odorless

Odor Threshold               No data available

pH                         No information available

Melting Point/Range               149  - 152 °C  / 300.2  - 305.6 °F

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                      2 °C  / 35.6 °F                 Method - No information available

Evaporation Rate                  Not applicable                         Solid

Flammability (solid,gas)         No information available

Explosion Limits               No data available

Vapor Pressure                No data available

Vapor Density                     Not applicable                         Solid

Specific Gravity  / Density        No data available

Bulk Density                  No data available

Water Solubility               No information available

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Autoignition Temperature        No data available

Decomposition Temperature      No data available

Viscosity                         Not applicable                         Solid

Explosive Properties            No information available

Oxidizing Properties            No information available

Molecular Formula              C18 H54 LaN3 Si6

Molecular Weight                  620.06

SECTION 10. STABILITY AND REACTIVITY

Stability                            Moisture sensitive.

Hazardous Reactions            None under normal processing. Reacts violently with water.

Hazardous Polymerization        No information available.

Conditions to Avoid               Exposure to moist air or water. Exposure to moisture.

Materials to avoid              No information available.

Hazardous Decomposition Products Trimethylsilane. Lanthanum oxide. Carbon monoxide (CO). Carbon dioxide (CO2). Nitrogen

oxides (NOx). Silicon dioxide.

SECTION 11. TOXICOLOGICAL INFORMATION

Product Information

(a) acute toxicity;

(b) skin corrosion/irritation;        Category 1 B

(c) serious eye damage/irritation;   Category 1

(d) respiratory or skin sensitization;

______________________________________________________________________________________________

Page 5

Page 6

ALFAA44381               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 6 / 8

Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)

______________________________________________________________________________________________

Respiratory                No data available

Skin                     No data available

(e) germ cell mutagenicity;       No data available

(f) carcinogenicity;             No data available

There are no known carcinogenic chemicals in this product

(g) reproductive toxicity;         No data available

(h) STOT-single exposure;        No data available

(i) STOT-repeated exposure;      No data available

Target Organs              No information available.

(j) aspiration hazard;               Not applicable

Solid

Symptoms  / effects,both acute and Product is a corrosive material. Use of gastric lavage or emesis is contraindicated.

delayed                             Possible perforation of stomach or esophagus should be investigated: Ingestion causes

severe swelling, severe damage to the delicate tissue and danger of perforation

SECTION 12. ECOLOGICAL INFORMATION

Ecotoxicity effects              May cause long-term adverse effects in the environment. Do not allow material to

contaminate ground water system.

Persistence and Degradability      Product contains heavy metals. Discharge into the environment must be avoided. Special

pre-treatment is necessary

Persistence                May persist.

Degradation in sewage          Contains substances known to be hazardous to the environment or not degradable in waste

treatment plant                 water treatment plants.

Bioaccumulative Potential          Product has a high potential to bioconcentrate

Mobility in soil                No information available

Endocrine Disruptor Information    This product does not contain any known or suspected endocrine disruptors

Persistent Organic Pollutant        This product does not contain any known or suspected substance

Ozone Depletion Potential           This product does not contain any known or suspected substance

SECTION 13. DISPOSAL CONSIDERATIONS

Waste from Residues/Unused      Waste is classified as hazardous. Dispose of in accordance with the European Directives

Products                        on waste and hazardous waste. Dispose of in accordance with local regulations.

Contaminated Packaging           Dispose of this container to hazardous or special waste collection point. Empty containers

retain product residue, (liquid and/or vapor), and can be dangerous. Keep product and

empty container away from heat and sources of ignition.

______________________________________________________________________________________________

Page 6

Page 7

ALFAA44381               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 7 / 8

Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)

______________________________________________________________________________________________

Other Information                Waste codes should be assigned by the user based on the application for which the product

was used. Do not flush to sewer. Can be landfilled or incinerated, when in compliance with

local regulations. Do not empty into drains. Large amounts will affect pH and harm aquatic

organisms.

SECTION 14. TRANSPORT INFORMATION

Road and Rail Transport

UN-No                       UN3396

Proper Shipping Name          ORGANOMETALLIC SUBSTANCE, SOLID, WATER- REACTIVE, FLAMMABLE

Technical Shipping Name        (Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III))

Hazard Class                        4.3

Subsidiary Hazard Class         4.1

Packing Group                                     II

IMDG/IMO

UN-No                       UN3396

Proper Shipping Name          ORGANOMETALLIC SUBSTANCE, SOLID, WATER-REACTIVE, FLAMMABLE

Technical Shipping Name        (Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III))

Hazard Class                        4.3

Subsidiary Hazard Class         4.1

Packing Group                                     II

IATA

UN-No                       UN3396

Proper Shipping Name          ORGANOMETALLIC SUBSTANCE, SOLID, WATER-REACTIVE, FLAMMABLE

Technical Shipping Name        (Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III))

Hazard Class                        4.3

Subsidiary Hazard Class         4.1

Packing Group                                     II

Special Precautions for User      No special precautions required

SECTION 15. REGULATORY INFORMATION

Measures on the Environmental Administration of New Chemical Substances Registration

Registration/Notification number      B1A222252545

Downstream users need to comply with the conditions of safe use of the chemical, understand the environmental and health

hazard and risk management measures identified on the SDS as well as the local/national regulations concerning the chemical.

International Inventories

X = listed, China (IECSC), Europe (EINECS/ELINCS/NLP), U.S.A. (TSCA), Canada (DSL/NDSL), Philippines (PICCS), Japan (ENCS), Japan

(ISHL), Australia (AICS), Korea (KECL).

National Regulations

SECTION 16. OTHER INFORMATION

Prepared By                         Health, Safety and Environmental Department

______________________________________________________________________________________________

Page 7

Page 8

ALFAA44381               SAFETY DATA SHEET                     Revision Date 31-Jul-2026Page 8 / 8

Tris[N,N-bis(trimethylsilyl)amide]lanthanum(III)

______________________________________________________________________________________________

Revision Date                      31-Jul-2026

Revision Summary                Not applicable.

Training Advice

Chemical hazard awareness training, incorporating labelling, Safety Data Sheets (SDS), Personal Protective Equipment (PPE) and

hygiene.

Use of personal protective equipment, covering appropriate selection, compatibility, breakthrough thresholds, care, maintenance, fit

and standards.

First aid for chemical exposure, including the use of eye wash and safety showers.

Legend

CAS - Chemical Abstracts Service                          TSCA - United States Toxic Substances Control Act Section 8(b)

Inventory

EINECS/ELINCS - European Inventory of Existing Commercial Chemical DSL/NDSL - Canadian Domestic Substances List/Non-Domestic

Substances/EU List of Notified Chemical Substances                  Substances List

PICCS - Philippines Inventory of Chemicals and Chemical Substances  ENCS - Japanese Existing and New Chemical Substances

IECSC - Chinese Inventory of Existing Chemical Substances          AICS - Australian Inventory of Chemical Substances

KECL - Korean Existing and Evaluated Chemical Substances         NZIoC - New Zealand Inventory of Chemicals

WEL - Workplace Exposure Limit                        TWA - Time Weighted Average

ACGIH - American Conference of Governmental Industrial Hygienists   IARC - International Agency for Research on Cancer

DNEL - Derived No Effect Level                           PNEC - Predicted No Effect Concentration

RPE - Respiratory Protective Equipment                        LD50 - Lethal Dose 50%

LC50 - Lethal Concentration 50%                            EC50 - Effective Concentration 50%

NOEC - No Observed Effect Concentration                 POW - Partition coefficient Octanol:Water

PBT - Persistent, Bioaccumulative, Toxic                       vPvB - very Persistent, very Bioaccumulative

ICAO/IATA - International Civil Aviation Organization/International Air   IMO/IMDG - International Maritime Organization/International Maritime

Transport Association                                          Dangerous Goods Code

ADR - European Agreement Concerning the International Carriage of   MARPOL - International Convention for the Prevention of Pollution from

Dangerous Goods by Road                                         Ships

OECD - Organisation for Economic Co-operation and Development    ATE - Acute Toxicity Estimate

BCF - Bioconcentration factor                           VOC - (Volatile Organic Compound)

Key literature references and sources for data

https://echa.europa.eu/information-on-chemicals

Suppliers safety data sheet, Chemadvisor - LOLI, Merck index, RTECS

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

End of Safety Data Sheet

______________________________________________________________________________________________

Page 8