Ytterbium-173 SDS

Publisher: Neonest AB

Source: https://resds.com/document/ytterbium-173-sds-yb-173-26bd6f8583

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

Section 1: Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name: Ytterbium Metal.

Molecular weight: 173.04

Chemical formula: Yb

Synonyms: None.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112 – begär Giftinformation.

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to: Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:                             Hazard statement:

Flammable Solid (Category 1)                                  H228

Acute Toxicity, Oral (Category 4)                             H302

Acute Toxicity, Dermal (Category 4)                           H312

Skin Irritant (Category 2)                                    H315

Eye Irritant (Category 2)                                     H319

Acute Toxicity, Inhalation (Category 4)                       H332

STOT SE (Category 3)                                          H335

Label elements:

Hazard pictograms:

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium    https://www.buyisotope.com              Page 1/10

Page 2

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

Signal Words: Danger.

Hazard Statements:

H228 Flammable solid.

H302 Harmful if swallowed.

H312 Harmful in contact with skin.

H315 Causes skin irritation.

H319 Causes serious eye irritation.

H332 Harmful if inhaled.

H335 May cause respiratory irritation.

Precautionary Statements:

P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking.

P233 Keep container tightly closed.

P264 Wash hands thoroughly after handling.

Use only outdoors or in a well-ventilated area.

P280 Wear protective gloves/eye protection.

P302 + P352 IF ON SKIN: Wash with plenty water.

P304 + P340 IF INHALED: Remove person to fresh air and keep comfortable for breathing.

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and

easy to do. Continue rinsing.

P312 Call a POISON CENTER/doctor if you feel unwell.

P332 + P313 If skin irritation occurs: Get medical advice/attention.

P337 + P313 If eye irritation persists: Get medical advice/attention.

P403 + P233 Store in a well-ventilated place. Keep container tightly closed.

P405 Store locked up.

Other hazards: None known.

Section 3: Composition/Information on Ingredients

Substance/Mixture: Substance.

Ingredients:

CAS-No.               Molecular      Concentration               Classification

Substance name (IUPAC/EC)

EC-No.                weight         % by weight                EC1272/2008

Flam. Sol. 1       H228

7440-64-4                                                    Acute Tox. 4       H302

Acute Tox. 4       H312

Ytterbium                                                         173.04              >99%             Skin Irrit. 2      H315

Eye Irrit. 2       H319

231-173-2                                                    Acute Tox. 4       H332

STOT SE 3          H335

For explanation of abbreviations see Section 16.

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium      https://www.buyisotope.com                        Page 2/10

Page 3

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

Section 4: First-Aid Measures

Description of first aid measures:

In case of inhalation: Remove to fresh air, keep warm and quiet, give oxygen if breathing is difficult. Seek medical

attention.

In case of skin contact: Remove contaminated clothing, brush material off skin, wash affected area with soap and

water. Seek medical attention if symptoms persist.

In case of eye contact: Flush eyes with lukewarm water, including under upper and lower eyelids, for at least 15

minutes. Seek medical attention if symptoms persist.

In case of ingestion: Rinse mouth with water. Do not induce vomiting. Seek medical attention. Never induce vomiting

or give anything by mouth to an unconscious person.

Most important symptoms and effects, both acute and delayed:

Inhalation: Inhalation of dusts may be damaging to the health of the individual. Material may cause respiratory

irritation in some persons. Persons with impaired respiratory function, airway diseases and conditions such as

emphysema or chronic bronchitis may incur further disability if excessive concentrations of particulate are inhaled.

Eyes: Material can cause eye irritation and damage in some persons. Contact the eye by metal dusts may cause

mechanical abrasion or foreign body penetration of the eyeball.

Skin contact: Not thought to produce adverse health effects following contact; however, open cuts, abraded or

irritated skin should not be exposed to this material. Entry into the bloodstream through cuts, abrasions or lesions

may produce systemic injury with harmful effects.

Ingestion: Not normally a hazard due to the physical form of the product. However, accidental ingestion of the

material may be damaging to the health of the individual. Lanthanide poisoning causes immediate defecation,

writhing, incoordination, laboured breathing, and inactivity. Respiratory and heart failure may follow, causing death.

Indication of any immediate medical attention and special treatment needed: Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Section 5: Fire-Fighting Measures

Extinguisher media:

Suitable extinguisher media: Use Class D dry powder extinguishing agent.

Unsuitable extinguishing media: Water spray.

Special hazards arising from the mixture: Flammable in the form of dust when exposed to heat, spark or flame. May

react with water under fire conditions liberating flammable hydrogen gas. May emit fumes of ytterbium oxide under

fire conditions.

Advice for fire-fighters: Full face, self-contained breathing apparatus and full protective clothing to prevent contact

with skin and eyes.

Further information: None.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions: Wear appropriate respiratory and protective equipment specified in section 8. Isolate spill area

and provide ventilation. Avoid breathing dust or fume. Avoid contact with skin and eyes. If conditions are dusty,

eliminate all sources of ignition.

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium   https://www.buyisotope.com                 Page 3/10

Page 4

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

Environmental precautions: Prevent product from entering drains. Prevent further leakage or spillage if safe to do so.

Local authorities should be advised if significant spillages cannot be contained.

Methods for containment and cleaning up:

Methods for cleaning up: Avoid dust formation. Sweep or scoop spilled product and place in a closed container for

further handling and disposal. Use only non-sparking tools and natural bristle brushes.

Reference to other sections:

Treat recovered material as described in the section "Disposal considerations".

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling: Handle in an enclosed, controlled process, under dry protective gas such as argon when

possible. Air- and moisture-sensitive. Use non-sparking tools. Protect from sources of ignition. Protect from

water/moisture. Avoid contact with skin and eyes. Wash thoroughly before eating or smoking. See Section 8 for

information on personal protection equipment.

Hygiene measures: Do not eat, drink or smoke when using this product.

Conditions for safe storage, including incompatibilities:

Requirements for storage areas and containers: Ytterbium metal should be stored in tightly-closed containers under

argon or mineral oil. Store in a cool, dry area. Protect from water/moisture. See section 10 for more information on

incompatible materials.

Section 8: Exposure Controls/Personal Protection

Control parameters:

Occupational exposure limits: Contains no substances with occupational exposure limit values.

Exposure controls:

Appropriate engineering controls: Handle in a humidity-controlled atmosphere. Handle in an enclosed, controlled

process when possible. Ensure adequate ventilation to maintain exposures below occupational limits. Whenever

possible, the use of local exhaust ventilation or other engineering controls is the preferred method of controlling

exposure to airborne dust and fume to meet established occupational exposure limits. Use good housekeeping and

sanitation practices. Do not use tobacco or food in work area. Wash thoroughly before eating or smoking. Do not blow

dust off clothing or skin with compressed air.

Individual protection measures, such as personal protective equipment:

Eye/face protection: Chemical safety goggles.

Hand protection: Impermeable gloves.

Body protection: Protective work clothing as necessary.

Respiratory protection: If permissible levels are exceeded, use NIOSH-approved respirator.

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium   https://www.buyisotope.com                 Page 4/10

Page 5

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form): Solid (powder).

Colour: Silver-grey.

Odour: No data available.

Odour threshold: No data available.

Molecular Weight: 173.04

pH (concentration): No data available.

Melting point/range (°C): 819 °C

Boiling point/range (°C): 1196 °C

Freezing point (°C): No data available.

Flash point (°C): No data available.

Evaporation rate: No data available.

Flammability (solid, gas): The substance is a flammable solid.

Ignition temperature (°C): No data available.

Upper/lower flammability/explosive limits: No data available.

Vapour pressure (20 °C): Negligible.

Vapour density: No data available.

Relative density (25 °C): 6.966 g/cm3

Water solubility (g/L) at 20 °C: Insoluble.

n-Octanol/Water partition coefficient: No data available.

Auto-ignition temperature: No data available.

Decomposition temperature: No data available.

Viscosity, dynamic (mPa s): No data available.

Explosive properties: The substance or mixture is not classified as explosive.

Oxidising properties: The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Reactivity: No dangerous reaction known under conditions of normal use.

Chemical stability: Stable under normal conditions.

Possibility of hazardous reactions: None known.

Conditions to avoid: Avoid creating or accumulating fines or dusts, especially near possible ignition sources such as

sparks or flame.

Incompatible materials: Acids, acid chlorides, oxidizing agents, halogens, water/moisture, air.

Hazardous decomposition products: Ytterbium oxides, ytterbium hydroxides, hydrogen.

Section 11: Toxicological Information

Information on toxicological effects:

Acute Toxicity:

Harmful if swallowed, in contact with skin, and inhaled.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Causes skin irritation.

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium   https://www.buyisotope.com                Page 5/10

Page 6

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

Serious eye damage/eye irritation:

Causes serious eye irritation. Material can cause eye irritation and damage in some persons. Contact the eye by metal

dusts may cause mechanical abrasion or foreign body penetration of the eyeball.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

IARC: Not identified as carcinogenic.

NTP: Not identified as carcinogenic.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity – single exposure (STOT):

May cause respiratory irritation. Inhalation of dusts may be damaging to the health of the individual. Material may

cause respiratory irritation in some persons. Persons with impaired respiratory function, airway diseases and

conditions such as emphysema or chronic bronchitis may incur further disability if excessive concentrations of

particulate are inhaled.

Specific target organ toxicity (STOT) – repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:

Atmospheric Fate: Metal-containing inorganic substances generally have negligible vapour pressure and are not

expected to partition to air.

Environmental Fate: Environmental processes, such as oxidation, the presence of acids or bases and microbiological

process, may transform insoluble materials to more soluble ionic forma. Environmental processes may enhance

bioavailability and may also be important in changing solubilities.

Aquatic/Terrestrial Fate: When released to dry soil, most metals will exhibit limited mobility and remain in the upper

layer; some will leach locally into ground water and/or surface water ecosystems when soaked by rain or melt ice. A

metal ion is considered infinitely persistent because it cannot degrade further.

Persistence and degradability: No data available.

Bioaccumulative potential: No data available.

Mobility in soil: No data available.

Results of PBT& vPvB assessment: Not relevant.

Other adverse effects: Do not allow material to be released to the environment.

Section 13: Disposal Considerations

Waste treatment methods: Dispose of in accordance with local and national regulations. In some areas, certain

wastes must be tracked.

Section 14: Transport Information

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium   https://www.buyisotope.com                Page 6/10

Page 7

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

DOT:

Proper Shipping Name: METAL POWDER, FLAMMABLE, N.O.S.

Hazard Class: 4.1

UN Number: 3089

Packing Group: II

Hazard Label:

IMDG:

Proper Shipping Name: METAL POWDER, FLAMMABLE, N.O.S.

Hazard Class: 4.1

UN Number: 3089

Packing Group: II

Marine Pollutant: No

IATA:

Proper Shipping Name: METAL POWDER, FLAMMABLE, N.O.S.

Hazard Class: 4.1

UN Number: 3089

Packing Group: II

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium   https://www.buyisotope.com              Page 7/10

Page 8

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations: The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

TSCA Listed: Ytterbium (7440-64-4) is listed in the US Toxic Substances Control Act (TSCA) Chemical Substance

Inventory.

Regulation (EC) No 1272/2008 (CLP): N/A

Canada WHMIS (CPR, SOR/88-66): N/A

HMIS Ratings: Health: 1 Flammability: 1 Physical: 1

NFPA Ratings: Health: 1 Flammability: 1 Instability: 1

National regulations: Follow national regulation for work with chemical agents.

Chemical safety assessment: No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium   https://www.buyisotope.com                 Page 8/10

Page 9

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium   https://www.buyisotope.com              Page 9/10

Page 10

Safety Data Sheet for Ytterbium Metal

Powder, Enriched Ytterbium Metal Powder

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 31-Aug-2019

Version: 1.1.1.

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H228 Flammable solid.

H302 Harmful if swallowed.

H312 Harmful in contact with skin.

H315 Causes skin irritation.

H319 Causes serious eye irritation.

H332 Harmful if inhaled.

H335 May cause respiratory irritation.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.

Safety Data Sheet (SDS) for Ytterbium Metal, Enriched Ytterbium   https://www.buyisotope.com              Page 10/10