Ytterbium (III) acetate hydrate 3787 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/ytterbium-iii-acetate-hydrate-prochem-en-8906dc7922

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1  PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Ytterbium (III) acetate hydrate

PRODUCT NO:       3787

CAS NO:             15280-58-7

SYNONYMS:           Acetic acid, ytterbium (+3) salt, hydrate

MANUFACTURER:

ProChem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL 61109                                      TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2  HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Not a dangerous substance according to the GHS

HMIS CLASSIFICATION:

Health: 0            Flammability: 0                  Reactivity: 0

NFPA RATING:

Health: 0              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: Causes slight to mild irritation of the eyes.

SKIN CONTACT: Causes slight to mild irritation of the skin.

INHALATION:  Irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: May be harmful if swallowed.

SECTION 3   COMPOSITION / INFORMATION ON INGREDIENTS

Formula:  Yb(OOCCH3)3 !XH2O

Molecular Weight: 350.16 (anhy.)

CHEMICAL NAME                CAS#              %               ACGIH (TWA)            OSHA (PEL)

Ytterbium (III) acetate hydrate              15280-58-7                   100               No data                   No data

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under eyelids. A

victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

YTTERBIUM (III) ACETATE HYDRATE                                                               PAGE 1

Page 2

SECTION 4   FIRST AID MEASURES (Cont.)

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if irritation

persists.

INHALATION: Remove the victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty in

breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical attention immediately. Keep the victim calm. Give the victim water (only if conscious).

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not applicable

AUTO IGNITION TEMPERATURE: Not applicable

EXPLOSION LIMITS: Not applicable

EXTINGUISHING MEDIUM: Carbon dioxide, dry powder, or foam.

SPECIAL FIRE FIGHTING PROCEDURES:   If involved in a fire, fire fighters should be equipped with a NIOSH approved positive

pressure self-contained breathing apparatus and full protective clothing.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Carbon oxides and ytterbium oxide.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Avoid dust formation. Avoid breathing dust, vapors, mist, or gas. Ensure adequate ventilation.

ENVIRONMENTAL PRECAUTIONS: Do not let product enter drains or allow to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with vermiculite or sodium

carbonate and swept up.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Provide appropriate exhaust ventilation at places where dust is formed.

CONDITIONS FOR SAFE STORAGE: Keep container tightly closed in a dry and well-ventilated place.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when handling a chemicals

substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

VENTILATION:  If possible, handle the material in an efficient fume hood.

RESPIRATOR:   If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9   PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White powder

MELTING POINT: No data available

BOILING POINT: No data available

MOLECULAR WEIGHT: 350.16 (anhy.)

ODOR: Odorless

SPECIFIC GRAVITY: 2.09

VAPOR PRESSURE: Not applicable

SOLUBILITY IN WATER: Very soluble

SECTION 10   STABILITY AND REACTIVITY

STABILITY: Stable under normal conditions.

HAZARDOUS POLYMERIZATION: No hazardous polymerization

YTTERBIUM (III) ACETATE HYDRATE                                                               PAGE 2

Page 3

SECTION 10   STABILITY AND REACTIVITY (Cont.)

CONDITIONS TO AVOID: Excess heat.

INCOMPATIBILITY: Strong oxidizing agents, acids, and bases.

DECOMPOSITION PRODUCTS: Carbon oxides and ytterbium oxide.

SECTION 11 TOXICOLOGICAL DATA

CARCINOGENIC EFFECTS: No component of this product present at levels greater than or equal to 0.1% is identified as probable,

possible or confirmed human carcinogen.

MUTAGENIC EFFECTS: No data

TETRATOGENIC EFFECTS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA Inventory

SARA 302 COMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA (TITLE 313): This material does not contain any chemical components with known CAS numbers that exceed the threshold (De

Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 HAZARDS: No SARA Hazards

MASSACHUSETTS RIGHT TO KNOW COMPONENETS: No components are subject to the Massachusetts Right to Know Act.

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals known to State of California to cause cancer,

birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE REVISED: 08/25

SDS DEPT.

YTTERBIUM (III) ACETATE HYDRATE                                                               PAGE 3