Ytterbium (III) Fluoride 3802 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/ytterbium-iii-fluoride-prochem-en-962a31ce48

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Ytterbium (III) Fluoride

PRODUCT NUMBER:  3802

CAS NUMBER:        13760-80-0

SYNONYMS:         Ytterbium Trifluoride

MANUFACTURER:

ProChem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL 61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H315              Causes skin irritation.

H319              Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                Avoid breathing dust/fume/gas/mist/vapors/spray.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P302+P352            IF ON SKIN: Wash with plenty of water.

P305+P351+P338     IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and east to

do. Continue rinsing.

P501                Dispose of contents/container to an approved waste disposal plant.

HMIS CLASSIFICATION:

Health: 1              Fire: 0                  Reactivity Hazard: 1

NFPA RATING:

Health: 1            Flammability: 0         Reactivity Hazard: 1

EYE CONTACT: Causes serious eye irritation.

SKIN CONTACT: Causes skin irritation.

INHALATION: May cause irritation to the respiratory tract. Symptoms may include coughing and shortness of breath.

INGESTION: May cause irritation.

YTTERBIUM (III) FLUORIDE                                                               PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: YbF3

Molecular Weight: 230.04

CHEMICAL NAME                                           CAS#                               %

Ytterbium (III) Fluoride                                                   13760-80-0                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under eyelids. A

victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if irritation

persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial respiration.

Get medical attention if necessary.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Ytterbium oxides and Hydrogen Fluoride.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Keep unprotected

persons away. Avoid dust formation. Avoid breathing dusts.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with powdered sodium

bicarbonate, lime, or calcium carbonate and swept up. Do not flush with water. Dispose of accordingly.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

contact with skin and eyes. Avoid formation of dust. Keep container tightly sealed. Handle under a dry atmosphere of air or nitrogen.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area in a tightly closed container.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS

Component                 Exposure Limits              Basis                           Entity

Ytterbium (III) Fluoride           2.5 mg/m3                 TLV                     ACGIH

2.5 mg/m3                 PEL                   OSHA

2.5 mg/m3                REL                     NIOSH

3 mg/g                       BEI-U                    ACGIH

10 mg/g                      BEI-U                    ACGIH

TLV: Threshold Limit Value over 8 hours of work.           REL: Recommended Exposure Limit

PEL: Permissible Exposure Limit                           BEI-U: Biological Exposure Indices in Urine

YTTERBIUM (III) FLUORIDE                                                               PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION (Cont.)

EYE PROTECTION: Wear chemical safety glasses or goggles.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White to off-white solid

ODOR: None

MOLECULAR WEIGHT: 230.04

BOILING POINT: 2200° C

MELTING POINT: 1157° C

SPECIFIC GRAVITY: 8.2

VAPOR DENSITY: No data available

SOLUBILITY IN WATER: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Avoid Moisture.

INCOMPATIBILITY: Oxidizing agents

DECOMPOSITION PRODUCTS: Hydrogen Fluoride and Ytterbium oxides.

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: ZG2487500

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good.

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Not Listed

YTTERBIUM (III) FLUORIDE                                                               PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION (Cont.)

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW: No components are subject to the Massachusetts Right to Know Act.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 10/14

YTTERBIUM (III) FLUORIDE                                                               PAGE 4