Ytterbium (III) Nitrate Pentahydrate 3807 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/ytterbium-iii-nitrate-pentahydrate-prochem-en-d43c99159a

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Ytterbium (III) Nitrate Pentahydrate

PRODUCT NUMBER:  3807

CAS NUMBER:        35725-34-9

MANUFACTURER:

Prochem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                    TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H272             May intensify fire; oxidizer.

H315              Causes skin irritation.

H319              Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces.

P220                 Keep/store away from clothing/combustible material.

P221               Take any precaution to avoid mixing with combustibles.

P261                Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338     IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

P362               Take off contaminated clothing and wash before reuse.

P370+P378             In case of fire: Use dry sand, dry chemical or alcohol-resistant foam for extinction.

P403+P233           Store in a well-ventilated place.

YTTERBIUM (III) NITRATE                                                               PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

P422                 Store contents under inert gas.

HMIS CLASSIFICATION:

Health: 0              Fire: 0                  Reactivity Hazard: 2

NFPA RATING:

Health: 0            Flammability: 0         Reactivity Hazard: 2       Specific Hazard: OX

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: May be irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: May be harmful if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Yb(NO3)3 ! 5H2O

Molecular Weight: 449.13

CHEMICAL NAME                                           CAS#                               %

Ytterbium (III) Nitrate Pentahydrate                                         35725-34-9                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use water spray, alcohol-resistant foam, dry chemical, or carbon dioxide.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Ytterbium oxides and nitrogen oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid dust

formation. Avoid breathing dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with vermiculite or sodium

carbonate and swept up.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

formation of dust. Keep away from heat and sources of ignition. Avoid contact with skin and eyes.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Keep away from heat and direct sunlight.

YTTERBIUM (III) NITRATE                                                                PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: This product contains no substances occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White solid

ODOR: None

MOLECULAR WEIGHT: 359.07

BOILING POINT: Not available

MELTING POINT: Not available

SPECIFIC GRAVITY: Not available

VAPOR DENSITY: No data available

SOLUBILITY: Very soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Contact with strong reducing agents or organic matter. Some nitrate can explode heated to high

temperatures.

INCOMPATIBILITY: Flammable substances, reducing agents, water/moisture, organic material, and metal powders

DECOMPOSITION PRODUCTS: Nitrogen oxides and ytterbium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN1477

Nitrates, inorganic, n.o.s.

(Ytterbium (III) nitrate)

CLASS 5.1

PG II

Marine Pollutant: No

YTTERBIUM (III) NITRATE                                                                PAGE 3

Page 4

.

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Reactivity

SARA (TITLE 313): Not Listed

PENNSYLVANIA RIGHT TO KNOW: Ytterbium nitrate pentahydrate

NEW JERSEY RIGHT TO KNOW: Ytterbium nitrate pentahydrate

CALIFORNIA PROP. 65: Not Listed

WHMIS CANADA: Class C: Oxidizing material. Class D-2B: Toxic material causing other toxic effects.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc. makes

no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the product based

on the information.

DATE PREPARED: 09/14

YTTERBIUM (III) NITRATE                                                                PAGE 4