Yttria-stabilized zirconia powder NIST SRM 1982 SDS

Publisher: National Institute of Standards and Technology (NIST)

Source: https://resds.com/document/yttria-stabilized-zirconia-spheroidal-thermal-spray-powder-nist-srm-1982-en-1116f1e987

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Date of Issue:

16 September 2016

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number: 1982

SRM Name:   Thermal Spray Powder – Particle Size Distribution

Yttria-Stabilized Zirconia (Spheroidal)

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM) is primarily intended for use in the calibration of equipment used to

measure particle size distributions (PSD) in the 10 μm to 150 μm range. A unit of SRM 1982 consists of a single

bottle containing approximately 10 g of yttria stabilized zirconia (YSZ) powder.

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

FAX: 301-948-3730                                  1-800-424-9300 (North America)

E-mail: SRMMSDS@nist.gov                        +1-703-527-3887 (International)

Website: http://www.nist.gov/srm

2. HAZARDS IDENTIFICATION

Note: SRM 1982 is supplied in a small quantity and under normal laboratory conditions it does not constitute a

combustible dust hazard. The physical properties of this material indicate that accumulated dust on surfaces generated

where operations produce fine particulates, may lead to combustible dust concentrations in air.

Classification

Physical Hazard:   Not classified.

Health Hazard:     Not classified.

Label Elements

Symbol

No Symbol/No Pictogram.

Signal Word

No signal word.

Hazard Statement(s): Not applicable.

Precautionary Statement(s): Not applicable.

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Yttria stabilized zirconia (YSZ) powder

Other Designations:   Zirconia (Zirconium oxide; zirconium dioxide; zirconium white)

Yttria (Diyttrium trioxide; yttrium oxide; yttrium sesquioxide; yttrium(3+) oxide)

Hafnium oxide (Hafnia, hafnium dioxide)

Components are listed in compliance with OSHA’s 29 CFR 1910.1200.

SRM 1982                                                                              Page 1 of 6

Page 2

Hazardous Component(s)      CAS Number    EC Number    Nominal Mass Concentration

(EINECS)               (%)

Zirconium oxide                   1314-23-4         215-227-2              >90

Yttrium oxide                     1314-36-9         215-233-5                   7.33

Hafnium oxide                   12055-23-1         235-013-2                   1.39

4. FIRST AID MEASURES

Description of First Aid Measures

Inhalation: If adverse effects occur, remove to uncontaminated area. Give artificial respiration if not breathing.

Get immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes while removing contaminated clothing and

shoes. Get medical attention, if needed. Thoroughly clean and dry contaminated clothing and shoes before reuse.

Eye Contact: Flush eyes with plenty of water for at least 15 minutes. Then get immediate medical attention.

Ingestion:  If a large amount is swallowed, get medical attention.

Most Important Symptoms/Effects, Acute and Delayed: Exposure may irritate the eyes, skin, respiratory system,

and mucous membranes.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek immediate medical attention.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards: Negligible fire hazard. Avoid generating dust; sufficient concentrations of fine dust

dispersed in air, and in the presence of an ignition source is a potential hazard. See Section 9, “Physical and Chemical

Properties” for flammability properties.

Extinguishing Media

Suitable: Regular dry chemical, carbon dioxide, water, regular foam.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: Not applicable.

Special Protective Equipment and Precautions for Fire-Fighters: Move container from fire area if it can be done

without personal risk. Avoid inhalation of material or combustion by-products. Wear full protective clothing and

NIOSH-approved self-contained breathing apparatus (SCBA).

NFPA Ratings (0 = Minimal; 1 = Slight; 2 = Moderate; 3 = Serious; 4 = Severe)

Health = 1            Fire = 0          Reactivity = 0

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures: Any accumulated material on surfaces

should be removed and properly disposed of. Use suitable protective equipment; see Section 8, “Exposure Controls

and Personal Protection”.

Methods and Materials for Containment and Clean up:  Collect spilled material in appropriate container for

disposal. Avoid generating dust. Clean up residue with a high-efficiency particulate filter vacuum.

7. HANDLING AND STORAGE

Safe Handling Precautions:  Minimize dust generation and accumulation on surfaces.  Use suitable personal

protection equipment (PPE). See Section 8, “Exposure Controls and Personal Protection”.

Storage and Incompatible Materials: Store and handle in accordance with all current regulations and standards.

Keep separated from incompatible substances (see Section 10, “Stability and Reactivity”).

SRM 1982                                                                              Page 2 of 6

Page 3

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits

Component(s)          OSHA (PEL)     ACGIH (TLV)         NIOSH (REL)

Zirconium Oxide          TWA: 5 mg/m3    TWA: 50 mg/m3      TWA: 5 mg/m3

as Zr (related to                           STEL: 10 mg/m3       STEL: 10 mg/m3

Zirconium compounds)                                          IDLH: 25 mg/m3

(except Zirconium tetrachloride)

Yttrium Oxide            No occupational    TWA: 1 mg/m3       TWA: 1 mg/m3

as Y (related to Yttrium      exposure limits                         IDLH: 500 mg/m3

compounds)                    established

Hafnium Oxide           No occupational    TWA: 0.5 mg/m3      TWA: 0.5 mg/m3

as Hf (related to            exposure limits                         IDLH: 50 mg/m3

Hafnium compounds)          established

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

Personal Protection Measures:  In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate PPE to

minimize exposure to this material.

Respiratory Protection:    If workplace conditions warrant a respirator, a respiratory protection program that

meets OSHA 29CFR 1910.134 must be followed. Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye Protection: Splash resistant safety goggles and emergency eyewash are recommended.

Skin and Body Protection: Chemical resistant clothing and gloves are recommended.

9. PHYSICAL AND CHEMICAL PROPERTIES

Descriptive Properties                        YSZ powder

Molar Mass (g/mol)                                        varies

Molecular Formula                                     not applicable

Appearance (physical state, color, etc.)                    fine powder

Odor                                                     odorless

Odor threshold                                          not available

pH                                                      not available

Evaporation rate                                        not available

Melting point/freezing point                             not available

Relative Density as Specific Gravity (water = 1)              5.8(a)

Vapor Pressure                                         not available

Vapor Density (air = 1)                                  not available

Viscosity                                                not available

Solubilities                                                insoluble in water

Partition coefficient (n-octanol/water)                   not available

Thermal Stability Properties

Autoignition Temperature                              not available

Thermal Decomposition                                 not available

Initial boiling point and boiling range                   not available

Explosive Limits, LEL (Volume %)                     not available

Explosive Limits, UEL (Volume %)                     not available

Flash Point (Closed Cup)                                not available

Flammability (solid, gas)                                not available

(a) Vendor supplied health and safety information.

SRM 1982                                                                              Page 3 of 6

Page 4

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:   X    Stable                Unstable

Possible Hazardous Reactions: Not applicable.

Conditions to Avoid:  Avoid heat, flames, sparks, and other ignition sources.  Avoid contact with incompatible

materials.

Incompatible Materials: Oxidizing materials.

Hazardous Decomposition: Miscellaneous decomposition products.

Hazardous Polymerization:              Will Occur       X    Will Not Occur

11. TOXICOLOGICAL INFORMATION

Route of Exposure:    X    Inhalation     X    Skin        X    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics: Exposure may irritate the eyes,

skin, respiratory system, and mucous membranes.

Potential Health Effects (Acute, Chronic, and Delayed)

Inhalation:  Inhalation exposure to zirconia in animals resulted in no detectable effects. Acute inhalation of

yttria, a rare earth, has caused sensitivity to heat and itching. Prolonged exposure to rare earth oxides has resulted

in transient chemical pneumonitis, subacute bronchitis and bronchiolitis.  Hafnia may be irritating to mucous

membranes; liver damage may occur.

Skin Contact: Acute and chronic exposure of zirconia to abraded skin may cause allergic granuloma formation.

Application of yttria and hafnia may cause irritation.

Eye Contact: Rare earths may irritate the conjunctiva.

Ingestion: Zirconia and rare earths are poorly absorbed by the gastrointestinal tract. Hafnia ingestion may cause

liver damage.

Numerical Measures of Toxicity

Acute Toxicity: Not classified.

Zirconium oxide: Mouse, Oral LD50: >8.8 g/kg

Yttrium oxide: Rat, Oral LD50: >10 g/kg

Hafnium oxide: No data available.

Skin Corrosion/Irritation: Not classified.

Zirconium oxide: Rabbit, Dermal: not irritating.

Hafnium compounds may cause mild irritation.

Serious Eye Damage/Irritation: Not classified.

Hafnium compounds may cause mild irritation.

Respiratory Sensitization: Not classified; no data available.

Skin Sensitization: Not classified; no data available.

Germ Cell Mutagenicity: Not classified; no data available.

Carcinogenicity: Not classified.

Listed as a Carcinogen/Potential Carcinogen              Yes         X   No

Zirconium oxide, yttrium oxide, and hafnium oxide are not  listed by OSHA, IARC, or NTP as a

carcinogen/potential carcinogen.

Reproductive Toxicity: Not classified; no data available.

Specific Target Organ Toxicity, Single Exposure: Not classified; no data available.

Specific Target Organ Toxicity, Repeated Exposure: Not classified; no data available.

Aspiration Hazard: Not applicable.

SRM 1982                                                                              Page 4 of 6

Page 5

12. ECOLOGICAL INFORMATION

Ecotoxicity Data: No data available.

Persistence and Degradability: No data available.

Bioaccumulative Potential: Zirconium oxide (no bioaccumulation expected); yttrium oxide (BCF 1.3 to 54, low to

moderate).

Mobility in Soil: No data available.

Other Adverse effects: No data available.

13. DISPOSAL CONSIDERATIONS

Waste Disposal: Dispose in accordance with all applicable federal, state, and local regulations.

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: Not regulated by DOT and IATA.

15. REGULATORY INFORMATION

U.S. Regulations

CERCLA Sections 102a/103 (40 CFR 302.4): Not regulated.

SARA Title III Section 302 (40 CFR 355.30): Not regulated.

SARA Title III Section 304 (40 CFR 355.40): Not regulated.

SARA Title III Section 313 (40 CFR 372.65): Not regulated.

OSHA Process Safety (29 CFR 1910.119): Not regulated.

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21)

ACUTE HEALTH:      No

CHRONIC HEALTH:    No

FIRE:               No

REACTIVE:          No

PRESSURE:          No

State Regulations

California Proposition 65: Not listed.

U.S. TSCA Inventory: Zirconium oxide, yttrium oxide, and hafnium oxide are listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations: WHMIS Information is not provided for this material.

16. OTHER INFORMATION

Issue Date: 16 September 2016

Sources:   ChemADVISOR, Inc., SDS Zirconium Oxide, 09 December 2015.

ChemADVISOR, Inc., SDS Yttrium Oxide, 09 December 2015.

ChemADVISOR, Inc., SDS Hafnium Oxide, 09 December 2015.

Metallurgical Technologies, Inc, Vendor MSDS, Metec 1081 Yttria stabilized zirconia 7 % Yttrium

oxide, 01 July 1988.

Hazardous Substances Data Bank (HSDB), National Library of Medicine's TOXNET system, Zirconium

Compounds; Hafnium Compounds; Yttrium Compounds; available at http://toxnet.nlm.nih.gov (accessed

Sep 2016).

ESIS, IUCLID Dataset, Zirconium Dioxide CAS No. 1314-23-4; available at https://echa.europa.eu/

(accessed Sep 2016).

ChemIDplus; National Library of Medicine's TOXNET system, Yttria CAS No.1314-36-9; available at

http://toxnet.nlm.nih.gov (accessed Sep 2016).

SRM 1982                                                                              Page 5 of 6

Page 6

Key of Acronyms:

ACGIH       American Conference of Governmental Industrial        NTP      National Toxicology Program

Hygienists

CAS         Chemical Abstracts Service                     OSHA    Occupational Safety and Health Administration

CERCLA     Comprehensive Environmental Response, Compensation,    PEL      Permissible Exposure Limit

and Liability Act

CFR         Code of Federal Regulations                    RCRA    Resource Conservation and Recovery Act

DOT         Department of Transportation                     REL     Recommended Exposure Limit

EINECS      European Inventory of Existing Commercial Chemical     RQ       Reportable Quantity

Substances

EPCRA      Emergency Planning and Community Right-to-Know Act   RTECS   Registry of Toxic Effects of Chemical Substances

IARC          International Agency for Research on Cancer           SARA    Superfund Amendments and Reauthorization Act

IATA          International Air Transport Association              SCBA    Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health           SRM     Standard Reference Material

LC50          Lethal Concentration                           STOT     Specific Target Organ Toxicity

LD50        Median Lethal Dose or Lethal Dose, 50 %              STEL     Short Term Exposure Limit

LEL         Lower Explosive Limit                         TLV      Threshold Limit Value

MSDS         Material Safety Data Sheet                       TPQ      Threshold Planning Quantity

NFPA         National Fire Protection Association                 TSCA    Toxic Substances Control Act

NIOSH        National Institute for Occupational Safety and Health     TWA    Time Weighted Average

NIST          National Institute of Standards and Technology          UEL     Upper Explosive Limit

n.o.s.         Not Otherwise Specified                      WHMIS  Workplace Hazardous Materials Information System

Disclaimer:  Physical and chemical data contained in  this SDS are provided only for use in assessing the

hazardous nature of the material. The SDS was prepared carefully, using current references; however, NIST does not

certify the data in the SDS.  The values for this material are given in the NIST Certificate of Analysis.

Users of this SRM should ensure that the SDS in their possession is current. This can be accomplished by contacting

the SRM Program: telephone (301) 975-2200; fax (301) 948-3730; e-mail srmmsds@nist.gov; or via the Internet at

http://www.nist.gov/srm.

SRM 1982                                                                              Page 6 of 6