Yttrium (III) Oxalate Nonahydrate 3859 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/yttrium-iii-oxalate-nonahydrate-prochem-en-8c0bf5f92d

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Yttrium (III) Oxalate Nonahydrate

PRODUCT NUMBER:  3859

CAS NUMBER:        13266-82-5

SYNONYMS:         Ethanedioic acid, yttrium salt; oxalic acid, yttrium salt.

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H302+H312          Harmful if swallowed or in contact with skin.

Precautionary Phrases

P280              Wear protective gloves/eye protection/face protection.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P301+P312            IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell.

HMIS CLASSIFICATION:

Health: 3              Fire: 0                  Reactivity Hazard: 1

NFPA RATING:

Health: 1            Flammability: 1         Reactivity Hazard: 0

EYE CONTACT: May cause moderate to severe irritation of the eyes.

SKIN CONTACT: May cause moderate to severe irritation of the skin.

INHALATION: Dust may be severely irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: May be harmful if swallowed.

YTTRIUM (III) OXALATE                                                                  PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Y2(C2O4)3  9H2O

Molecular Weight: 604.01

CHEMICAL NAME                                           CAS#                               %

Yttrium (III) Oxalate                                                      13266-82-5                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Carbon dioxide, extinguishing powder, or water spray. Fight large fires with water spray.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Carbon oxides and yttrium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Keep unprotected

persons away.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with vermiculite or sodium

carbonate and swept up.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Keep away

from heat and moisture.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store in a rightly sealed container.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS

Component                 Exposure Limits              Basis                           Entity

Yttrium                      1 mg/m3                   TLV                     ACGIH

1 mg/m3                  PEL                   OSHA

TLV: Threshold Limit Value over 8 hours of work.            PEL: Permissible Exposure Limit

EYE PROTECTION: Wear chemical safety glasses or goggles.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

YTTRIUM (III) OXALATE                                                                  PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White powder

ODOR: None

MOLECULAR WEIGHT: 604.01

BOILING POINT: Not available

MELTING POINT: Not available (dec.)

SPECIFIC GRAVITY: Not available

SOLUBILITY: Slightly soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Exposure to air or moisture over prolonged periods of time.

INCOMPATIBILITY: Strong oxidizing agents and strong sulfuric acid.

DECOMPOSITION PRODUCTS: Carbon oxides and yttrium oxide

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: may cause long-term adverse effects in the aquatic environment. Harmful in contact with skin and if swallowed.

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Yttrium is similar to the lanthanons and may have an anticoagulant effect on blood. Exposure to lanthanons may

also lead to sensitivity to heat, itching, increased awareness of odor and taste, and liver damage. Oxalates are powerful irritants and

corrosive to tissue. They have a caustic effect on mouth, esophagus, and stomach. Readily absorbed, they can cause severe kidney

damage.

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Not Listed

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

WHMIS CANADA: Not Listed

YTTRIUM (III) OXALATE                                                                  PAGE 3

Page 4

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 09/14

YTTRIUM (III) OXALATE                                                                  PAGE 4