Yttrium (III) oxide 3862 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/yttrium-iii-oxide-sds-prochem-en-ac1f425105

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Yttrium (III) oxide

PRODUCT NUMBER:  3862

CAS NUMBER:        1314-36-9

SYNONYMS:             Yttria, diyttrium trioxide, yttrium sesquioxide

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Not a dangerous substance according to the GHS

HMIS CLASSIFICATION:

Health: 0            Flammability: 0                   Reactivity: 0

NFPA RATING:

Health: 0              Fire: 0                           Reactivity Hazard: 0

EYE CONTACT: May cause irritation of the eyes.

SKIN CONTACT: May cause irritation to the skin.

INHALATION: May cause irritation to the respiratory tract.

INGESTION: May be harmful if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Y2O3

Molecular Weight: 225.82

CHEMICAL NAME                                           CAS#                               %

Yttrium (III) oxide                                                         1314-36-9                                               100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Check for and remove any contact lenses. Immediately flush eyes with running water for at least 15 minutes,

keeping eyelids open. Seek medical attention.

SKIN EXPOSURE: After contact with skin, wash immediately with plenty of water.  If irritation persists, seek medical attention.

INHALATION: Allow the victim to rest in a well-ventilated area. Seek immediate medical attention.

YTTRIUM (III) OXIDE                                                                           PAGE 1

Page 2

SECTION 4   FIRST AID MEASURES (Cont.)

INGESTION: Do not induce vomiting. Loosen tight clothing such as a collar, tie, belt, or waistband.  If the victim is not breathing,

perform mouth-to-mouth resuscitation. Seek immediate medical attention.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable.

AUTO IGNITION TEMPERATURE: Not applicable

EXPLOSION LIMITS: Not applicable

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: If involved in a fire, fire fighters should be equipped with a NIOSH approved

positive self-contained breathing apparatus, flame and chemical resistant clothing, hats, boots, and gloves.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: None

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear approved personal protective equipment. Avoid formation of dust. Ensure adequate ventilation.

ENVIRONMENTAL PRECAUTIONS: Do not let product enter drains or allow to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Pick up and arrange disposal without creating dust. Sweep

up and shovel. Keep in suitable, closed containers for disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Avoid contact with skin and eyes. Avoid formation of dust. Ensure adequate ventilation.

CONDITIONS FOR SAFE STORAGE: Keep container tightly closed and store in a cool, dry, and well-ventilated place.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, or face shield when handling a chemical

substance in the laboratory.

SKIN PROTECTION: Chemical resistant gloves.

VENTILATION: Handle in an efficient chemical fume hood.

RESPIRATOR:  If in form of fine dust and ventilation is not available a respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29 CFR 19-10.134.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: White solid

MOLECULAR WEIGHT: 225.82

BOILING POINT: 4300°C

MELTING POINT: 2410° C (dec.)

SPECIFIC GRAVITY: 5.01 gm/ml

VAPOR PRESSURE: No data

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: No data available

INCOMPATIBILITY: Water, Strong acids, Carbon dioxide, and Ammonium salts

DECOMPOSITION PRODUCTS: None

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: LD50 Oral – rat – 230 mg/kg

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified as a confirmed

carcinogen by IARC, ACGIH, NTP, and OSHA.

YTTRIUM(III) OXIDE                                                                           PAGE 2

Page 3

SECTION 11 TOXICOLOGICAL DATA (Cont.)

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: No data available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Non-hazardous material

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 313 COMPONENTS: This material does not contain any chemical components with know CAS numbers that exceed the

threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

SARA 302 COMPONENTS: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA 311/312 HAZARDS: Acute Health Hazard

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right To Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Yttrium oxide

NEW JERSEY RIGHT TO KNOW COMPONENTS: Yttrium oxide

CALIFORNIA PROP. 65 COMPONENTS: This product does not contain any chemicals known to State of California to cause cancer,

birth defects, or any other reproductive harm.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 05/14

YTTRIUM (III) OXIDE                                                                           PAGE 3