Yttrium Nitrate Solution SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/yttrium-nitrate-solution-sds-prochem-en-084897b07e

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:   Yttrium Nitrate Solution

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word    Warning

Hazard Statements

H272             May intensify fire; oxidizer

H302                Harmful if swallowed

H315               Causes skin irritation

H318               Causes serious eye irritation

Precautionary Phrases

P210              Keep away from heat/sparks/openflames/hot surfaces. – No smoking

P262             Do not get in eyes, on skin, or on clothing.

P264             Wash skin thoroughly after handling

P270             Do not eat, drink, or smoke when using this product.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P301 + P312           IF SWALLOWED: call a POISON CENTER or doctor/physician IF you feel unwell.

P302 + P352           IF ON SKIN: wash with plenty of soap and water.

P305 + P351 + P338   IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing.

P332 + P313           IF SKIN irritation occurs: Get medical advice/attention.

P337 + P313           IF eye irritation persists: Get medical advice/attention.

P362               Take off contaminated clothing and wash before reuse.

YTTRIUM NITRATE SOLUTION                                                                    PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

HMIS RATING:

Health: 1                         Flammability: 0                        Reactivity: 0

NFPA RATING:

Health: 1                            Fire: 0                                     Stability: 0

POTENTIAL HEALTH EFFECTS: Eye, Skin, and Inhalation

EYE CONTACT: Causes severe irritation of the eyes.

SKIN CONTACT: Causes irritation to respiratory system.

INHALATION: Causes irritation to respiratory system.

INGESTION: No data

ACUTE HEALTH EFFECTS: No data

CHRONIC HEALTH EFFECTS: No information available on long-­‐term chronic effects.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula:

Molecular Weight: No data

CHEMICAL NAME                        CAS#               %                ACGIH (TWA)            OSHA (PEL)

Yttrium Nitrate Hexahydrate                          13494-98-9                    72-78                  Not established               Not established

Water                                            7732-18-5                     22-28

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water (lukewarm if possible) for several minutes. Assure

flushing under eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with soap and water. Remove contaminated clothes if necessary. Seek medical

assistance if irritation or redness persists.

INHALATION: Remove victim to fresh air. Closely monitor the victim for signs of respiratory problems, such as difficulty in breathing,

coughing, wheezing, or pain. In such cases seek immediate medical assistance, Give oxygen if deemed necessary.  If not breathing

administer CPR. Get emergency care immediately.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Do not induce vomiting.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not applicable

AUTO IGNITION TEMPERATURE: Not applicable

EXPLOSION LIMITS: Not applicable

EXTINGUISHING MEDIUM: Use water spray. This product may lower the ignition temperature of flammable substances. This

product contains oxidizers. Ignition may be caused through the heat of the reaction with reducing agents and the agentʼs

combustible components.

SPECIAL FIRE FIGHTING PROCEDURES:   If this product is involved in a fire, fire fighters should be equipped with a NIOSH

approved positive pressure self-contained breathing apparatus and full protective clothing. Use water spray to cool fire-exposed

containers and structures.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: None known.

UNUSUAL FIRE OR EXPLOSION HAZARDS: None known

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECUATIONS: Evacuate area. Use adequate protection when handling. Use self-contained breathing apparatus for

large spills.

ENVIRONMENTAL PRECAUTIONS:. This material should be prevented from contaminating soil or from sewer and drainage systems

and bodies of water.

YTTRIUM NITRATE SOLUTION                                                                    PAGE 2

Page 3

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Isolate spillage. Absorb spill with inert materials, then place

in applicable chemical waste container. Retain all contaminated solage from removal and treatment. Dispose of in accordance with all

waste disposal regulations.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Prevent prolonged breathing of vapors or spray mist. Avoid contact with eyes and skin. Do

not take internally. Do not handle until the manufacturers safety precautions have been read and understood. Handle in accordance

with good industrial hygiene and safety practice. Protect container against physical damage.

CONDITIONS FOR SAFE STORAGE: Keep container tightly closed. Keep containers upright to prevent leakage. Use and store this

material in a cool, dry, well-ventilated area away from heat and all sources of ignition. Store away from incompatible materials. Do not

store near reducing agents, acids, alkalies, and flammable substances.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EYE PROTECTION: Always wear approved safety glasses with side shields, safety goggles, and face shield when

handling a chemical substance in the laboratory.

SKIN PROTECTION: Wear PVC, rubber, neoprene

VENTILATION: Mechanical: Acceptable.    If possible, handle the material in a local exhaust hood.

RESPIRATOR: If ventilation is not available, a high efficiency acid/mist respirator should be worn. The use of respirators requires a

Respirator Protection Program to be in compliance with 29CFR 1910.34.

ADDITIONAL PROTECTION: Protection to avoid contact with clothing and shoes. (Chemical-resistant lab apron, overalls, booties,

sleevelets)

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Clear liquid

MOLECULAR WEIGHT: No data

MELTING POINT: Not applicable

SPECIFIC GRAVITY: 1.633

BOILING POINT: No data

VAPOR PRESSURE: Not applicable

VAPOR DENSITY: Not applicable

% VOLATILE BY VOLUME: Non volatile

SOLUBILITY IN WATER: Soluble

ODOR: Slight nitric odor

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: None known

INCOMPATIBILITY: Avoid reducing agents. Incompatible with strong acids, alkali, oxidizing agents, water, reducing

agents, flammables, alkali metals, and aluminum alloys.

DECOMPOSITION PRODUCTS: None known

SECTION 11 TOXICOLOGICAL DATA

CARCINOGENIC EFFECTS: No

MUTAGENIC EFFECTS: No

TETRATOGENIC EFFECTS: No

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

YTTRIUM NITRATE SOLUTION                                                                    PAGE 3

Page 4

SECTION 12  ECOLOGICAL DATA

No information available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local state and federal regulations.

SECTION 14  TRANSPORTATION DATA

Nitrates, inorganic, aqueous solution, n.o.s. (Yttrium Nitrate Solution)

Class 5.1

UN3218

PG II

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in TSCA inventory

SARA (TITLE 313): Total Nitrates (in solution)

RCRA STATUS: Meets criteria for nitrates and should be manages as a hazardous waste.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 10/13

YTTRIUM NITRATE SOLUTION                                                             PAGE 4