Yttrium, powder 3826 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/yttrium-powder-sds-prochem-en-86e1123197

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:     Yttrium, powder

PRODUCT NUMBER:  3826

CAS NUMBER:        7440-65-5

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228               Flammable solid.

H250                Catches fire spontaneously if exposed to air.

H302+H312+H332    Harmful if swallowed, in contact with skin, or if inhaled.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces.

P222             Do not allow contact with air.

P231                Handle under inert gas.

P240                Ground/bond container and receiving equipment.

P241              Use explosion-proof electrical/ventilating/lighting equipment.

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P270             Do not eat, drink, or smoke when using this product.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/eye protection/face protection.

P301+P312            IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P335+P334          Brush off loose particles from skin. Immerse in cool water/wrap in wet bandages

P363             Wash contaminated clothing before reuse

P370+P378             In case of fire: Use dry sand, dry chemical or alcohol-resistant foam for extinction

YTTRIUM POWDER                                                                     PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

HMIS CLASSIFICATION:

Health: 2              Fire: 3                 Reactivity Hazard: 3

NFPA RATING:

Health: 2            Flammability: 3         Reactivity Hazard: 3

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: Inhalation of fine dust may be irritating to the nasal cavity, mucous membranes, and respiratory tract.

INGESTION: May be harmful if swallowed.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Y

Molecular Weight: 88.90

CHEMICAL NAME                                           CAS#                               %

Yttrium powder                                                          7440-65-5                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under the

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Special powder for metal fires. Do not use water.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Yttrium oxides

UNUSUAL FIRE OR EXPLOSION HAZARDS: Flammable solid. Burns with intense heat. Dust may form explosive mixtures

with air. Evolves hydrogen with hot water.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Keep unprotected

persons away. Keep away from sources of ignition.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be covered in sodium carbonate to reduce

formation of dust. Sweep up mixture.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Avoid

contact with skin and eyes. Avoid formation of dust. Keep away from sources of ignition-No smoking.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store in a tightly sealed container under an inert

atmosphere of argon. Store away from air, moisture, and water. Moisture and air sensitive. Protect from humidity and water.

YTTRIUM POWDER                                                                     PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS

Component                 Exposure Limits              Basis                           Entity

Yttrium                      1 mg/m3                   TLV                     ACGIH

1 mg/m3                  PEL                   OSHA

1 mg/m3                  REL                     NIOSH

TLV: Threshold Limit Value over 8 hours of work.            REL: Recommended Exposure Limit

PEL: Permissible Exposure Limit

EYE PROTECTION: Wear chemical safety glasses or goggles.

SKIN PROTECTION: Wear nitrile or rubber gloves, and ensure that your protective clothing is fire retardant.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Gray powder

ODOR: None

MOLECULAR WEIGHT: 88.90

BOILING POINT: 3338°C

MELTING POINT: 1522°C

SPECIFIC GRAVITY: 4.47 gm/ml

VAPOR DENSITY: No data available

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Contact with hot water. Heat, flames, and sparks. Extremes of temperature and direct sunlight.

INCOMPATIBILITY: Strong oxidizing agents, water, halogens, halocarbons, and mineral acids.

DECOMPOSITION PRODUCTS: Yttrium Oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: ZG2980000

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

YTTRIUM POWDER                                                                     PAGE 3

Page 4

SECTION 14  TRANSPORTATION DATA

DOT:   UN3089                                                 IATA:  UN3089

Metal powders, flammable, n.o.s.                                    Metal powder, flammable, n.o.s.

(Yttrium)                                                                  (Yttrium)

CLASS 4.1                                          CLASS 4.1

PG II                                          PG II

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Fire Hazard, Acute Health Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW: Yttrium

PENNSYLVANIA RIGHT TO KNOW: Yttrium

NEW JERSEY RIGHT TO KNOW: Yttrium

WHMIS CANADA: Class B-4: Flammable solid

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 09/14

YTTRIUM POWDER                                                                     PAGE 4