Gadolinium SDS

Publisher: Trace Sciences

Source: https://resds.com/document/gadolinium-sds-trace-en-2a31353ddd

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

Section 1: Identification

Product name:         Gadolinium

Product use:           For laboratory research purposes.

Supplier:              Trace Sciences International

40 Vogell Rd Suite 42

Richmond Hill, ON L4B 3N6

CANADA

Telephone:           +1 905-770-1100

Fax:                +1 905-770-1160

Emergency Phone:    CANUTEC +1-613-996-6666

Section 2: Hazard(s) Identification

2.1 GHS Classification

Substances and mixtures which in contact with water emit flammable gases (Category 3), H261

2.2 GHS Label elements, including precautionary statements

Pictogram

Signal word              Warning

Hazard statement(s):

H261                          In contact with water releases flammable gases.

Precautionary statement(s):

P231 + P232              Handle under inert gas. Protect from moisture.

P280                  Wear protective gloves/ protective clothing/ eye protection/ face protection

P370 + P378                  In case of fire: Use dry sand, dry chemical or alcohol-resistant foam to

extinguish.

P402 + P404               Store in a dry place. Store in a closed container.

P501                     Dispose of contents/ container to an approved waste disposal plant.

Section 3: Composition/ Information on Ingredients

Formula                 :  Gd

Molecular Weight    :  157.25  g/mol

Material          CAS-No.          EC-No.            Index-No.          Concentration

Gadolinium         7440-54-2          231-162-2               -               <=100%

Section 4: First-Aid Measures

4.1 Description of first aid measures

General advice

Consult a physician. Show this safety data sheet to the doctor in attendance. Move person out of

dangerous area if safe to do so.

Gadolinium SDS                       Page 1 of 6                 Trace Sciences International

Page 2

If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a

physician.

In case of skin contact

Take off contaminated clothing and shoes immediately. Wash off with soap and plenty of water for at

least 15 minutes. Use chemical shower if available. Consult a physician.

In case of eye contact

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician. Remove

contacts if possible.

If swallowed

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with

water. Consult a physician.

Section 5: Fire-Fighting Measures

5.1 Conditions of flammability

May burn in presence of air or emit a flammable gas in the presence of water or water vapour. Keep away

from heat/sparks/open flame/hot surface/air/water. No smoking.

5.2 Suitable extinguishing media

Dry powder, carbon dioxide (CO2), DO NOT USE WATER

5.3 Hazardous combustion products

Hazardous decomposition products formed under fire conditions: gadolinium oxides

5.4 Special protective equipment for firefighters

Wear self-contained breathing apparatus for firefighting if necessary.

5.5 Further information

No data available

Section 6: Accidental Release Measures

6.1 Personal precautions

Use personal protective equipment. Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure

adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

6.2 Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

6.3 Methods and materials for containment and cleaning up

Sweep up and shovel. Contain spillage, and then collect with an electrically protected vacuum cleaner or by

wet-brushing and place in container for disposal according to local regulations. Keep in suitable, closed

containers for disposal.

Section 7: Handling and Storage

7.1 Precautions for safe handling

Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Provide appropriate exhaust

ventilation at places where dust is formed.

Gadolinium SDS                       Page 2 of 6                 Trace Sciences International

Page 3

7.2 Conditions for safe storage

Air and moisture sensitive. Never allow product to get in contact with water during storage.

Keep container tightly closed in a dry and well-ventilated place.

Section 8: Exposure Controls/Personal Protection

8.1 Components with workplace control parameters

No data available

8.2 Personal protective equipment

Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle

respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering

controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use

respirators and components tested and approved under appropriate government standards such as

NIOSH (US) or CEN (EU).

Hand protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique

(without touching glove's outer surface) to avoid skin contact with this product. Dispose of

contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

Eye protection

Safety glasses with side-shields conforming to EN166. Use equipment for eye protection tested and

approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin and body protection

Flame retardant protective clothing, the type of protective equipment must be selected according to

the concentration and amount of the dangerous substance at the specific workplace.

Hygiene measures

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks

and at the end of workday.

Specific engineering controls

Use mechanical exhaust or laboratory fume hood to avoid exposure.

Section 9: Physical and Chemical Properties

Appearance

Form                                 Pieces

Colour                                    Light grey

Safety Data

pH                           No data available

Melting point/freezing point              1,313 °C (2,395 °F)

Boiling point                           3,273 °C (5,923 °F)

Flash point                           Not applicable

Flammability (solid, gas)            No data available

Ignition temperature               No data available

Auto-ignition temperature            No data available

Lower explosion limit               No data available

Upper explosion limit               No data available

Gadolinium SDS                       Page 3 of 6                 Trace Sciences International

Page 4

Vapour pressure                  No data available

Density                               7.886 g/mL at 25 °C (77 °F)

Water solubility                   No data available

Partition coefficient: n-octanol/water     No data available

Relative vapour density             No data available

Odour                         No data available

Odour Threshold                  No data available

Evaporation rate                  No data available

Section 10: Stability and Reactivity

10.1 Chemical stability

Stable under recommended storage conditions

10.2 Possibility of hazardous reactions

Reacts violently with water.

Violent reactions possible with:

Oxygen, Strong acids, Strong oxidizing agents

10.3 Conditions to avoid

Exposure to moisture.

10.4 Materials to avoid

Oxygen, Strong acids, Strong oxidizing agents

10.5 Hazardous decomposition products

See section 5

Section 11: Toxicological Information

Acute toxicity

Oral

No data available

Inhalation

No data available

Dermal

No data available

Other information on acute toxicity

No data available

Skin corrosion/irritation

No data available

Serious eye damage/eye irritation

No data available

Respiratory or skin sensitization

No data available

Germ cell mutagenicity

No data available

Gadolinium SDS                       Page 4 of 6                 Trace Sciences International

Page 5

Carcinogenicity

No data available

Reproductive toxicity

No data available

Specific target organ toxicity - single exposure (Globally Harmonized System)

No data available

Specific target organ toxicity - repeated exposure (Globally Harmonized System)

No data available

Aspiration hazard

No data available

Signs and Symptoms of Exposure

No data available

Additional Information

RTECS: Not available

Section 12: Ecological Information

12.1 Toxicity

No data available

12.2 Persistence and degradability

No data available

12.3 Bioaccumulative potential

No data available

12.4 Mobility in soil

No data available

12.5 PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

12.6 Other adverse effects

No data available

Section 13: Disposal Considerations

Product

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting as

this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company.

Contact a licensed professional waste disposal service to dispose of this material.

Contaminated packaging

Dispose of as unused product.

Gadolinium SDS                       Page 5 of 6                 Trace Sciences International

Page 6

Section 14: Transport Information

IATA

UN number:  UN3208  Class:  4.3       Packing group:     III

Proper shipping name:   Metallic substance, water-reactive, n.o.s. (Gadolinium)

Section 15: Regulatory Information

This product has been classified in accordance with the hazard criteria of the Hazardous Products

Regulations (HPR) and the SDS contains all the information required by the HPR

Section 16: Other Information

Further information

Although reasonable care has been taken in the preparation of this document, we extend no warranties and

make no representations as to the accuracy or completeness of the information contained herein, and

assume no responsibility regarding the suitability of this information for the user’s intended purposes or for

the consequences of its use. Each individual should make a determination as to the suitability of the

information for his or her particular purpose(s).

Trace Sciences International.

See https://www.tracesciences.com/ for additional terms and conditions of sale.

Date Prepared: February 10, 2025

Gadolinium SDS                       Page 6 of 6                 Trace Sciences International