Thulium 3586 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/thulium-prochem-en-997fd94f6d

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Thulium

PRODUCT NUMBER:  3586

CAS NUMBER:        7440-30-4

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228               Flammable solid

H319               Causes serious eye irritation

H335             May cause respiratory irritation

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces

P240                Ground/bond container and receiving equipment

P241              Use explosion-proof electrical/ventilating/lighting equipment

P261                 Avoid breathing dust/fume/gas/mist/vapours/spray.

P264             Wash skin thoroughly after handling.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/protective clothing/eye protection/face protection

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338      IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

P312                    Call a POISON CNTER or doctor/physician if you feel unwell.

P370+P378             In case of fire: Use dry sand, dry chemical or alcohol-resistant foam for extinction

P403+P233            Store in a well-ventilated place.

P501                Dispose of contents/container to an approved waste disposal plant.

THULIUM                                                                           PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 0

EYE CONTACT: Dust, if present, may cause slight to mild irritation of the eyes.

SKIN CONTACT: Dust, if present, may cause slight to mild irritation of the skin.

INHALATION: Inhalation of any fine dust may be irritating to the nasal cavity, mucous membranes, and respiratory tract.

INGESTION: May be harmful if swallowed

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Tm

Molecular Weight: 168.93

CHEMICAL NAME                                           CAS#                               %

Thulium                                                                7440-30-4                                     100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Do NOT induce vomiting. Never give anything by mouth to an unconscious person.  If conscious, wash out mouth

with water. Get medical attention if necessary.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use water spray, alcohol-resistant foam, dry chemical, or carbon dioxide. Use water to spray unopened

containers.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Thulium oxide.

UNUSUAL FURE OR EXPLSION HAZARDS: May burn with intense heat. Dust may form explosive mixtures with air.

Evolves hydrogen with hot water.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid breathing

dust. Remove all sources of ignition

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up mixture and place in a closed container. Do not

mix with water or other incompatible materials.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Wash thoroughly after using. Keep container closed when not in use. Avoid formation of dusts and aerosols. Keep

away from sources of ignition – No smoking. Take measures to prevent the build-up of electrostatic charge.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Air and moisture sensitive. Store and handle under

an inert atmosphere of argon.

THULIUM                                                                           PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Grey chips

ODOR: None

MOLECULAR WEIGHT: 168.93

BOILING POINT: 1950° C

MELTING POINT: 1545° C

SPECIFIC GRAVITY: 9.332 gm/ml

VAPOR DENSITY: No data available

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: No hazardous polymerization.

CONDITIONS TO AVOID: Contact with hot water, fine dust mixed with air. Heats, flames, and sparks. Extremes of temperature and

direct sunlight

INCOMPATIBILITY: Strong oxidizing agents, water, alcohols, oxygen, halogens, halocarbons, and strong acids.

DECOMPOSITION PRODUCTS: Thulium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

Not a dangerous good.

THULIUM                                                                           PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

DSCL (EEC): Listed on the DSCL inventory

SARA 302/304: Not Listed

SARA 311/312: Acute Health Hazard

SARA (TITLE 313): Not Listed

PENNSYLVANIA RIGHT TO KNOW COMPONENETS: Thulium

NEW JERSEY RIGHT TO KNOW COMPONENTS: Thulium

CALIFORNIA PROP. 65: Not Listed

WHMIS CANADA: Not Listed

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 07/14

THULIUM                                                                           PAGE 4