Thulium (III) Fluoride, anhydrous 3604 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/thulium-iii-fluoride-anhydrous-prochem-en-27a69c7dfd

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Thulium (III) Fluoride, anhydrous

PRODUCT NUMBER:  3604

CAS NUMBER:        13760-79-7

SYNONYMS:         Thulium trifluoride

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H301+H311+H331     Toxic if swallowed, in contact with skin, or if inhaled.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P261                 Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling.

P270             Do not eat, drink, or smoke when using this product.

P271              Use only outdoors or in a well-ventilated area.

P280              Wear protective gloves/protective clothing/eye protection/face protection

P301+P310            IF SWALLOWED: Immediately call a POISON CENTER or doctor/physician.

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P312                   Call a POISON CNTER or doctor/physician if you feel unwell.

P361               Remove/take off immediately all contaminated clothing.

P363             Wash contaminated clothing before reuse.

P403+P233           Store in a well-ventilated place.

THULIUM (III) FLUORIDE                                                                 PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

HMIS CLASSIFICATION:

Health: 2              Fire: 0                  Reactivity Hazard: 0

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 0

EYE CONTACT: Causes irritation of the eyes.

SKIN CONTACT: Causes slight to mild irritation of the skin.

INHALATION:  Irritating to the respiratory tract.

INGESTION: Toxicity reduced to the extent that this compound is relatively insoluble. In large amounts may cause vomiting,

abdominal pain, and diarrhea.

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: TmF3

Molecular Weight: 225.93

CHEMICAL NAME                                           CAS#                               %

Thulium (III) Fluoride                                                    13760-79-7                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Do NOT induce vomiting. Never give anything by mouth to an unconscious person.  If conscious, wash out mouth

with water. Get medical attention if necessary.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is not flammable

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use fire fighting measures that suit the surrounding fire.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Hydrogen fluoride and Thulium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid breathing

dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Pick up and arrange disposal without creating dust. Sweep

up and keep in suitable, closed containers for disposal. Do not flush with water. Dispose of all waste and cleanup materials in

accordance with regulations.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Use with adequate ventilation. Wash

thoroughly after using. Keep container closed when not in use. Avoid formation of dusts and aerosols.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Handle under a dry atmosphere or air or nitrogen.

Prolong exposure to the atmosphere may degrade the product. Store away from water, moisture, and acids.

THULIUM (III) FLUORIDE                                                                 PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS

Component                 Exposure Limits              Basis                           Entity

Thulium (III) Fluoride             2.5 mg/m3                  TLV                     ACGIH

2.5 mg/m3                 PEL                   OSHA

2.5 mg/m3                 REL                     NIOSH

3mg/g*                       BEI-U                    ACGIH

10mg/g**                     BEI-U                    ACGIH

*Prior to shift    **End of shift

TLV: Threshold Limit Value over 8 hours of work.            REL: Recommended Exposure Limit

PEL: Permissible Exposure Limit                            BEI-U: Biological Exposure Indices in Urine

EYE PROTECTION: Wear chemical safety glasses or goggles.

SKIN PROTECTION: Wear nitrile or rubber gloves, apron, or lab coat.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Off white powder

ODOR: None

MOLECULAR WEIGHT: 225.93

BOILING POINT: 2200° C

MELTING POINT: 1158° C

SPECIFIC GRAVITY: 7.9 gm/ml

VAPOR DENSITY: No data available

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Contact with moisture

INCOMPATIBILITY: Strong oxidizing agents, acids, water, and moisture.

DECOMPOSITION PRODUCTS: Thulium oxides and hydrogen fluoride

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Harmful if inhaled, in contact with skin, or swallowed. Danger through skin absorption.

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

THULIUM (III) FLUORIDE                                                                 PAGE 3

Page 4

SECTION 14  TRANSPORTATION DATA

UN3288

Toxic solid, inorganic, n.o.s.

(Thulium (III) fluoride)

CLASS 6.1

PG III

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

DSCL (EEC): Listed on the DSCL inventory

SARA 302/304: Not Listed

SARA 311/312: Acute Health Hazard

SARA (TITLE 313): Not Listed

PENNSYLVANIA RIGHT TO KNOW: Thulium (III) Fluoride

NEW JERSEY RIGHT TO KNOW: Thulium (III) Fluoride

CALIFORNIA PROP. 65: Not Listed

WHMIS CANADA: Class D-1B: Toxic material causing immediate and serious toxic effects. Class D-2B: Toxic material causing other

toxic effects.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 09/14

THULIUM (III) FLUORIDE                                                                 PAGE 4