Praseodymium (III) Nitrate Hexahydrate 2974 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/praseodymium-iii-nitrate-hexahydrate-prochem-en-a4348dd6cb

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Praseodymium (III) Nitrate Hexahydrate

PRODUCT NUMBER:  2974

CAS NUMBER:        15878-77-0

SYNONYMS:           Nitric acid, praseodymium(3+) salt, hexahydrate

MANUFACTURER:

ProChem, Inc.                                        PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                    TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Warning

Hazard Statements

H272             May intensify fire; oxidizer.

H315              Causes skin irritation

H319              Causes serious eye irritation

H335             May cause respiratory irritation

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces.-No Smoking

P220                 Keep/store away from clothing/combustible material

P221               Take any precaution to avoid mixing with combustibles.

P261                Avoid breathing dust/fume/gas/mist/vapors/spray.

P264             Wash skin thoroughly after handling.

P280              Wear protective gloves/protective clothing/eye protection/face protection

P302+P352            IF ON SKIN: Wash with plenty of soap and water.

P304+P340            IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P305+P351+P338     IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

P370+P378            In case of fire: use dry sand, dry chemical or alcohol-resistant foam for extinction.

P403+P233           Store in a well-ventilated place. Keep container tightly closed.

PRASEODYMIUM NITRATE                                                               PAGE 1

Page 2

SECTION 2    HAZARDS IDENTIFICATION (Cont.)

HMIS CLASSIFICATION:

Health: 1              Fire: 0                 Reactivity Hazard: 1

NFPA RATING:

Health: 2            Flammability: 0         Reactivity Hazard: 1       Specific Hazard: OX

EYE CONTACT: May cause slight to mild irritation to the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: May be irritating to the nose, mucous membranes, and respiratory tract.

INGESTION: May be harmful if swallowed. Ingestion may lead to dizziness, abdominal cramps, vomiting, bloody diarrhea, weakness,

and convulsions

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Pr(NO3)3 ! 6H2O

Molecular Weight: 435.01

CHEMICAL NAME                                           CAS#                               %

Praseodymium Nitrate Hexahydrate                                        15878-77-0                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate medical attention.

SKIN EXPOSURE: Wash affected area with water. Remove contaminated clothes if necessary. Seek medical assistance if

irritation persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention if necessary.

INGESTION: Do NOT induce vomiting. Never give anything by mouth to an unconscious person.  If conscious, wash out mouth

with water. Get medical attention if necessary.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Carbon dioxide, dry powder, or foam.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Nitrogen oxides and praseodymium oxides.

This product is an oxidizer and its heat of reaction with reducing agents or combustibles may cause ignition.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Keep away from

heat and sources of ignition.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Small spills can be mixed with vermiculite or sodium

carbonate and swept up.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Keep away from heat sources of ignition.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store away from flammable substances, reducing

agents, and metal powders. Hygroscopic.

PRASEODYMIUM NITRATE                                                               PAGE 2

Page 3

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Light green solid

ODOR: None

MOLECULAR WEIGHT: 435.01

BOILING POINT: No data available

MELTING POINT: No data available

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY IN WATER: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Contact with strong reducing agents or organic matter. High temperature (some nitrates can explode if

heated to high temperature).

INCOMPATIBILITY: Strong reducing agents, powdered metals, strong acids, bases, flammable substances, organic materials,

sulfur, and phosphorus.

DECOMPOSITION PRODUCTS: Nitrogen oxides and praseodymium oxide.

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: LD50 Oral – rat – 3,500mg/kg.

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: TU1400000

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

TOXICITY TO FISH: LC50-Onorhynchus mykiss (rainbow trout)-0.7 mg/l – 96h

TOXICITY TO DAPHNIA AND OTHER AQUATIC INVERTEBRATES: EC50-Daphnia 0 <1 mg/l – 48h

TOXICITY TO ALGAE: IC50-algae - < 1 mg/l – 72 h

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

PRASEODYMIUM NITRATE                                                               PAGE 3

Page 4

SECTION 14  TRANSPORTATION DATA

UN1477                                Marine Pollutant: No

Nitrates, inorganic, n.o.s.

(Praseodymium Nitrate)

CLASS 5.1

PG III

SECTION 15  REGULATORY INFORMATION

TSCA: Not listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Reactivity Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

NEW JERSEY RIGHT TO KNOW COMPONENTS: Praseodymium (III) Nitrate Hexahydrate

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Praseodymium (III) Nitrate Hexahydrate

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, ProChem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE REVISED: 03/22

SDS DEPT.

PRASEODYMIUM NITRATE                                                               PAGE 4