Platinum-195 SDS

Publisher: Neonest AB

Source: https://resds.com/document/cisplatin-enriched-cisplatin-sds-33f6f14d4e

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Cisplatin, Enriched Cisplatin

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 21-Oct-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Cisplatin, Enriched Cisplatin   https://www.buyisotope.com   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Cisplatin, Enriched Cisplatin.

Molecular weight:   300.05

Chemical formula:   Pt(NH 3 ) 2 Cl 2

Synonyms:   cis-Diammineplatinum(II) dichloride, Cisplatinum, cis-Dichlorodiammine platinum(II), cis-Platinum(II)

diammine dichloride.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Acute Toxicity (Category 2)   H300

Eye Damage (Category 1)   H318

Carcinogenic (Category 1B)   H350

Label elements:

Hazard pictograms:

Page 2

Safety Data Sheet for Cisplatin, Enriched Cisplatin

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 21-Oct-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Cisplatin, Enriched Cisplatin   https://www.buyisotope.com   Page 2/9

Signal Words:   Danger.

Hazard Statements:

H300 Fatal if swallowed.

H318 Causes serious eye damage.

H350 May cause cancer.

Precautionary Statements:

P201 Obtain special instructions before use.

P202 Do not handle until all safety precautions have been read and understood.

P264 Wash hands thoroughly after handling.

P270 Do not eat, drink or smoke when using this product.

P280 Wear protective gloves/protective clothing/eye protection/face protection.

P301 + P310 IF SWALLOWED: Immediately call a POISON CENTER/doctor.

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and

easy to do. Continue rinsing.

P308 + P313 IF exposed or concerned: Get medical advice/attention.

P310 Immediately call a POISON CENTER/doctor.

P330 Rinse mouth.

P405 Store locked up.

P501 Dispose of contents/container in accordance with local/national regulations.

Other hazards:   None known.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Molecular

weight

Concentration

% by weight

Classification

EC1272/2008 EC - No.

Cisplatin

15663-27-1

300.05   >99%

Acute Tox. 2   H300

Eye Dam. 1   H318

Carc. 1B   H350 239-733-8

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:   If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact:   Wash off with soap and plenty of water. Take victim immediately to hospital. Consult a

physician.

In case of eye contact:   Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:   Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed:

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11.

Indication of any immediate medical attention and special treatment needed:   Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Page 3

Safety Data Sheet for Cisplatin, Enriched Cisplatin

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 21-Oct-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Cisplatin, Enriched Cisplatin   https://www.buyisotope.com   Page 3/9

Section 5: Fire - Fighting Measures

Extinguisher media:

Suitable extinguisher media:   Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Unsuitable extinguishing media:   None known.

Special hazards arising from the mixture:   Nitrogen oxides (NOx), Hydrogen chloride gas.

Advice for fire-fighters:   Wear self-contained breathing apparatus for firefighting if necessary.

Further information:   No data available.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions:   Wear respiratory protection. Avoid dust formation. Avoid breathing vapours, mist or gas.

Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

Environmental precautions:   Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods for containment and cleaning up:

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed containers for

disposal.

Reference to other sections:

For disposal see section 13.

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling:   Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Avoid exposure -

obtain special instructions before use. Provide appropriate exhaust ventilation at places where dust is formed.

Hygiene measures:   Do not eat, drink or smoke when using this product.

Conditions for safe storage, including incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Recommended storage

temperature 2 -   8 °C. Storage class (TRGS 510): Combustible solids, toxic.

Specific end use(s):   Apart from the uses mentioned in section 1.2 no other specific uses are stipulated.

Section 8: Exposure Controls/ Personal Protection

Control parameters:

Occupational exposure limits:   No data available.

Exposure controls:

Appropriate engineering controls:   Avoid contact with skin, eyes and clothing. Wash hands before breaks and

immediately after handling the product.

Individual protection measures, such as personal protective equipment:

Eye/face protection:   Face shield and safety glasses Use equipment for eye protection tested and approved under

Page 4

Safety Data Sheet for Cisplatin, Enriched Cisplatin

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 21-Oct-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Cisplatin, Enriched Cisplatin   https://www.buyisotope.com   Page 4/9

appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection:   Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique

(without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after

use in accordance with applicable laws and good laboratory practices. Wash and dry hands. The selected protective

gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Body Protection:   Complete suit protecting against chemicals, the type of protective equipment must be selected

according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory Protection:   Where risk assessment shows air-purifying respirators are appropriate use (EN 143) respirator

cartridges as a backup to engineering controls. If the full-face supplied air respirator. Use respirators and components

tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Control of environmental exposure:   Prevent further leakage or spillage if safe to do so. Do not let product enter

drains.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Solid (powder).

Colour:   Yellow.

Odour:   No data available.

Odour threshold:   No data available.

Molecular Weight:   300.05

pH (concentration):   No data available.

Melting point/range (° C):   270 °C

Boiling point/range (° C):   No data available.

Freezing point   (°C):   No data available.

Flash point (° C):   Not applicable.

Evaporation rate:   No data available.

Flammability (solid, gas):   No data available.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour   pressure (20 ° C):   No data available.

Vapour density:   No data available.

Relative densi ty (25 ° C):   3.7 g/cm 3

Water solubility (g/L ) at 20 ° C:   <0.1 g/100 mL

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Reactivity:   No data available.

Chemical stability:   Stable under recommended storage conditions.

Possibility of hazardous reactions:   No data available.

Conditions to avoid:   No data available.

Page 5

Safety Data Sheet for Cisplatin, Enriched Cisplatin

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 21-Oct-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Cisplatin, Enriched Cisplatin   https://www.buyisotope.com   Page 5/9

Incompatible materials:   Strong oxidizing agents.

Hazardous decomposition products:   Hazardous decomposition products formed under fire conditions: Nitrogen

oxides (NOx), Hydrogen chloride gas.

Section 11: Toxicological Information

Information on toxicological effects:

Acute toxicity:   LD50 Oral - Rat - 25.8 mg/kg (Cisplatin)

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

Prolonged or repeated exposure may cause allergic reactions in certain sensitive individuals.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Possible human carcinogen.   IARC: 2A   -   Group 2A: Probably carcinogenic to humans

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecolo gical Information

Toxicity:   No data available.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results   of   PBT&   vPvB   assessment:   PBT/vPvB   assessment   not   available   as   chemical   safety   assessment   not

required/not conducted.

Other adverse effects:   No data available.

Section 13: Disposal Considerations

Product:   Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material with

a combustible solvent and burn in a chem scrubber.

Contaminated packaging:   Dispose of as unused product.

Section 14: Transport Information

Page 6

Safety Data Sheet for Cisplatin, Enriched Cisplatin

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 21-Oct-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Cisplatin, Enriched Cisplatin   https://www.buyisotope.com   Page 6/9

ADR/RID:

Proper Shipping Name:   TOXIC SOLID, INORGANIC, N.O.S. (Cisplatin).

Hazard Class : 6.1

Hazard Label:

UN Number:   3288

Packing Group:   II

IATA:

Proper Shipping Name:   TOXIC SOLID, INORGANIC, N.O.S. (Cisplatin).

Hazard Class : 6.1

UN Number:   3288

Packing Group:   II

IMDG:

Proper Shipping Name:   TOXIC SOLID, INORGANIC, N.O.S. (Cisplatin).

Hazard Class : 6.1

UN Number:   3288

Packing Group:   II

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Page 7

Safety Data Sheet for Cisplatin, Enriched Cisplatin

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 21-Oct-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Cisplatin, Enriched Cisplatin   https://www.buyisotope.com   Page 7/9

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations:   The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other   Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

Page 8

Safety Data Sheet for Cisplatin, Enriched Cisplatin

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 21-Oct-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Cisplatin, Enriched Cisplatin   https://www.buyisotope.com   Page 8/9

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

Page 9

Safety Data Sheet for Cisplatin, Enriched Cisplatin

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 21-Oct-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Cisplatin, Enriched Cisplatin   https://www.buyisotope.com   Page 9/9

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungskl assen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H300 Fatal if swallowed.

H318 Causes serious eye damage.

H350 May cause cancer.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.