Europium Powder 1753 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/europium-powder-prochem-en-6ec18a8feb

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Europium Powder

PRODUCT NUMBER:  1753

CAS NUMBER:        7440-53-1

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228               Flammable solid.

H260                    In contact with water releases flammable gases which may ignite spontaneously.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces. No smoking.

P231+P232          Handle under inert gas. Protect from moisture.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P370+P378             In case of fire: Use dry sand, dry chemical or alcohol-resistant foam for extinction.

P402+P404           Store in a dry place. Store in a closed container.

HMIS CLASSIFICATION:

Health: 0              Fire: 3                  Reactivity Hazard: 1

NFPA RATING:

Health: 0            Flammability: 0         Reactivity Hazard: 1       Specific Hazard: W

EYE CONTACT: May cause moderate irritation.

SKIN CONTACT: May cause irritation.

INHALATION: May cause irritation to the upper respiratory system.

INGESTION: Considered to have low toxicity by ingestion.

EUROPIUM METAL                                                                     PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Eu

Molecular Weight: 151.96

CHEMICAL NAME                                           CAS#                               %

Europium                                                                7440-53-1                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water fore at least 15 minutes. Assure flushing under

eyelids. Seek medical attention.

SKIN EXPOSURE: Remove contaminated clothing, brush material off skin, wash affected area with mild soap and water.

Seek medical attention.

INHALATION: Remove to fresh air. Keep the victim warm and calm. Give oxygen if breathing is difficult. Seek medical

attention.

INGESTION: Never give anything by mouth to an unconscious person.  If conscious, wash out mouth with water. Get medical

attention.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Product is highly flammable. Contact with water liberates extremely flammable gases.

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Special powder for metal fires. Do not use water.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Europium oxide

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Keep away from

sources of ignition. Avoid breathing dusts and vapors.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Without creating dust, sweep up and place in a suitable

container for proper disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Avoid formation of dusts and aerosols. Keep away from sources of ignition – no smoking.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry well ventilated area. Air and moisture sensitive. Store away from air, water,

moisture, oxidizing agents, halogens, acid chlorides, and acids. Store under dry inert gas.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

EUROPIUM METAL                                                                     PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Gray to silver solid

ODOR: None

MOLECULAR WEIGHT: 151.56

BOILING POINT: 1439° C

MELTING POINT: 822°C

SPECIFIC GRAVITY: 5.244 m/ml

VAPOR DENSITY: No data available

SOLUBILITY: Insoluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Air and moisture

INCOMPATIBILITY: Oxidizing agents, halogens, and strong acids

DECOMPOSITION PRODUCTS: Europium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

DOT:   UN3178                            DOT:   UN3178

Flammable solid, inorganic, n.o.s.                 Flammable solids, inorganic, n.o.s.

(Europium metal)                                (Europium metal)

CLASS 4.1                             CLASS 4.1

PG II                              PG II

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

SARA 302/304: Not Listed

SARA 311/312: Reactivity Hazard

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

EUROPIUM METAL                                                                     PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION (Cont.)

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Europium

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Europium

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 12/14

EUROPIUM METAL                                                                     PAGE 4