Gadolinium SDS

Publisher: CDH

Source: https://resds.com/document/gadolinium-sds-en-1df11b0d43

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

GADOLINIUM            MATERIAL SAFETY DATA SHEET

CAS No 7440-54-2                  SDS/MSDS

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1   Product identifiers

Product name                        :                                    Gadolinium

CAS-No.                                 :  7440-54-2

1.2   Relevant identified uses of the substance or mixture and uses advised against

Identified uses                        :  Laboratory chemicals, Industrial & for professional use only.

1.3    Details of the supplier of the safety data sheet

Company                               :   Central Drug House (P) Ltd

7/28 Vardaan House

New Delhi -110002

INDIA

Telephone                              :  +91 11 49404040

Email                                     :  care@cdhfinechemical.com

1.4   Emergency telephone number

Emergency Phone #              :  +91 11 49404040 (9:00am - 6:00 pm) [Office hours]

SECTION 2: Hazards identification

2.1    Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008

Substances, which in contact with water, emit flammable gases (Category 3), H261

For the full text of the H-Statements mentioned in this Section, see Section 16.

2.2   Label elements

Labelling according Regulation (EC) No 1272/2008

Pictogram

Signal word                 Warning

Hazard statement(s)

H261                             In contact with water releases flammable gases.

Precautionary statement(s)

P231 + P232                Handle under inert gas. Protect from moisture.

P422                         Store contents under inert gas.

Supplemental Hazard         none

Statements

2.3   Other hazards - none

Page 1 of 6

Page 2

SECTION 3: Composition/information on ingredients

3.1   Substances

Formula                                  :  Gd

Molecular weight                    :  157.25 g/mol

CAS-No.                                 :  7440-54-2

EC-No.                                   :  231-162-2

No components need to be disclosed according to the applicable regulations.

For the full text of the H-Statements mentioned in this Section, see Section 16.

SECTION 4: First aid measures

4.1   Description of first aid measures

General advice

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact

Flush eyes with water as a precaution.

If swallowed

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

4.2   Most important symptoms and effects, both acute and delayed

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in

section 11

4.3    Indication of any immediate medical attention and special treatment needed

No data available

SECTION 5: Firefighting measures

5.1   Extinguishing media

Suitable extinguishing media

Dry powder Carbon dioxide (CO2)

Unsuitable extinguishing media

Water

5.2   Special hazards arising from the substance or mixture

gadolinium oxides

5.3   Advice for firefighters

Wear self-contained breathing apparatus for firefighting if necessary.

5.4   Further information

No data available

SECTION 6: Accidental release measures

6.1   Personal precautions, protective equipment and emergency procedures

Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Evacuate

personnel to safe areas.

For personal protection see section 8.

Page 2 of 6

Page 3

6.2   Environmental precautions

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

6.3   Methods and materials for containment and cleaning up

Sweep up and shovel.\'20 Contain spillage, and then collect with an electrically protected vacuum

cleaner or by wet-brushing and place in container for disposal according to local regulations (see section

13). Do not flush with water. Keep in suitable, closed containers for disposal.

6.4   Reference to other sections

For disposal see section 13.

SECTION 7: Handling and storage

7.1   Precautions for safe handling

Provide appropriate exhaust ventilation at places where dust is formed.Keep away from sources of ignition

- No smoking.

For precautions see section 2.2.

7.2   Conditions for safe storage, including any incompatibilities

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Never allow product to get in contact with water during storage.

Air and moisture sensitive.

Storage class (TRGS 510): Solid substances which give off flammable gases in contact with water

7.3   Specific end use(s)

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated

SECTION 8: Exposure controls/personal protection

8.1   Control parameters

8.2   Exposure controls

Appropriate engineering controls

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and

at the end of workday.

Personal protective equipment

Eye/face protection

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested and

approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique

(without touching glove's outer surface) to avoid skin contact with this product. Dispose of

contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

Body Protection

Flame retardant protective clothing, The type of protective equipment must be selected according

to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection

Where risk assessment shows air-purifying respirators are appropriate use (EN 143) respirator

cartridges as a backup to engineering controls. If th full-face supplied air respirator. Use respirators

and components tested and approved under appropriate government standards such as NIOSH

(US) or CEN (EU).

Control of environmental exposure

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Page 3 of 6

Page 4

SECTION 9: Physical and chemical properties

9.1   Information on basic physical and chemical properties

a)  Appearance            Form: solid in various forms

Colour: light grey

b)  Odour              No data available

c)  Odour Threshold      No data available

d)  pH                No data available

e)  Melting point/freezing     Melting point/range: 1,313 °C

point

f)    Initial boiling point and    3,273 °C

boiling range

g)  Flash point           No data available

h)  Evaporation rate       No data available

i)   Flammability (solid, gas) No data available

j)   Upper/lower          No data available

flammability or

explosive limits

k)  Vapour pressure       No data available

l)   Vapour density        No data available

m)  Relative density         7.886 g/mL at 25 °C

n)  Water solubility           insoluble

o)  Partition coefficient: n-   No data available

octanol/water

p)  Auto-ignition          No data available

temperature

q)  Decomposition        No data available

temperature

r)   Viscosity            No data available

s)  Explosive properties    No data available

t)   Oxidizing properties    No data available

9.2   Other safety information

No data available

SECTION 10: Stability and reactivity

10.1  Reactivity

No data available

10.2  Chemical stability

Stable under recommended storage conditions.

10.3  Possibility of hazardous reactions

Reacts violently with water.

10.4  Conditions to avoid

Exposure to moisture

10.5  Incompatible materials

Oxygen, Strong acids, Strong oxidizing agents

Page 4 of 6

Page 5

10.6  Hazardous decomposition products

Hazardous decomposition products formed under fire conditions. - gadolinium oxides

Other decomposition products - No data available

In the event of fire: see section 5

SECTION 11: Toxicological information

11.1  Information on toxicological effects

Acute toxicity

No data availableGadolinium

Skin corrosion/irritation

No data available(Gadolinium)

Serious eye damage/eye irritation

No data available(Gadolinium)

Respiratory or skin sensitisation

No data available(Gadolinium)

Germ cell mutagenicity

No data available(Gadolinium)

Carcinogenicity

IARC:    No component of this product present at levels greater than or equal to 0.1% is identified as

probable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity

No data available(Gadolinium)

Specific target organ toxicity - single exposure

No data available(Gadolinium)

Specific target organ toxicity - repeated exposure

No data available

Aspiration hazard

No data available(Gadolinium)

Additional Information

RTECS: Not available

SECTION 12: Ecological information

12.1  Toxicity

No data available

12.2  Persistence and degradability

No data available

12.3  Bioaccumulative potential

No data available

12.4  Mobility in soil

No data available(Gadolinium)

12.5  Results of PBT and vPvB assessment

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted

12.6  Other adverse effects

No data available

Page 5 of 6

Page 6

SECTION 13: Disposal considerations

13.1  Waste treatment methods

Product

Burn in a chemical incinerator equipped with an afterburner and scrubber b highly flammable. Offer

surplus and non-recyclable solutions to a licensed disposal company. Contact a licensed

professional waste disposal service to dispose of this material.

Contaminated packaging

Dispose of as unused product.

SECTION 14: Transport information

14.1  UN number

ADR/RID: 3208                IMDG: 3208                    IATA: 3208

14.2  UN proper shipping name

ADR/RID: METALLIC SUBSTANCE, WATER-REACTIVE, N.O.S. (Gadolinium)

IMDG:   METALLIC SUBSTANCE, WATER-REACTIVE, N.O.S. (Gadolinium)

IATA:    METALLIC SUBSTANCE, WATER-REACTIVE, N.O.S. (Gadolinium)

14.3  Transport hazard class(es)

ADR/RID: 4.3                 IMDG: 4.3                       IATA: 4.3

14.4  Packaging group

ADR/RID: III                  IMDG: III                        IATA: III

14.5  Environmental hazards

ADR/RID: no                IMDG Marine pollutant: no         IATA: no

14.6  Special precautions for user

No data available

SECTION 15: Regulatory information

15.1  Safety, health and environmental regulations/legislation specific for the substance or mixture

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

15.2  Chemical safety assessment

For this product a chemical safety assessment was not carried out

SECTION 16: Other information

Full text of H-Statements referred to under sections 2 and 3.

H261                In contact with water releases flammable gases.

Further information

The above information is believed to be correct but does not purport to be all inclusive and shall be

used only as a guide. The information in this document is based on the present state of our knowledge

and is applicable to the product with regard to appropriate safety precautions. It does not represent

any guarantee of the properties of the product. Central Drug House (P) Ltd and its Affiliates shall not

be held liable for any damage resulting from handling or from contact with the above product. See

www.cdhfinechemical.com for additional terms and conditions of sale.

Page 6 of 6