Gadolinium, Chips 1801 SDS

Publisher: ProChem

Source: https://resds.com/document/gadolinium-chips-sds-en-34922770d9

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Gadolinium, Chips

PRODUCT NUMBER:  1801

CAS NUMBER:        7440-54-2

SYNONYMS:       None

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H228               Flammable solid.

H260                    In contact with water releases flammable gases which may ignite spontaneously.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces.

P231+P232          Handle under inert gas. Protect from moisture.

P280              Wear protective gloves/protective clothing/eye protection/face protection.

P370+P378             In case of fire: Use dry sand, dry chemical or alcohol-resistant foam for extinction.

P402+P404           Store in a dry place. Store in a closed container.

HMIS CLASSIFICATION:

Health: 0              Fire: 3                  Reactivity Hazard: 1

NFPA RATING:

Health: 0            Flammability: 3         Reactivity Hazard: 1       Specific Hazard: W

EYE CONTACT: May cause slight to mild irritation of the eyes.

SKIN CONTACT: May cause slight to mild irritation of the skin.

INHALATION: May be irritating to the nasal cavity, mucous membranes, and respiratory tract.

INGESTION: No specific information is available on the physiological effects of ingestion.

GADOLINIUM CHIPS                                                                    PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Gd

Molecular Weight: 157.25

CHEMICAL NAME                                           CAS#                               %

Gadolinium                                                              7440-54-2                                              100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for at least 15 minutes. Assure flushing under

eyelids. A victim may need assistance in keeping their eyelids open. Get immediate competent medical attention.

SKIN EXPOSURE: Wash affected area with plenty of water. Remove contaminated clothes if necessary. Seek medical

assistance if irritation persists.

INHALATION: Remove to fresh air and keep at rest. Closely monitor the victim for signs of respiratory problems, such as

difficulty in breathing, coughing, wheezing, or pain. In such cases, seek immediate medical assistance.

INGESTION: Seek medical assistance immediately. Keep the victim calm. Give the victim water (only if conscious). Induce

vomiting only if directed by medical personnel.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Dry chemical or class D metal fire agent. Do not use water. Contact with water liberates flammable

gases.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Gadolinium oxides

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust. Keep away from ignition sources

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Sweep up and place in suitable container for proper

disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Avoid contact with skin and eyes. Use with

adequate ventilation. Avoid formation of dusts and aerosols. Handle under dry protective gas. Keep away from sources of ignition – No

smoking.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store under dry inert gas. Protect from humidity and

water. Air sensitive.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

GADOLINIUM CHIPS                                                                    PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Gray pieces

ODOR: None

MOLECULAR WEIGHT: 157.26

BOILING POINT: 3233°C

MELTING POINT: 1313°C

SPECIFIC GRAVITY: 7.898 gm/ml

SOLUBILITY: Reacts with water

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Air sensitive

HAZARDOUS POLYMERIZATION: Reacts violently with water.

CONDITIONS TO AVOID: Exposure to moisture. Contact with water liberates flammable gases.

INCOMPATIBILITY: Air, oxidizing agents, water, moisture, alcohols, halogens, halocarbons, and mineral acids

DECOMPOSITION PRODUCTS: Gadolinium oxides

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: TDLo Implant – mouse – 25gm/kg.

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

CHRONIC TOXICITY: Not available

RTECS: No data

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available.

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN3208

Metallic substance, water-reactive, n.o.s.

(Gadolinium)

CLASS 4.3

PG III

Marine Pollutant: No

SECTION 15  REGULATORY INFORMATION

TSCA: Listed in the TSCA inventory

SARA 302/304: Not Listed

SARA 311/312: Not Listed

SARA (TITLE 313): Reactivity Hazard

GADOLINIUM CHIPS                                                                    PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION (Cont.)

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: Gadolinium

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: Gadolinium

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 01/15

SDS DEPT.

GADOLINIUM CHIPS                                                                    PAGE 4