Dysprosium (III) Nitrate, Pentahydrate 1651 SDS

Publisher: ProChem, Inc.

Source: https://resds.com/document/dysprosium-iii-nitrate-pentahydrate-prochem-en-5d1294b402

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SECTION 1     PRODUCT AND COMPANY INFORMATION

PRODUCT NAME:    Dysprosium (III) Nitrate, Pentahydrate

PRODUCT NUMBER:  1651

CAS NUMBER:        10031-49-9

MANUFACTURER:

Prochem, Inc.                                         PHONE:        815-398-1788

826 Roosevelt Road                                          FAX:           815-398-1810

Rockford, IL  61109                                     TOLL FREE:    800-795-8788

IN CASE OF TRANSPORTATION EMERGENCY CONTACT CHEM-TREC: 1-800-424-9300

SECTION 2    HAZARDS IDENTIFICATION

CLASSIFICATION OF SUBSTANCE OR MIXTURE

Pictogram

Signal Word         Danger

Hazard Statements

H272             May intensify fire; oxidizer.

H315               Causes skin irritation.

H319               Causes serious eye irritation.

H335             May cause respiratory irritation.

Precautionary Phrases

P210              Keep away from heat/sparks/open flames/hot surfaces.

P220                 Keep/store away from clothing/combustible material.

P221               Take any precaution to avoid mixing with combustibles.

P305+P351+P338      IF IN EYES: Rinse cautiously with waster for several minutes. Remove contact lenses if present and easy to

do. Continue rinsing.

P370+P378             In case of fire: Use carbon dioxide, extinguishing powder, or water spray for extinction.

HMIS CLASSIFICATION:

Health: 1              Fire: 0                  Reactivity Hazard: 2

NFPA RATING:

Health: 1            Flammability: 0         Reactivity Hazard: 2

EYE CONTACT: Causes serious eye irritation

SKIN CONTACT: May be harmful if absorbed through the skin. Causes skin irritation

INHALATION: May be harmful if inhaled. Causes respiratory tract irritation.

INGESTION: May be harmful if swallowed

DYSPROSIUM (III) NITRATE                                                               PAGE 1

Page 2

SECTION 3      COMPOSITION / INFORMATION ON INGREDIENTS

Formula: Dy(NO3)3  5H2O

Molecular Weight: 438.59

CHEMICAL NAME                                           CAS#                               %

Dysprosium (III) Nitrate                                                   10031-49-9                                             100

SECTION 4   FIRST AID MEASURES

EYE EXPOSURE: Immediately flush the eyes with copious amounts of water for several minutes. Assure adequate flushing under

eyelids. Consult a physician.

SKIN EXPOSURE: Wash affected area with soap and water. Remove contaminated clothes if necessary. Seek medical

assistance if irritation or redness persists.

INHALATION: Remove to fresh air and keep at rest.  If breathing is difficult, give oxygen.  If not breathing, give artificial

respiration. Get medical attention.

INGESTION: Seek medical assistance immediately. Keep the victim calm.

SECTION 5   FIREFIGHTING MEASURES

FLASH POINT: Not available

AUTO IGNITION TEMPERATURE: Not available

EXPLOSION LIMITS: Not available

EXTINGUISHING MEDIUM: Use carbon dioxide, extinguishing powder, or water spray. Fight larger fired with water spray or alcohol

resistant foam. Halocarbon extinguishers are not appropriate for this material.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained, approved breathing apparatus and full protective clothing,

including eye protection and boots.

HAZARDOUS COMBUSTION AND DECOMPOSITION PRODUCTS: Nitrogen oxides and dysprosium oxide.

SECTION 6   ACCIDENTAL RELEASE MEASURES

PERSONAL PRECAUTIONS: Wear all appropriate equipment when using this material. Ensure adequate ventilation. Avoid the

formation of dust. Avoid breathing vapors, mist, gas, or dust.

ENVIRONMENTAL PRECAUTIONS: Prevent spillage from entering drains or allowing to be released into the environment.

METHODS AND MATERIALS FOR CONTAINMENT AND CLEANING UP: Absorb spill with inert materials then place in a suitable

container for proper disposal.

SECTION 7   HANDLING AND STORAGE

PRECAUTIONS FOR SAFE HANDLING: Wear appropriate personal protective equipment. Ensure adequate ventilation. Keep

unprotected persons away.

CONDITIONS FOR SAFE STORAGE: Store in cool, dry, and well-ventilated area. Store away from flammable substances, reducing

agents, metal powder, and organic materials.

SECTION 8  EXPOSURE CONTROLS AND PERSONAL PROTECTION

EXPOSURE CONTROLS: Contains no substances with occupational exposure limit values.

EYE PROTECTION: Wear chemical safety glasses or goggles and face shield.

SKIN PROTECTION: Wear nitrile or rubber gloves, and a complete suit protecting against chemicals.

VENTILATION: Provide local exhaust, preferably mechanical.

RESPIRATOR: Use an approved respirator.

ADDITIONAL PROTECTION: Provide eyewash stations, quick-drench showers and washing facilities accessible to areas of use and

handling.

DYSPROSIUM (III) NITRATE                                                               PAGE 2

Page 3

SECTION 9 PHYSICAL AND CHEMICAL PROPERTIES

COLOR AND FORM: Light yellow solid

ODOR: No data available

MOLECULAR WEIGHT: 438.59

BOILING POINT: No data available

MELTING POINT: 88.5° C

SPECIFIC GRAVITY: No data available

VAPOR DENSITY: No data available

SOLUBILITY: Soluble

SECTION 10  STABILITY AND REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur

CONDITIONS TO AVOID: Water and moisture. Hygroscopic, picks up moisture readily from air.

INCOMPATIBILITY: Reducing agents, flammables, organic materials, and metal powders

DECOMPOSITION PRODUCTS: Nitrogen oxides and dysprosium oxide.

SECTION 11 TOXICOLOGICAL DATA

ACUTE TOXICITY: Not available

CARCINOGENIC EFFECTS: No components of this product present at levels greater than or equal to 0.1% is identified a carcinogen.

MUTAGENIC EFFECTS: Not available

TETRATOGENIC EFFECTS: Not available

RTECS: No data

Lanthanons can cause delayed blood clotting leading to hemorrhages. Exposure may also lead to sensitivity heat, itching, increased

awareness of odor and taste, and liver damage.

Small doses of nitrates may cause weakness, general depression, headache and mental impairment. Larger doses may cause

dizziness abdominal cramps, vomiting, bloody diarrhea, convulsions, and collapse.

To the best of our knowledge the toxicological effects of this compound have not been fully investigated.

SECTION 12  ECOLOGICAL DATA

No data available

SECTION 13  DISPOSAL CONSIDERATIONS

Dispose of in according to local, state, and federal regulations.

SECTION 14  TRANSPORTATION DATA

UN1477

Nitrates, inorganic, n.o.s.

(Dysprosium (III) nitrate)

CLASS 5.1

PG III

Marine Pollutant: No

DYSPROSIUM (III) NITRATE                                                               PAGE 3

Page 4

SECTION 15  REGULATORY INFORMATION

SARA (TITLE 313): Not Listed

CALIFORNIA PROP. 65: Not Listed

MASSACHUSETTS RIGHT TO KNOW COMPONENTS: No components are subject to the Massachusetts Right to Know Act.

NEW JERSEY RIGHT TO KNOW COMPONENTS: No components are subject to the New Jersey Right to Know Act.

PENNSYLVANIA RIGHT TO KNOW COMPONENTS: No components are subject to the Pennsylvania Right to Know Act.

SECTION 16  OTHER INFORMATION

DISCLAIMER: The information herein is believed to be accurate and reliable as of the date compiled. However, Prochem, Inc.

makes no representation, warranty, or guarantee of any kind with respect to the information in this document or any use of the

product based on the information.

DATE PREPARED: 12/14

DYSPROSIUM (III) NITRATE                                                               PAGE 4