Gadolinium - pieces, chips, ingot, rod, foil (≥0.005"), sheet, target SDS

Publisher: ESPI

Source: https://resds.com/document/gadolinium-pieces-chips-ingot-rod-foil-0-005-sheet-target-sds-en-fee523eb3e

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Gadolinium | Gadolinium

Gadolinium

SAFETY DATA SHEET

1 PRODUCT AND SUPPLIER IDENTIFICATION

Product Name:  Gadolinium - pieces, chips, ingot, rod, foil (≥0.005"), sheet, target

Formula:       Gd

Supplier:        ESPI Metals

1050 Benson Way

Ashland, OR 97520

Telephone:      800-638-2581

Fax:            541-488-8313

Email:           sales@espimetals.com

Emergency:      Infotrac 800-535-5053 (US) or 352-323-3500 (24 hour)

Recommended Uses:  Scientific Research

2 HAZARDS IDENTIFICATION

GHS Classification (29 CFR 1910.1200):  Not classified as hazardous

GHS Label Elements:

Signal Word: N/A

Hazard Statements: N/A

Precautionary Statements:  N/A

3 COMPOSITION/INFORMATION ON INGREDIENTS

Ingredient:      Gadolinium

CAS#:           7440-54-2

%:             100

EC#:            231-162-2

4  FIRST AID MEASURES

http://www.espimetals.com/index.php/msds/131-gadolinium?tmpl=component&print=1&page=[1/29/2016 8:37:31 AM]

Page 2

Gadolinium | Gadolinium

General Measures: No special requirements.

INHALATION: Remove to fresh air, keep warm and quiet, give oxygen if breathing is difficult. Seek medical attention.

INGESTION: Rinse mouth with water. Do not induce vomiting. Seek medical attention. Never induce vomiting or give anything by

mouth to an unconscious person.

SKIN: Remove contaminated clothing, brush material off skin, wash affected area with soap and water. Seek medical attention if

symptoms persist.

EYES: Flush eyes with lukewarm water, including under upper and lower eyelids, for at least 15 minutes. Seek medical attention if

symptoms persist.

Most Important Symptoms/Effects, Acute and Delayed: May cause irritation. See section 11 for more information.

Indication of Immediate Medical Attention and Special Treatment: No other relevant information available.

5 FIREFIGHTING MEASURES

Extinguishing Media: Use Class D dry powder extinguishing agent.

Unsuitable Extinguishing Media: Do not use water.

Specific Hazards Arising from the Material: Flammable in the form of dust or very thin foil when exposed to heat, spark or flame.

May react with water under fire conditions liberating flammable hydrogen gas. May emit fumes of gadolinium oxide under fire

conditions.

Special Protective Equipment and Precautions for Firefighters: Full face, self-contained breathing apparatus and full protective

clothing to prevent contact with skin and eyes.

6 ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment, and Emergency Procedures: Wear appropriate respiratory and protective

equipment specified in section 8. Isolate spill area and provide ventilation. Avoid breathing dust or fume. Avoid contact with skin and

eyes. Eliminate all sources of ignition.

Methods and Materials for Containment and Cleaning Up: Avoid dust formation. Sweep or scoop spilled product and place in a

closed container for further handling and disposal. Use only non-sparking tools and natural bristle brushes.

Environmental Precautions: Do not allow to enter drains or to be released to the environment.

7 HANDLING AND STORAGE

Precautions for Safe Handling: Handle in an enclosed, controlled process. Handle thin foils or small chips under dry protective gas

such as argon. Use non-sparking tools. Protect from sources of ignition. Avoid creating dust. Avoid breathing dust or fumes. Provide

adequate ventilation if dusts are created. Avoid contact with skin and eyes. Wash thoroughly before eating or smoking. See section 8

for information on personal protection equipment.

Conditions for Safe Storage, Including Any Incompatibilities: Store in tightly-sealed containers. Store thin foils or small chips

under argon. Store in a cool, dry area. Protect from moisture and air. See section 10 for more information on incompatible materials.

8 EXPOSURE CONTROLS AND PERSONAL PROTECTION

http://www.espimetals.com/index.php/msds/131-gadolinium?tmpl=component&print=1&page=[1/29/2016 8:37:31 AM]

Page 3

Gadolinium | Gadolinium

Exposure Limits:   Gadolinium

OSHA/PEL:        No exposure limit established

ACGIH/TLV:       No exposure limit established

Appropriate Engineering Controls: Ensure adequate ventilation to maintain exposures below occupational limits. Whenever possible

the use of local exhaust ventilation or other engineering controls is the preferred method of controlling exposure to airborne dust and

fume to meet established occupational exposure limits. Use good housekeeping and sanitation practices. Do not use tobacco or food in

work area. Wash thoroughly before eating or smoking. Do not blow dust off clothing or skin with compressed air.

Individual Protection Measures, Such as Personal Protective Equipment:

Respiratory Protection: If permissible levels are exceeded, use NIOSH approved dust respirator.

Eye Protection: Safety glasses

Skin Protection: Wear impermeable gloves, protective work clothing as necessary.

9 PHYSICAL AND CHEMICAL PROPERTIES

Appearance:

Form:              Solid in various forms

Color:              Silver gray metallic

Odor:            Not determined

Odor Threshold:  Not determined

pH:                          N/A

Melting Point:                1313 oC

Boiling Point:                 3273 oC

Flash Point:                  N/A

Evaporation Rate:            N/A

Flammability:               No data

Upper Flammable Limit:       No data

Lower Flammable Limit:       No data

Vapor Pressure:             No data

Vapor Density:                N/A

Relative Density (Specific Gravity):   7.901 g/cc

Solubility in H2O:                 Insoluble

Partition Coefficient (n-octanol/water):  Not determined

Autoignition Temperature:     No data

Decomposition Temperature:   No data

Viscosity:                     N/A

10  STABILITY AND REACTIVITY

http://www.espimetals.com/index.php/msds/131-gadolinium?tmpl=component&print=1&page=[1/29/2016 8:37:31 AM]

Page 4

Gadolinium | Gadolinium

Reactivity: No data

Chemical Stability: Stable under recommended storage conditions.

Possibility of Hazardous Reactions: Dusts are flammable. Contact with acids may evolve hydrogen gas. May react with water under

fire conditions liberating flammable hydrogen gas.

Conditions to Avoid: Avoid creating or accumulating fines or dusts, especially near possible ignition sources such as sparks or flame.

Incompatible Materials: Acids, acid chlorides, oxidizing agents, halogens, water/moisture, air.

Hazardous Decomposition Products: Gadolinium oxide, gadolinium hydroxides, hydrogen.

11 TOXICOLOGICAL INFORMATION

Likely Routes of Exposure: Inhalation, skin, eyes.

Symptoms of Exposure: May cause irritation.

Acute and Chronic Effects: No data

Acute Toxicity: No data

Carcinogenicity: NTP: Not identified as carcinogenic  IARC: Not identified as carcinogenic

To the best of our knowledge the chemical, physical and toxicological characteristics of the substance are not fully known.

12 ECOLOGICAL INFORMATION

Ecotoxicity: No data

Persistence and Degradability: No data

Bioaccumulative Potential: No data

Mobility in Soil: No data

Other Adverse Effects: Do not allow material to be released to the environment. No further relevant information available.

13 DISPOSAL CONSIDERATIONS

Waste Disposal Method:

Product: Dispose of in accordance with Federal, State and Local regulations.

Packaging: Dispose of in accordance with Federal, State and Local regulations.

14 TRANSPORT INFORMATION

Shipping Regulations: Not regulated

UN Number:               N/A

UN Proper Shipping Name:  N/A

Transport Hazard Class:    N/A

Packing Group:            N/A

http://www.espimetals.com/index.php/msds/131-gadolinium?tmpl=component&print=1&page=[1/29/2016 8:37:31 AM]

Page 5

Gadolinium | Gadolinium

Marine Pollutant:          No

15 REGULATORY INFORMATION

TSCA Listed: All components are listed.

Regulation (EC) No 1272/2008 (CLP): N/A

Canada WHMIS Classification (CPR, SOR/88-66): N/A

HMIS Ratings: Health: 1   Flammability: 1   Physical: 1

NFPA Ratings: Health: 1   Flammability: 1   Instability: 1

Chemical Safety Assessment: A chemical safety assessment has not been carried out.

16 OTHER INFORMATION

The information contained in this document is based on the state of our knowledge at the time of publication and is believed to be

correct, but does not purport to be all inclusive and shall be used only as a guide. ESPI Metals makes no representation, warranty, or

guarantee of any kind with respect to the information contained in this document or any use of the product based on this information.

ESPI Metals shall not be held liable for any damages resulting from handling or from contact with the above product. Users should

satisfy themselves that they have all current data relevant to their particular use.

Prepared by:        ESPI Metals

Revised/Reviewed:  July 2015

http://www.espimetals.com/index.php/msds/131-gadolinium?tmpl=component&print=1&page=[1/29/2016 8:37:31 AM]