Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml ALFAA14373 SDS

Publisher: Avocado Research Chemicals Ltd. (Part of Thermo Fisher Scientific)

Source: https://resds.com/document/gadolinium-plasma-standard-solution-specpure-gd-10-000%CE%BCg-ml-sds-avocado-research-chemicals-ltd-part-of-thermo-en-7c0eaeb81d

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Page 1 / 9             SAFETY DATA SHEET

Revision Date 04-Aug-2026

Version 7

ALFAA14373

Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

SECTION 1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

产品说明:产品说明:产品说明:产品说明:             钆,等离子标准溶液,钆,等离子标准溶液,钆,等离子标准溶液,钆,等离子标准溶液,Specpure®,Specpure®,Specpure®,Specpure®,GdGdGdGd10,000µg/ml10,000µg/ml10,000µg/ml10,000µg/ml

Product Description:              Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

Cat No. :                        14373

Molecular Formula              Gd2 O3 in 5% HN O3

Supplier                         Avocado Research Chemicals Ltd.

(Part of Thermo Fisher Scientific)

Shore Road, Heysham

Lancashire, LA3 2XY,

United Kingdom

Office Tel: +44 (0) 1524 850506

Office Fax: +44 (0) 1524 850608

Emergency Telephone Number     For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

E-mail address                    begel.sdsdesk@thermofisher.com

Recommended Use                 Laboratory chemicals.

Uses advised against           No Information available

SECTION 2. HAZARD IDENTIFICATION

Physical State                      Appearance                          Odor

Liquid                                   Clear                    No information available

Emergency Overview

Causes severe skin burns and eye damage. May be corrosive to metals. Harmful if inhaled.

Classification of the substance or mixture

Substances/mixtures corrosive to metal                                              Category 1

Acute Inhalation Toxicity - Vapors                                                   Category 4

Skin Corrosion/Irritation                                                           Category 1 A

Serious Eye Damage/Eye Irritation                                                  Category 1

Label Elements

Signal Word                    Danger

Hazard Statements

______________________________________________________________________________________________

Page 1

Page 2

ALFAA14373               SAFETY DATA SHEET                    Revision Date 04-Aug-2026Page 2 / 9

Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

______________________________________________________________________________________________

H290 - May be corrosive to metals

H314 - Causes severe skin burns and eye damage

H332 - Harmful if inhaled

Precautionary Statements

Prevention

P234 - Keep only in original packaging

P264 - Wash face, hands and any exposed skin thoroughly after handling

P271 - Use only outdoors or in a well-ventilated area

P280 - Wear protective gloves/protective clothing/eye protection/face protection

Response

P301 + P330 + P331 - IF SWALLOWED: rinse mouth. Do NOT induce vomiting

P303 + P361 + P353 - IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower

P304 + P340 - IF INHALED: Remove person to fresh air and keep comfortable for breathing

P305 + P351 + P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing

P310 - Immediately call a POISON CENTER or doctor

P363 - Wash contaminated clothing before reuse

P390 - Absorb spillage to prevent material damage

Storage

P402 - Store in a dry place

P403 + P233 - Store in a well-ventilated place. Keep container tightly closed

P406 - Store in corrosion resistant polypropylene container with a resistant inliner

Disposal

P501 - Dispose of contents/ container to an approved waste disposal plant

Physical and Chemical Hazards

May be corrosive to metals.

Health Hazards

Corrosive. Causes skin and eye burns. Causes serious eye damage. Harmful if inhaled.

Environmental hazards

Contains no substances known to be hazardous to the environment or not degradable in waste water treatment plants. Will likely

be mobile in the environment due to its water solubility. The product is water soluble, and may spread in water systems.

No information available

This product does not contain any known or suspected endocrine disruptors. Toxic to terrestrial vertebrates.

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

Component                              CAS No                Weight %

Water                                            7732-18-5                  94

Nitricacid…% [C≤70%]                                  7697-37-2                   5

Gadolinium oxide (Gd2O3)                                  12064-62-9                  1

SECTION 4. FIRST AID MEASURES

General Advice

Show this safety data sheet to the doctor in attendance. Immediate medical attention is required.

Eye Contact

Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Immediate medical attention is required.

Skin Contact

Wash off immediately with plenty of water for at least 15 minutes. Remove and wash contaminated clothing and gloves, including

the inside, before re-use. Call a physician immediately.

Inhalation

If not breathing, give artificial respiration. Remove from exposure, lie down. Do not use mouth-to-mouth method if victim ingested or

inhaled the substance; give artificial respiration with the aid of a pocket mask equipped with a one-way valve or other proper

respiratory medical device. Call a physician immediately.

______________________________________________________________________________________________

Page 2

Page 3

ALFAA14373               SAFETY DATA SHEET                    Revision Date 04-Aug-2026Page 3 / 9

Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

______________________________________________________________________________________________

Ingestion

Do NOT induce vomiting. Clean mouth with water. Never give anything by mouth to an unconscious person. Call a physician

immediately.

Most important symptoms and effects

Causes burns by all exposure routes. Product is a corrosive material. Use of gastric lavage or emesis is contraindicated. Possible

perforation of stomach or esophagus should be investigated: Ingestion causes severe swelling, severe damage to the delicate

tissue and danger of perforation

Self-Protection of the First Aider

Ensure that medical personnel are aware of the material(s) involved, take precautions to protect themselves and prevent spread of

contamination.

Notes to Physician

Treat symptomatically.

SECTION 5. FIRE-FIGHTING MEASURES

Suitable Extinguishing Media

Not combustible. CO 2, dry chemical, dry sand, alcohol-resistant foam.

Extinguishing media which must not be used for safety reasons

No information available.

Specific Hazards Arising from the Chemical

Thermal decomposition can lead to release of irritating gases and vapors. The product causes burns of eyes, skin and mucous

membranes.

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear. Thermal decomposition can lead to release of irritating gases and vapors.

SECTION 6. ACCIDENTAL RELEASE MEASURES

Personal Precautions

Ensure adequate ventilation. Use personal protective equipment as required. Evacuate personnel to safe areas. Keep people away

from and upwind of spill/leak.

Environmental Precautions

Should not be released into the environment. Do not allow material to contaminate ground water system. Do not flush into surface

water or sanitary sewer system.

Methods for Containment and Clean Up

Soak up with inert absorbent material. Keep in suitable, closed containers for disposal.

Refer to protective measures listed in Sections 8 and 13.

SECTION 7. HANDLING AND STORAGE

Handling

Wear personal protective equipment/face protection. Do not get in eyes, on skin, or on clothing. Use only under a chemical fume

hood. Do not breathe mist/vapors/spray. Do not ingest. If swallowed then seek immediate medical assistance.

Storage

Corrosives area. Keep containers tightly closed in a dry, cool and well-ventilated place.

Specific Use(s)

______________________________________________________________________________________________

Page 3

Page 4

ALFAA14373               SAFETY DATA SHEET                    Revision Date 04-Aug-2026Page 4 / 9

Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

______________________________________________________________________________________________

Use in laboratories

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Control Parameters

Component                China              ACGIH TLV           OSHA PEL              NIOSH

Nitricacid…% [C≤70%]                   -                TWA: 2 ppm           (Vacated) TWA: 2 ppm          IDLH: 25 ppm

STEL: 4 ppm          (Vacated) TWA: 5 mg/m3     REL = 2 ppm (TWA)

(Vacated) STEL: 4 ppm    REL = 5 mg/m3 (TWA)

(Vacated) STEL: 10 mg/m3        STEL: 4 ppm

TWA: 2 ppm            STEL: 10 mg/m3

TWA: 5 mg/m3

Component            Hong Kong                Thailand            European Union       The United Kingdom

Nitricacid…% [C≤70%]      TWA: 2 ppm            TWA: 2 ppm          STEL: 1 ppm (15min)       STEL: 1 ppm 15 min

TWA: 5.2 mg/m3                              STEL: 2.6 mg/m3 (15min)    STEL: 2.6 mg/m3 15 min

STEL: 4 ppm

STEL: 10 mg/m3

Legend

ACGIH - American Conference of Governmental Industrial Hygienists

OSHA - Occupational Safety and Health Administration

NIOSH: NIOSH - National Institute for Occupational Safety and Health

Monitoring methods

MDHS 91 Metals and metalloids in workplace air by X-ray fluorescence spectrometry MDHS 99 Metals in air by ICP-AES

Exposure Controls

Engineering Measures

Ensure that eyewash stations and safety showers are close to the workstation location. Wherever possible, engineering control

measures such as the isolation or enclosure of the process, the introduction of process or equipment changes to minimise release

or contact, and the use of properly designed ventilation systems, should be adopted to control hazardous materials at source.

Personal protective equipment

Eye Protection                Goggles (European standard - EN 166)

Hand Protection                  Protective gloves

Glove material     Breakthrough time  Glove thickness   EU standard             Glove comments

Neoprene       See manufacturers               -          EN 374             (minimum requirement)

recommendations

Inspect gloves before use.

Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves.

(Refer to manufacturer/supplier for information)

Ensure gloves are suitable for the task: Chemical compatability, Dexterity, Operational conditions, User susceptibility, e.g.

sensitisation effects, also take into consideration the specific local conditions under which the product is used, such as the danger

of cuts, abrasion.

Remove gloves with care avoiding skin contamination.

Skin and body protection       Long sleeved clothing

Respiratory Protection       When workers are facing concentrations above the exposure limit they must use

appropriate certified respirators.

To protect the wearer, respiratory protective equipment must be the correct fit and be used

and maintained properly

Large scale/emergency use      In case of insufficient ventilation, wear suitable respiratory equipment

Recommended Filter type: Acid gases filter

Small scale/Laboratory use     Use a NIOSH/MSHA or European Standard EN 149:2001 approved respirator if exposure

______________________________________________________________________________________________

Page 4

Page 5

ALFAA14373               SAFETY DATA SHEET                    Revision Date 04-Aug-2026Page 5 / 9

Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

______________________________________________________________________________________________

limits are exceeded or if irritation or other symptoms are experienced.

When RPE is used a face piece Fit Test should be conducted

Hygiene Measures                Handle in accordance with good industrial hygiene and safety practice.

Environmental exposure controls  No information available.

SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

Appearance                        Clear

Physical State                        Liquid

Odor                       No information available

Odor Threshold               No data available

pH                         No information available

Melting Point/Range            No data available

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                   No information available           Method - No information available

Evaporation Rate              No data available

Flammability (solid,gas)            Not applicable                         Liquid

Explosion Limits               No data available

Vapor Pressure                  23 hPa @ 20 °C

Vapor Density                 No data available                         (Air = 1.0)

Specific Gravity  / Density        No data available

Bulk Density                      Not applicable                         Liquid

Water Solubility                      Miscible

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Component                        log Pow

Nitricacid…% [C≤70%]            -2.3

Autoignition Temperature        No data available

Decomposition Temperature      No data available

Viscosity                    No data available

Explosive Properties            No information available

Oxidizing Properties            No information available

Molecular Formula              Gd2 O3 in 5% HN O3

SECTION 10. STABILITY AND REACTIVITY

Stability                             Stable under normal conditions.

Hazardous Reactions            None under normal processing.

Hazardous Polymerization        No information available.

Conditions to Avoid             None known.

Materials to avoid                  Strong bases.

Hazardous Decomposition Products Nitrogen oxides (NOx). Metal oxides.

SECTION 11. TOXICOLOGICAL INFORMATION

Product Information

______________________________________________________________________________________________

Page 5

Page 6

ALFAA14373               SAFETY DATA SHEET                    Revision Date 04-Aug-2026Page 6 / 9

Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

______________________________________________________________________________________________

(a) acute toxicity;

Toxicology data for the components

Component                    LD50 Oral                LD50 Dermal             LC50 Inhalation

Water                                               -                                            -                                            -

Nitricacid…% [C ≤70%]                                                          LC50 = 2500 ppm. (Rat) 1h

Gadolinium oxide (Gd2O3)           LD50 > 5 g/kg  ( Rat )                               LC50 > 5.04 mg/L  ( Rat ) 4 h

(b) skin corrosion/irritation;        Category 1 B

(c) serious eye damage/irritation;   Category 1

(d) respiratory or skin sensitization;

Respiratory                No data available

Skin                     No data available

(e) germ cell mutagenicity;       No data available

(f) carcinogenicity;             No data available

There are no known carcinogenic chemicals in this product

(g) reproductive toxicity;         No data available

(h) STOT-single exposure;        No data available

(i) STOT-repeated exposure;      No data available

Target Organs               None known.

(j) aspiration hazard;            No data available

Symptoms  / effects,both acute and Product is a corrosive material. Use of gastric lavage or emesis is contraindicated.

delayed                             Possible perforation of stomach or esophagus should be investigated: Ingestion causes

severe swelling, severe damage to the delicate tissue and danger of perforation

SECTION 12. ECOLOGICAL INFORMATION

Ecotoxicity effects              May cause long-term adverse effects in the environment. Do not allow material to

contaminate ground water system.

Persistence and Degradability      Product contains heavy metals. Discharge into the environment must be avoided. Special

pre-treatment is necessary

Persistence                  based on information available, May persist.

Degradation in sewage          Contains substances known to be hazardous to the environment or not degradable in waste

treatment plant                 water treatment plants.

Bioaccumulative Potential        May have some potential to bioaccumulate

Component                                 log Pow                           Bioconcentration factor (BCF)

Nitricacid…% [C ≤70%]                                -2.3                            No data available

______________________________________________________________________________________________

Page 6

Page 7

ALFAA14373               SAFETY DATA SHEET                    Revision Date 04-Aug-2026Page 7 / 9

Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

______________________________________________________________________________________________

Mobility in soil                  The product is water soluble, and may spread in water systems Will likely be mobile in the

environment due to its water solubility Highly mobile in soils

Endocrine Disruptor Information    This product does not contain any known or suspected endocrine disruptors

Persistent Organic Pollutant        This product does not contain any known or suspected substance

Ozone Depletion Potential           This product does not contain any known or suspected substance

SECTION 13. DISPOSAL CONSIDERATIONS

Waste from Residues/Unused      Waste is classified as hazardous. Dispose of in accordance with the European Directives

Products                        on waste and hazardous waste. Dispose of in accordance with local regulations.

Contaminated Packaging           Dispose of this container to hazardous or special waste collection point.

Other Information                Waste codes should be assigned by the user based on the application for which the product

was used. Do not empty into drains. Do not flush to sewer. Large amounts will affect pH

and harm aquatic organisms.

SECTION 14. TRANSPORT INFORMATION

Road and Rail Transport

UN-No                       UN2031

Proper Shipping Name            NITRIC ACID

Hazard Class                    8

Packing Group                                     II

IMDG/IMO

UN-No                       UN2031

Proper Shipping Name            NITRIC ACID

Hazard Class                    8

Packing Group                                     II

IATA

UN-No                       UN2031

Proper Shipping Name            NITRIC ACID

Hazard Class                    8

Packing Group                                     II

Special Precautions for User      No special precautions required

SECTION 15. REGULATORY INFORMATION

International Inventories

China, X = listed, Australia, U.S.A. (TSCA), Canada (DSL/NDSL), Europe (EINECS/ELINCS/NLP), Australia (AICS), Korea (KECL), China (IECSC),

Japan (ENCS), Philippines (PICCS), Taiwan (TCSI), Japan (ISHL), New Zealand (NZIoC), Japan (ISHL).

Component        The        List of    IECSC   EINECS   TSCA   DSL   PICCS  ENCS    ISHL    AICS    KECL

Inventory of dangerous

Hazardous  goods GB

Chemicals   12268 -

(2015      2012

Edition)

Water                    -                 -        X      231-791-2    X      X      X      X             X     KE-35400

______________________________________________________________________________________________

Page 7

Page 8

ALFAA14373               SAFETY DATA SHEET                    Revision Date 04-Aug-2026Page 8 / 9

Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

______________________________________________________________________________________________

Nitricacid…% [C≤70    X        X       X      231-714-2    X      X      X      X      X      X     KE-25911

%]

Gadolinium oxide            -                 -        X      235-060-9    X      X      X      X      X      X     KE-10558

(Gd2O3)

National Regulations

SECTION 16. OTHER INFORMATION

Prepared By                         Health, Safety and Environmental Department

Revision Date                     04-Aug-2026

Revision Summary            SDS sections updated.

Training Advice

Chemical hazard awareness training, incorporating labelling, Safety Data Sheets (SDS), Personal Protective Equipment (PPE) and

hygiene.

Legend

CAS - Chemical Abstracts Service                          TSCA - United States Toxic Substances Control Act Section 8(b)

Inventory

EINECS/ELINCS - European Inventory of Existing Commercial Chemical DSL/NDSL - Canadian Domestic Substances List/Non-Domestic

Substances/EU List of Notified Chemical Substances                  Substances List

PICCS - Philippines Inventory of Chemicals and Chemical Substances  ENCS - Japanese Existing and New Chemical Substances

IECSC - Chinese Inventory of Existing Chemical Substances          AICS - Australian Inventory of Chemical Substances

KECL - Korean Existing and Evaluated Chemical Substances         NZIoC - New Zealand Inventory of Chemicals

WEL - Workplace Exposure Limit                        TWA - Time Weighted Average

ACGIH - American Conference of Governmental Industrial Hygienists   IARC - International Agency for Research on Cancer

DNEL - Derived No Effect Level                           PNEC - Predicted No Effect Concentration

RPE - Respiratory Protective Equipment                        LD50 - Lethal Dose 50%

LC50 - Lethal Concentration 50%                            EC50 - Effective Concentration 50%

NOEC - No Observed Effect Concentration                 POW - Partition coefficient Octanol:Water

PBT - Persistent, Bioaccumulative, Toxic                       vPvB - very Persistent, very Bioaccumulative

ICAO/IATA - International Civil Aviation Organization/International Air   IMO/IMDG - International Maritime Organization/International Maritime

Transport Association                                          Dangerous Goods Code

ADR - European Agreement Concerning the International Carriage of   MARPOL - International Convention for the Prevention of Pollution from

Dangerous Goods by Road                                         Ships

OECD - Organisation for Economic Co-operation and Development    ATE - Acute Toxicity Estimate

BCF - Bioconcentration factor                           VOC - (Volatile Organic Compound)

Key literature references and sources for data

https://echa.europa.eu/information-on-chemicals

Suppliers safety data sheet, Chemadvisor - LOLI, Merck index, RTECS

Physical hazards              On basis of test data

Health Hazards                       Calculation method

Environmental hazards              Calculation method

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

______________________________________________________________________________________________

Page 8

Page 9

ALFAA14373               SAFETY DATA SHEET                    Revision Date 04-Aug-2026Page 9 / 9

Gadolinium, plasma standard solution, Specpure®, Gd 10,000µg/ml

______________________________________________________________________________________________

End of Safety Data Sheet

______________________________________________________________________________________________

Page 9