Gadolinium Standard: 1000 μg/mL Gd in 5% HNO3 [500ml bottle] SDS

Publisher: Agilent

Source: https://resds.com/document/gadolinium-standard-1000-%CE%BCg-ml-gd-in-5-hno3-500ml-bottle-sds-en-5af516e81c

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

Revision date  25-Jun-2024                                                                Revision Number 1

1. Identification

Product identifier

Product Name                     Gadolinium Standard: 1000 μg/mL Gd in 5% HNO3 [500ml bottle]

Other means of identification

Product Code(s)                  5190-8456

Synonyms                     None

Recommended use of the chemical and restrictions on use

Recommended use               Reagents and Standards for Analytical Chemical Laboratory Use

Restrictions on use                Not to be used for human or animal consumption

Details of the supplier of the safety data sheet

Supplier Address

Agilent Technologies, Inc.

5301 Stevens Creek Blvd

Santa Clara, CA 95051, USA

800-227-9770

E-mail                             pdl-msds_author@agilent.com

Emergency telephone number

Emergency Telephone

CHEMTREC®: 1-800-424-9300

2. Hazard(s) identification

Classification

Classified according to OSHA.

Skin corrosion/irritation                                                            Category 2

Serious eye damage/eye irritation                                                   Category 1

Corrosive to metals                                                               Category 1

Hazards not otherwise classified (HNOC)

AGHS   / EN                                                                             Page   1 / 13

Page 2

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

Not applicable

Label elements

Danger

Hazard statements

Classified according to OSHA.

Causes skin irritation

Causes serious eye damage

May be corrosive to metals

Precautionary Statements - Prevention

Wash face, hands and any exposed skin thoroughly after handling

Keep only in original packaging

Wear protective gloves/eye protection/face protection

Precautionary Statements - Response

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing

Immediately call a POISON CENTER or doctor

IF ON SKIN: Wash with plenty of water and soap

If skin irritation occurs: Get medical advice/attention

Take off contaminated clothing and wash it before reuse

Absorb spillage to prevent material damage

Precautionary Statements - Storage

Store in corrosion resistant container with a resistant inner liner

Other information

No information available.

3. Composition/information on ingredients

Substance

Not applicable.

AGHS   / EN                                                                             Page   2 / 13

Page 3

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

Mixture

Chemical nature                  aqueous solution.

Chemical name                 CAS No.              Weight-%             Trade secret

Nitric Acid                         7697-37-2              3 - <5                          *

Additional information

The concentration of the acid stated in this SDS is calculated as an absolute mass concentration (%w/v). This is less than the acid

concentration stated on the product label and COA, which reflects a percent value of the commercially available concentrated

aqueous form of the acid.

4. First-aid measures

Description of first aid measures

General advice                Show this safety data sheet to the doctor in attendance. Immediate medical attention is

required.

Inhalation                    Remove to fresh air. Get medical attention immediately if symptoms occur.

Eye contact                       Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes.

Remove contact lenses, if present and easy to do. Continue rinsing. Keep eye wide open

while rinsing. Do not rub affected area. Get immediate medical attention.

Skin contact                  Wash off immediately with soap and plenty of water for at least 15 minutes. Get medical

attention if irritation develops and persists.

Ingestion                         Rinse mouth. Never give anything by mouth to an unconscious person. Do NOT induce

vomiting. Call a physician.

Self-protection of the first aider     Avoid contact with skin, eyes or clothing. Wear personal protective clothing (see section 8).

Most important symptoms and effects, both acute and delayed

Symptoms                         Burning sensation.

Indication of any immediate medical attention and special treatment needed

Note to physicians                  Treat symptomatically.

5. Fire-fighting measures

AGHS   / EN                                                                             Page   3 / 13

Page 4

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

Suitable Extinguishing Media      Use extinguishing measures that are appropriate to local circumstances and the

surrounding environment.

Large Fire                  CAUTION: Use of water spray when fighting fire may be inefficient.

Unsuitable extinguishing media    Do not scatter spilled material with high pressure water streams.

Specific hazards arising from the   No information available.

chemical

Explosion data

Sensitivity to mechanical impact None.

Sensitivity to static discharge   None.

Special protective equipment and    Firefighters should wear self-contained breathing apparatus and full firefighting turnout gear.

precautions for fire-fighters       Use personal protection equipment.

6. Accidental release measures

Personal precautions, protective equipment and emergency procedures

Personal precautions              Avoid contact with skin, eyes or clothing. Ensure adequate ventilation. Use personal

protective equipment as required.

Other information                  Refer to protective measures listed in Sections 7 and 8.

Methods and material for containment and cleaning up

Methods for containment           Prevent further leakage or spillage if safe to do so.

Methods for cleaning up            Pick up and transfer to properly labeled containers.

7. Handling and storage

Precautions for safe handling

Advice on safe handling           Handle in accordance with good industrial hygiene and safety practice. Avoid contact with

skin, eyes or clothing. Do not eat, drink or smoke when using this product. Take off

contaminated clothing and wash before reuse.

Conditions for safe storage, including any incompatibilities

AGHS   / EN                                                                             Page   4 / 13

Page 5

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

Storage Conditions                Please refer to the manufacturer's certificate for specific storage and transport temperature

conditions. Store only in the original receptacle unless other advice is given on the CoA.

Keep containers tightly closed in a dry, cool and well-ventilated place. Protect from

moisture. Store locked up. Keep out of the reach of children. Store away from other

materials.

8. Exposure controls/personal protection

Control parameters

Exposure Limits

Chemical name             ACGIH TLV             OSHA PEL                NIOSH

Nitric Acid               TWA: 2 ppm              TWA: 2 ppm                 IDLH: 25 ppm

7697-37-2                STEL: 4 ppm             TWA: 5 mg/m3              TWA: 2 ppm

(vacated) TWA: 2 ppm          TWA: 5 mg/m3

(vacated) TWA: 5 mg/m3           STEL: 4 ppm

(vacated) STEL: 4 ppm          STEL: 10 mg/m3

(vacated) STEL: 10 mg/m3

Biological occupational exposure limits

This product, as supplied, does not contain any hazardous materials with biological limits established by the region specific

regulatory bodies.

Appropriate engineering controls

Engineering controls             Showers

Eyewash stations

Ventilation systems.

Individual protection measures, such as personal protective equipment

Eye/face protection                Avoid contact with eyes. Wear safety glasses with side shields (or goggles). Tight sealing

safety goggles.

Hand protection                Wear protective Neoprene™ gloves. The protective gloves to be used must comply with the

specifications of EC Directive 89/686/EEC and the related standard EN374. Wear suitable

gloves. Impervious gloves.

Skin and body protection         Wear suitable protective clothing. Long sleeved clothing.

Respiratory protection          No protective equipment is needed under normal use conditions. If exposure limits are

exceeded or irritation is experienced, ventilation and evacuation may be required.

AGHS   / EN                                                                             Page   5 / 13

Page 6

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

Environmental exposure controls  Do not allow into any sewer, on the ground or into any body of water.

General hygiene considerations    Avoid contact with skin, eyes or clothing. Do not eat, drink or smoke when using this

product. Wash hands before breaks and after work. Wear suitable gloves and eye/face

protection. Regular cleaning of equipment, work area and clothing is recommended.

9. Physical and chemical properties

Information on basic physical and chemical properties

Physical state                        Liquid

Appearance                          Liquid

Color                                  colorless

Odor                              Odorless

Odor threshold                No information available

Property                         Values                        Remarks  • Method

pH                         No data available                None known

Melting point / freezing point      No data available                None known

Initial boiling point and boiling rangeNo data available                None known

Flash point                   No data available                None known

Evaporation rate               No data available                None known

Flammability                  No data available                None known

Flammability Limit in Air                                       None known

Upper flammability or explosive  No data available

limits

Lower flammability or explosive  No data available

limits

Vapor pressure                No data available                None known

Relative vapor density           No data available                None known

Relative density               No data available                None known

Water solubility                No data available                None known

Solubility(ies)                 No data available                None known

Partition coefficient             No data available                None known

Autoignition temperature         No data available                None known

Decomposition temperature                                    None known

Kinematic viscosity             No data available                None known

Dynamic viscosity              No data available                None known

Other information

Explosive properties            No information available

Oxidizing properties            No information available

Softening point                No information available

Molecular weight              No information available

VOC content                  No information available

Liquid Density                No information available

AGHS   / EN                                                                             Page   6 / 13

Page 7

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

Bulk density                  No information available

10. Stability and reactivity

Reactivity                    No information available.

Chemical stability                   Stable under normal conditions.

Possibility of hazardous reactions  None under normal processing.

Conditions to avoid               Exposure to air or moisture over prolonged periods.

Incompatible materials              Oxidizing agent. Strong acids. Strong bases.

Hazardous decomposition products None known based on information supplied.

11. Toxicological information

Information on likely routes of exposure

Product Information

Inhalation                         Specific test data for the substance or mixture is not available.

Eye contact                       Specific test data for the substance or mixture is not available. Causes serious eye damage.

May cause irreversible damage to eyes.

Skin contact                      Specific test data for the substance or mixture is not available. Causes skin irritation. (based

on components).

Ingestion                          Specific test data for the substance or mixture is not available. Ingestion may cause

gastrointestinal irritation, nausea, vomiting and diarrhea.

Symptoms related to the physical, chemical and toxicological characteristics

Symptoms                       Redness. Burning. May cause blindness. May cause redness and tearing of the eyes.

Acute toxicity

Numerical measures of toxicity

The following values are calculated based on chapter 3.1 of the GHS document

ATEmix (oral)                   99,999.00 mg/kg

ATEmix (dermal)                99,999.00 mg/kg

AGHS   / EN                                                                             Page   7 / 13

Page 8

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

ATEmix (inhalation-gas)         99,999.00 ppm

ATEmix (inhalation-dust/mist)   99,999.00  mg/l

ATEmix (inhalation-vapor)      66.70  mg/l

Component Information

Chemical name                   Oral LD50                 Dermal LD50                   Inhalation LC50

Nitric Acid                                    -                                           -                = 2500 ppm  ( Rat ) 1 h

7697-37-2                                                      ATE (vapours) = 2.65 mg/L

Delayed and immediate effects as well as chronic effects from short and long-term exposure

Skin corrosion/irritation              Classification based on data available for ingredients. Causes skin irritation.

Serious eye damage/eye irritation    Classification based on data available for ingredients. Causes burns. Causes serious eye

damage.

Respiratory or skin sensitization   No information available.

Germ cell mutagenicity          No information available.

Carcinogenicity               No information available.

Reproductive toxicity           No information available.

STOT - single exposure          No information available.

STOT - repeated exposure        No information available.

Target organ effects                 Respiratory system, Eyes, Skin, Teeth.

Aspiration hazard              No information available.

Other adverse effects           No information available.

Interactive effects              No information available.

AGHS   / EN                                                                             Page   8 / 13

Page 9

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

12. Ecological information

Ecotoxicity                     The environmental impact of this product has not been fully investigated.

Persistence and degradability     No information available.

Bioaccumulation                  There is no data for this product.

Component Information

Chemical name                                                    Partition coefficient

Nitric Acid                                                            -2.3

7697-37-2

Other adverse effects           No information available.

13. Disposal considerations

Disposal methods

Waste from residues/unused       Dispose of in accordance with local regulations. Dispose of waste in accordance with

products                           environmental legislation.

Contaminated packaging         Do not reuse empty containers.

California Hazardous Waste Status  This product contains one or more substances that are listed with the State of California as

a hazardous waste.

14. Transport information

DOT

UN number or ID number      UN3264

Extended proper shipping name Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid)

Transport hazard class(es)     8

Packing group                               III

Reportable Quantity (RQ)          (Nitric Acid: RQ (kg)= 454.00)  Nitric Acid: RQ (lb)= 1000.00

Reportable quantity (kg)           Nitric Acid: RQ (kg)= 10088.89

(calculated)

Reportable quantity (lbs)          Nitric Acid: RQ (lb)= 22222.00

(calculated)

AGHS   / EN                                                                             Page   9 / 13

Page 10

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

Special Provisions               IB3, T7, TP1, TP28

DOT Marine Pollutant        NP

Description                  UN3264,  Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid), 8, III

Emergency Response Guide    154

Number

TDG

UN number or ID number      UN3264

UN proper shipping name       Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid)

Transport hazard class(es)     8

Packing group                               III

Special Provisions            16

Marine pollutant           NP

Description                  UN3264,  Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid), 8, III

MEX

UN number or ID number      UN3264

UN proper shipping name       Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid)

Transport hazard class(es)     8

Packing group                               III

Technical Name                     Nitric Acid

Description                  UN3264,  Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid), 8, III

Special Provisions              223, 274

IATA

UN number or ID number      UN3264

UN proper shipping name       Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid)

Transport hazard class(es)     8

Packing group                               III

Technical Name                     Nitric Acid

Description                  UN3264,  Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid), 8, III

Special Provisions              A3, A803

ERG Code                   8L

IMDG

UN number or ID number      UN3264

UN proper shipping name       Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid)

Transport hazard class(es)     8

Packing group                               III

EmS-No.                        F-A, S-B

Special Provisions              223, 274

Marine pollutant           NP

Description                  UN3264,  Corrosive liquid, acidic, inorganic, n.o.s. (Nitric Acid), 8, III

15. Regulatory information

AGHS   / EN                                                                            Page  10 / 13

Page 11

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

International Inventories

TSCA                         LGC, to the best of its ability, has confirmed that the chemical substances in this product are

listed as “Active” in the EPA (Environmental Protection Agency) “TSCA Inventory

Notification (Active-Inactive) Requirements Rule” (“the Final Rule”) of Feb 2019, as

amended Feb 2021.”.

Chemical name               CAS No.              Inventory Listing Status       Commercial Activity

Designation

Nitric Acid                      7697-37-2                   Present                       Active

DSL/NDSL                         Contact supplier for inventory compliance status.

EINECS/ELINCS                    Contact supplier for inventory compliance status.

ENCS                              Contact supplier for inventory compliance status.

IECSC                             Contact supplier for inventory compliance status.

KECI                               Contact supplier for inventory compliance status.

PICCS                             Contact supplier for inventory compliance status.

AIIC                               Contact supplier for inventory compliance status.

Legend:

TSCA  - United States Toxic Substances Control Act Section 8(b) Inventory

DSL/NDSL  - Canadian Domestic Substances List/Non-Domestic Substances List

EINECS/ELINCS  - European Inventory of Existing Chemical Substances/European List of Notified Chemical Substances

ENCS  - Japan Existing and New Chemical Substances

IECSC  - China Inventory of Existing Chemical Substances

KECL  - Korean Existing and Evaluated Chemical Substances

PICCS  - Philippines Inventory of Chemicals and Chemical Substances

AICS  - Australian Inventory of Chemical Substances

US Federal Regulations

SARA 313

Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 (SARA).  This product contains a chemical

or chemicals which are subject to the reporting requirements of the Act and Title 40 of the Code of Federal Regulations, Part 372.

Chemical name                           SARA 313 - Threshold Values %

Nitric Acid - 7697-37-2                                                  1.0

SARA 311/312 Hazard Categories

Should this product meet EPCRA 311/312 Tier reporting criteria at 40 CFR 370, refer to Section 2 of this SDS for appropriate

classifications.

CWA (Clean Water Act)

AGHS   / EN                                                                            Page  11 / 13

Page 12

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

This product contains the following substances which are regulated pollutants pursuant to the Clean Water Act (40 CFR 122.21 and

40 CFR 122.42).

Chemical name     CWA - Reportable   CWA - Toxic Pollutants CWA - Priority Pollutants   CWA - Hazardous

Quantities                                                         Substances

Nitric Acid              1000 lb                             -                                  -                 X

7697-37-2

CERCLA

This material, as supplied, contains one or more substances regulated as a hazardous substance under the Comprehensive

Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302).

Chemical name           Hazardous Substances RQs     Extremely Hazardous      Reportable Quantity (RQ)

Substances RQs

Nitric Acid                     1000 lb                  1000 lb          RQ 1000 lb final RQ

7697-37-2                                                RQ 454 kg final RQ

US State Regulations

California Proposition 65

This product does not contain any Proposition 65 chemicals.

U.S. State Right-to-Know Regulations

Chemical name             New Jersey                 Massachusetts                 Pennsylvania

Water                                       -                                           -                     X

7732-18-5

Nitric Acid                  X                     X                     X

7697-37-2

U.S. EPA Label Information

EPA Pesticide Registration Number Not applicable

16. Other information

NFPA                 Health hazards 3      Flammability 0          Instability 0           Special hazards  -

HMIS                  Health hazards 3      Flammability 0        Physical hazards 0    Personal protection X

Key or legend to abbreviations and acronyms used in the safety data sheet

Legend  Section 8: Exposure controls/personal protection

AGHS   / EN                                                                            Page  12 / 13

Page 13

SAFETY DATA SHEET

This safety data sheet was created pursuant to the requirements of:

OSHA HCS2012

5190-8456 - Gadolinium Standard: 1000 μg/mL Gd in                           Revision date  25-Jun-2024

5% HNO3 [500ml bottle]

TWA         TWA (time-weighted average)           STEL          STEL (Short Term Exposure Limit)

Ceiling         Maximum limit value                      Sk*               Skin designation

Key literature references and sources for data used to compile the SDS

Agency for Toxic Substances and Disease Registry (ATSDR)

U.S. Environmental Protection Agency ChemView Database

European Food Safety Authority (EFSA)

Environmental Protection Agency

Acute Exposure Guideline Level(s) (AEGL(s))

U.S. Environmental Protection Agency Federal Insecticide, Fungicide, and Rodenticide Act

U.S. Environmental Protection Agency High Production Volume Chemicals

Food Research Journal

Hazardous Substance Database

International Uniform Chemical Information Database (IUCLID)

Japan GHS Classification

Australia National Industrial Chemicals Notification and Assessment Scheme (NICNAS)

NIOSH (National Institute for Occupational Safety and Health)

National Library of Medicine's ChemID Plus (NLM CIP)

National Library of Medicine’s PubMed database (NLM PUBMED)

U.S. National Toxicology Program (NTP)

New Zealand's Chemical Classification and Information Database (CCID)

Organization for Economic Co-operation and Development Environment, Health, and Safety Publications

Organization for Economic Co-operation and Development High Production Volume Chemicals Program

Organization for Economic Co-operation and Development Screening Information Data Set

World Health Organization

Revision date                     25-Jun-2024

Revision Note                 No information available.

Disclaimer

The information contained in this document is based on Agilent’s state of knowledge at the time of preparation. No

warranty as to its accurateness, completeness or suitability for a particular purpose is expressed or implied.

End of Safety Data Sheet

AGHS   / EN                                                                            Page  13 / 13