Helium-3, Enriched Helium-3 SDS

Publisher: Neonest AB

Source: https://resds.com/document/helium-3-isotope-sds-he-3-3he-helium-3-enriched-helium-3-0ca2e7fa06

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Helium-3,

Enriched Helium-3

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 11-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Helium-3, Enriched Helium-3   https://www.buyisotope.com   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Helium-3.

Molecular weight:   3.016

Chemical formula:   ³He

Synonyms : Helium, Isotopic Helium,   ³He , He-3

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Compressed Gas   H280

Label elements:

Hazard pictograms:

Signal Words:   Warning.

Hazard Statements:

H280 Contains gas under pressure; may explode if heated.

Page 2

Safety Data Sheet for Helium-3,

Enriched Helium-3

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 11-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Helium-3, Enriched Helium-3   https://www.buyisotope.com   Page 2/9

Precautionary Statements:

P410 + P403 Protect from sunlight. Store in a well-ventilated place.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Concentration

% by weight

Classification

EC1272/2008 EC - No.

(3He)helium

14762-55-1

>99.9%   Press. Gas (Comp.)   H280

238-822-9

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

Eye Contact:   None under normal use. Get medical attention if symptoms occur.

Skin Contact:   None under normal use. Get medical attention if symptoms occur.

Inhalation:   PROMPT MEDICAL ATTENTION IS MANDATORY IN ALL CASES OF INHALATION OVEREXPOSURE. RESCUE

PERSONNEL SHOULD BE EQUIPPED WITH SELF-CONTAINED BREATHING APPARATUS. Conscious inhalation victims

should be assisted to an uncontaminated area and inhale fresh air. If breathing is difficult, administer oxygen.

Unconscious persons should be moved to an uncontaminated area and, as necessary, given artificial resuscitation and

supplemental oxygen. Treatment should be symptomatic and supportive.

Ingestion:   None under normal use. Get medical attention if symptoms occur. Treat symptomatically.

Most important symptoms and effects, both acute and delayed:

Inhalation:   Principal route of exposure is inhalation. Simple asphyxiant. May cause suffocation by displacing the

oxygen in the air. Exposure to oxygen deficient atmosphere (<19.5%) may cause dizziness, drowsiness, nausea,

vomiting, excess salivation, diminished mental alertness, loss of consciousness and death. Exposure to atmospheres

containing 8-10% or less oxygen will bring about unconsciousness without warning and so quickly that the individuals

cannot help or protect themselves. Lack of sufficient oxygen may cause serious injury or death.

Indication of any immediate medical attention and special treatment needed:   None known.

Section 5: Fire - Fighting Measures

Extinguisher media:

Flammable Properties:   Not flammable.

Suitable Extinguishing Media:   Use extinguishing measures that are appropriate to local circumstances and the

surrounding environment.

Sensitivity to Mechanical Impact:   None.

Sensitivity to Static Discharge:   None.

Specific Hazards Arising from the Chemical:   Cylinders may rupture under extreme heat. Continue to cool fire-exposed

cylinders until flames are extinguished. Damaged cylinders should be handled only by specialists.

Protective Equipment and Precautions for Firefighters:   As in any fire, wear Self-Contained Breathing Apparatus

pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear.

Page 3

Safety Data Sheet for Helium-3,

Enriched Helium-3

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 11-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Helium-3, Enriched Helium-3   https://www.buyisotope.com   Page 3/9

Section 6: Accidental   Release Measures

Personal precautions, protective equipment and emergency procedures:

Ensure adequate ventilation. Evacuate personnel to safe areas. Use personal protective equipment. Monitor oxygen

level.

Environmental precautions:   Prevent spreading of vapours through sewers, ventilation systems and confined areas.

Methods and materials for containment and cleaning up:

Stop the flow of gas or remove cylinder to outdoor location if this can be done without risk. If leak is in container or

container valve, contact the appropriate emergency telephone number in Section 1.

Section 7: Handling and Storage

Precautions for safe handling:

Handling:   Use only in ventilated areas. Never attempt to lift a cylinder by its valve protection cap. Protect cylinders

from physical damage; do not drag, roll, slide or drop. When moving cylinders, even for short distance, use a cart

designed to transport cylinders. Use equipment rated for cylinder pressure. Use backflow preventive device in piping.

Never insert an object (e.g. wrench, screwdriver, pry bar, etc.) into valve cap openings. Doing so may damage valve,

causing leak to occur.

Use an adjustable strap wrench to remove over-tight or rusted caps. Close valve after each use and when empty. If

user experiences any difficulty operating cylinder valve, discontinue use and contact supplier. Never put cylinders into

trunks of cars or unventilated areas of passenger vehicles. Never attempt to refill a compressed gas cylinder without

the owner's written consent. Never strike an arc on a compressed gas cylinder or make a cylinder a part of an

electrical circuit. Keep out of the reach of children. Handle in accordance with good industrial hygiene and safety

practice.

Conditions for safe storage, including any incompatibilities:

Protect from physical damage. Cylinders should be stored upright with valve protection cap in place and firmly

secured to prevent falling. Store in cool, dry, well-ventilated area of non-combustible construction, away from heavily

trafficked areas and emergency exits. Keep at temperatures   below 52°C / 125°F. Full and empty cylinders should be

segregated.

Section 8: Exposure Controls/ Personal Protection

Control parameters:

Occupational exposure limits:   This product does not contain any hazardous materials with occupational exposure

limits established by the region-specific regulatory bodies.

Exposure controls:

Appropriate engineering controls:   Local exhaust ventilation to prevent accumulation of high concentrations and

maintain air-oxygen levels at or above 19.5%. Ensure adequate ventilation, especially in confined areas.

Individual protection measures, such as personal protective equipment:

Eye/Face Protection:   Wear protective eyewear (safety glasses).

Page 4

Safety Data Sheet for Helium-3,

Enriched Helium-3

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 11-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Helium-3, Enriched Helium-3   https://www.buyisotope.com   Page 4/9

Skin and Body Protection:   Work gloves and safety shoes are recommended when handling cylinders.

Respiratory Protection:   No special protective equipment required.

Emergency Use:   Use positive pressure airline respirator with escape cylinder or Self-Contained Breathing Apparatus

for oxygen-deficient atmospheres (<19.5%).

Hygiene Measures:   Wear suitable gloves and eye/face protection.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Gas.

Colour:   Colourless.

Odour:   Odourless.

Odour threshold:   No data available.

pH (concentration):   Not applicable.

Melting point/range (° C):   No data available.

Boiling point/range (° C):   No data available.

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   Not applicable.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour pressure (20   ° C):   No data available.

Vapour density:   No data available.

Relative densi ty (25 ° C):   No data available.

Water solubility (g/L ) at 20 ° C:   8.61 m³/1 kg water

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Stability:   Stable.

Incompatible Products:   None known.

Conditions to Avoid:   None known.

Hazardous Decomposition Products:   None known, based on information supplied.

Hazardous Polymerization:   Hazardous polymerization does not occur.

Section 11: Toxicological Information

Information on toxicological effects:

Not classified based on available information.

Classification according to GHS (1272/2008/EG, CLP)

Page 5

Safety Data Sheet for Helium-3,

Enriched Helium-3

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 11-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Helium-3, Enriched Helium-3   https://www.buyisotope.com   Page 5/9

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Not classified based on available information.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:   No data available.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   No data available.

Other adverse effects:   No data available.

Section 13: Disposal Considerations

Waste Disposal Methods:   Do not attempt to dispose of residual waste or unused quantities. Return in the shipping

container PROPERLY LABELED, WITH ANY VALVE OUTLET PLUGS OR CAPS SECURED, AND VALVE PROTECTION CAP IN

PLACE, to the supplier for proper disposal.

Section 14: Transport Information

UN number:   1046

UN proper shipping name:   HELIUM, COMPRESSED.

Transport hazard class:   Class 2.2

Transport hazard label:

Marine pollutant:   No.

Page 6

Safety Data Sheet for Helium-3,

Enriched Helium-3

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 11-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Helium-3, Enriched Helium-3   https://www.buyisotope.com   Page 6/9

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Page 7

Safety Data Sheet for Helium-3,

Enriched Helium-3

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 11-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Helium-3, Enriched Helium-3   https://www.buyisotope.com   Page 7/9

Not listed.

Additional Information:

OSHA Hazards:   Compressed Gas.

SARA 313 Components:   Section 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986

(SARA). This product does not contain any chemicals which are subject to the reporting requirements of the Act and

Title 40 of the Code of Federal Regulations, Part 372.

SARA 311/312 Hazards:   Sudden Release of Pressure Hazard.

Clean Water Act:   This product does not contain any substances regulated as pollutants pursuant to the Clean Water

Act (40 CFR 122.21 and 40 CFR 122.42).

Clean Air Act, Section 112 Hazardous Air Pollutants (HAPs) (see 40 CFR 61):   This product does not contain any

substances regulated as hazardous air pollutants (HAPS) under Section 112 of the Clean Air Act Amendments of 1990.

CERCLA/SARA:   This material, as supplied, does not contain any substances regulated as hazardous substances under

the Comprehensive Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302) or the Superfund

Amendments and Reauthorization Act (SARA) (40 CFR 355). There may be specific reporting requirements at the local,

regional or state level pertaining to releases of this material.

California Prop. 65 Components:   This product does not contain any Proposition 65 chemicals.

Canadian Regulations:   WHMIS Classification: A

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

Page 8

Safety Data Sheet for Helium-3,

Enriched Helium-3

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 11-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Helium-3, Enriched Helium-3   https://www.buyisotope.com   Page 8/9

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H280 Contains gas under pressure; may explode if heated.

Revision information:

Page 9

Safety Data Sheet for Helium-3,

Enriched Helium-3

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 11-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Helium-3, Enriched Helium-3   https://www.buyisotope.com   Page 9/9

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.