Holmium (Ho) Standard Solution SRM 3123a SDS

Publisher: National Institute of Standards and Technology (NIST)

Source: https://resds.com/document/holmium-ho-standard-solution-nist-srm-3123a-en-b27a28d452

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Date of Issue:

15 October 2021

SAFETY DATA SHEET

1. SUBSTANCE AND SOURCE IDENTIFICATION

Product Identifier

SRM Number:   3123a

SRM Name:    Holmium (Ho) Standard Solution

Other Means of Identification: Not applicable.

Recommended Use of This Material and Restrictions of Use

This Standard Reference Material (SRM) is intended for use as a primary calibration standard for the quantitative

determination of holmium. A unit of SRM 3123a consists of five 10 mL sealed borosilicate glass ampoules of an

acidified aqueous solution prepared gravimetrically from high purity holmium oxide to contain a known mass

fraction of holmium.

Company Information

National Institute of Standards and Technology

Standard Reference Materials Program

100 Bureau Drive, Stop 2300

Gaithersburg, Maryland 20899-2300

Telephone: 301-975-2200                         Emergency Telephone ChemTrec:

E-mail: SRMMSDS@nist.gov                         1-800-424-9300   (North America)

Website: https://www.nist.gov/srm                    +1-703-527-3887    (International)

2. HAZARDS IDENTIFICATION

Classification

Physical Hazard:    Not classified.

Health Hazard:      Skin Corrosion/Irritation            Category 1B

Serious Eye Damage/Eye Irritation   Category 1

Label Elements

Symbol

Signal Word

DANGER

Hazard Statement(s)

H314               Causes severe skin burns and eye damage.

Precautionary Statement(s)

P260            Do not breathe fumes, mists, vapors, or spray.

P264             Wash hands thoroughly after handling.

P280              Wear protective gloves, protective clothing, and eye protection.

P301 + P330 + P331   If swallowed: Rinse mouth. Do NOT induce vomiting.

P303 + P361 + P353   If on skin (or hair): Remove immediately all contaminated clothing.  Rinse skin with

water.

P304 + P340             If inhaled: Remove person to fresh air and keep comfortable for breathing.

P305 + P351 + P338   If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if

present and easy to do. Continue rinsing.

P310                Immediately call a doctor.

P363             Wash contaminated clothing before reuse.

P405                 Store locked up.

SRM 3123a                                                                            Page 1 of 7

Page 2

P501                Dispose of container and contents according to local regulations.

Hazards Not Otherwise Classified: Not applicable.

Ingredients(s) with Unknown Acute Toxicity: Not applicable.

3. COMPOSITION AND INFORMATION ON HAZARDOUS INGREDIENTS

Substance: Holmium in nitric acid solution

Other Designations:

Nitric Acid (Aqua fortis; hydrogen nitrate; azotic acid; engraver’s acid)

Holmium Nitrate (Holmium trinitrate)

NOTE: Holmium in nitric acid solution forms a solvated holmium nitrate salt. The health and physical hazard

information provided in this SDS are for nitric acid and holmium nitrate. No physical or chemical data are listed for

this solution. The actual effects of the solution may differ from the individual components.

Components are listed in compliance with OSHA’s 29 CFR 1910.1200; for the actual values see the Certificate of

Analysis.

Hazardous Component(s)       CAS Number     EC Number     Nominal Mass Concentration

(EINECS)                (%)

Nitric acid                           7697-37-2         231-714-2              <20

Holmium nitrate                     10168-82-8         233-438-8                 1

Non-Hazardous Component(s)

Water                               7732-18-5         231-791-2                80

4. FIRST AID MEASURES

Description of First Aid Measures:

Inhalation:  If adverse effects occur, remove to uncontaminated area. If not breathing, give artificial respiration

or oxygen by qualified personnel. Seek immediate medical attention.

Skin Contact: Wash skin with soap and water for at least 15 minutes while removing contaminated clothing and

shoes. Get immediate medical attention. Thoroughly clean and dry contaminated clothing before reuse. Destroy

contaminated shoes.

Eye Contact: Immediately flush eyes, including under the eyelids with copious amounts of water for at least 15

minutes. Seek immediate medical attention.

Ingestion: Contact a poison control center immediately for instructions. Do not induce vomiting. Give water to

rinse out mouth. Never give liquids to a person with reduced awareness or becoming unconscious.  If vomiting

occurs, keep head lower than hips to prevent aspiration.  If not breathing, give artificial respiration by qualified

personnel. Seek immediate medical attention.

Most Important Symptoms/Effects, Acute and Delayed: Acid burns to skin, eyes, and lungs.

Indication of any immediate medical attention and special treatment needed, if necessary:  If any of the above

symptoms are present, seek immediate medical attention.

5. FIRE FIGHTING MEASURES

Fire and Explosion Hazards:  Negligible fire hazard.  See Section 9, “Physical and Chemical Properties” for

flammability properties.

Extinguishing Media:

Suitable: Use extinguishing media appropriate to the surrounding fire.

Unsuitable: None listed.

Specific Hazards Arising from the Chemical: Thermal decomposition will form oxides of nitrogen.

SRM 3123a                                                                            Page 2 of 7

Page 3

Special Protective Equipment and Precautions for Fire-Fighters: Avoid inhalation of material or combustion

byproducts. Wear full protective clothing and NIOSH approved self-contained breathing apparatus (SCBA).

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions, Protective Equipment and Emergency Procedures:  Immediately contact emergency

personnel. Keep unnecessary personnel away. Use suitable protective equipment; see Section 8, “Exposure Controls

and Personal Protection”.

Methods and Materials for Containment and Clean up: Do not touch spilled material. Notify safety personnel

of spills. Absorb with sand or other non-combustible material. Collect spilled material in appropriate container for

disposal. Isolate hazard area and deny entry.

7. HANDLING AND STORAGE

Safe Handling Precautions: See Section 8, “Exposure Controls and Personal Protection”. Handle glass ampoules

with care.

Storage:  Store and handling in accordance with all current regulations and standards.  Keep separated from

incompatible substances (acids, combustible materials, halo carbons, amines, bases, oxidizing materials, metals,

halogens, metal salts, metal oxides, reducing agents, peroxides, metal carbide, cyanides).

8. EXPOSURE CONTROLS AND PERSONAL PROTECTION

Exposure Limits:

Component: Nitric acid

NIOSH (REL):     5 mg/m3 (2 ppm; TWA)

10 mg/m3 (4 ppm; STEL)

65 mg/m3(25 ppm; IDLH)

ACGIH (TLV):    5 mg/m3 (2 ppm; TWA)

10 mg/m3 (4 ppm; STEL)

OSHA (PEL):      5 mg/m3

Component: Holmium nitrate; No occupational limits established.

Engineering Controls:  Provide local exhaust or process enclosure ventilation system. Ensure compliance with

applicable exposure limits.

Personal Protection: In accordance with OSHA 29 CFR 1910.132, subpart I, wear appropriate Personal Protective

Equipment (PPE) to minimize exposure to this material.

Respiratory Protection:   If workplace conditions warrant a respirator, a respiratory protection program that meets

OSHA 29CFR 1910.134 must be followed.  Refer to NIOSH 42 CFR 84 for applicable certified respirators.

Eye/Face Protection: Wear splash resistant safety goggles with a face shield. An eyewash station should be readily

available near areas of use.

Skin and Body Protection: Personal protective equipment for the body should be selected based on the task being

performed and the risks involved and should be approved by a specialist before handling this product. Chemical-

resistant gloves should be worn at all times when handling chemicals.

SRM 3123a                                                                            Page 3 of 7

Page 4

9. PHYSICAL AND CHEMICAL PROPERTIES

Descriptive Properties:

Appearance                                          colorless to yellow liquid

(physical state, color, etc.):

Molecular Formula:                              not available

Molar Mass (g/mol):                              not available

Odor:                                             not available

Odor threshold:                                  not available

pH:                                                  acidic

Evaporation rate:                                 not available

Melting point/freezing point (ºC):                 not available

Relative Density (g/L):                            not available

Vapor Pressure (mmHg):                         not available

Vapor Density (air = 1):                           not available

Viscosity (cP):                                    not available

Solubility(ies):                                     miscible with water and ether

Partition coefficient                               not available

(n-octanol/water):

Particle Size (if relevant)                          not applicable

Thermal Stability Properties:

Autoignition Temperature (ºC):                   not applicable

Thermal Decomposition (ºC):                     not available

Initial boiling point and boiling range (ºC):        not available

Explosive Limits, LEL (Volume %):              not applicable

Explosive Limits, UEL (Volume %):              not applicable

Flash Point (ºC)                                   not applicable

Flammability (solid, gas):                         not applicable

10. STABILITY AND REACTIVITY

Reactivity: Stable at normal temperatures and pressure.

Stability:     X       Stable              Unstable

Possible Hazardous Reactions: None listed.

Conditions to Avoid: Contact with combustible or incompatible materials.

Incompatible Materials:  Acids, combustible materials, halo carbons, amines, bases, oxidizing materials, metals,

halogens, metal salts, metal oxides, reducing agents, peroxides, metal carbide, cyanides.

Fire/Explosion Information: See Section 5, “Fire Fighting Measures”.

Hazardous Decomposition: Thermal decomposition will produce oxides of nitrogen.

Hazardous Polymerization:         Will Occur     X    Will Not Occur

11. TOXICOLOGICAL INFORMATION

Route of Exposure:   X    Inhalation     X    Skin        X    Ingestion

Symptoms Related to the Physical, Chemical and Toxicological Characteristics: Burning pain and severe skin

corrosion, and eye damage.

Potential Health Effects (Acute, Chronic and Delayed):

SRM 3123a                                                                            Page 4 of 7

Page 5

Inhalation: Inhalation of nitric acid can damage the mucous membranes and upper respiratory tract. Short term

exposure may cause irritation and inflammation of the upper respiratory tract, coughing, choking, sore throat,

shortness of breath, headache, dizziness, and nausea. Long term exposure to acid fumes may cause damage to

teeth, bronchial irritation, chronic cough, bronchial pneumonia, and gastrointestinal disturbances.  Holmium

nitrate exposure may cause irritation; chronic exposure may result in itching, sensitivity to heat, lung damage.

Skin Contact:  Nitric acid can cause severe skin burns.  Severity of the damage depends on the concentration

and duration of exposure. Effects of acid burns may be delayed. Holmium nitrate exposure may cause hair loss.

Eye Contact:  Nitric acid can cause severe eye irritation, corneal burns, permanent eye damage, or blindness.

Severity of the damage depends on the concentration and duration of exposure. Holmium nitrate exposure may

cause irritation.

Ingestion:  If ingested, nitric acid can cause severe burns and damage to the gastrointestinal tract.  Intestinal

absorption is usually poor for rare earth salts, holmium nitrate.

Numerical Measures of Toxicity:

Acute Toxicity: Not classified.

Nitric acid, Rat, Inhalation LC50: 130 mg/m3 (4 h)

Holmium nitrate, Rat, Oral LD50: 2313 mg/kg

Skin Corrosion/Irritation: This SRM contains >1 % of nitric acid and it is classified as Category 1B.

Serious Eye damage/Eye irritation: This SRM contains >1 % nitric acid and it is classified as Category 1.

Respiratory Sensitization: No data available.

Skin Sensitization: No data available.

Germ Cell Mutagenicity: No data available.

Carcinogenicity: Not classified.

Listed as a Carcinogen/Potential Carcinogen              Yes       X   No

Nitric acid is not listed by NTP, IARC or OSHA as a carcinogen.

Holmium nitrate is not listed by NTP, IARC or OSHA as a carcinogen.

Reproductive Toxicity: Not classified.

Nitric acid, Rat, Oral TDLo: 21 150 mg/kg (pregnant 1 d to 21 d)

Nitric acid, Rat, Oral TDLo: 2345 mg/kg (pregnant 18 d)

Holmium nitrate: No data available.

Specific Target Organ Toxicity, Single Exposure: Not classified.

Specific Target Organ Toxicity, Repeated Exposure: Not classified.

Aspiration Hazard: No data available.

12. ECOLOGICAL INFORMATION

Ecotoxicity Data:

Nitric acid, mosquitofish (Gambusia affinis) LC50: 72 mg/L (96 h)

Holmium nitrate: No data available.

Persistence and Degradability: No data available.

Bioaccumulative Potential: No data available.

Mobility in Soil: No data available.

Other Adverse effects: No data available.

13. DISPOSAL CONSIDERATIONS

Waste Disposal: Dispose of waste in accordance with all applicable federal, state, and local regulations. Nitric acid

subject to disposal regulations: U.S. EPA 40 CFR 262, Hazardous Waste Numbers: D001, D002.

14. TRANSPORTATION INFORMATION

U.S. DOT and IATA: UN1760, Corrosive liquid, n.o.s. (contains nitric acid), Hazard Class 8, Packing

Group II, Excepted Quantities E2.

SRM 3123a                                                                            Page 5 of 7

Page 6

15. REGULATORY INFORMATION

U.S. Regulations:

CERCLA Sections 102a/103 (40 CFR 302.4):  Nitric acid, 1000 lbs (454 kg) RQ

SARA Title III Section 302 (40 CFR 355.30):  Nitric acid, 1000 lbs (454 kg) TPQ

SARA Title III Section 304 (40 CFR 355.40):  Nitric acid, 1000 lbs (454 kg) EPCRA RQ

SARA Title III Section 313 (40 CFR 372.65):  Nitric acid, 1 % de minimis concentration.

OSHA Process Safety (29 CFR 1910.119):     Regulated for nitric acid at higher concentrations

500 lb TQ (≥94.5 % by weight).

SARA Title III Sections 311/312 Hazardous Categories (40 CFR 370.21):

ACUTE HEALTH:         Yes.

CHRONIC HEALTH:     No.

FIRE:                   No.

REACTIVE:             No.

PRESSURE:             No.

State Regulations:

California Proposition 65: Not listed.

U.S. TSCA Inventory: Nitric acid and holmium nitrate are listed.

TSCA 12(b), Export Notification: Not listed.

Canadian Regulations:

WHMIS Information: Not provided for this material.

SRM 3123a                                                                            Page 6 of 7

Page 7

16. OTHER INFORMATION

Issue Date: 15 October 2021

Sources:   ChemAdvisor, Inc., MSDS Nitric Acid, 09 December 2015.

ChemAdvisor, Inc., MSDS Holmium Nitrate, 13 March 2013.

PubChem,    National    Library    of    Medicine,   Holmium     Nitrate,     available     at

https://pubchem.ncbi.nlm.nih.gov/compound/25024 (accessed Sep 2021).

CDC; NIOSH; NIOSH Pocket Guide to Chemical Hazards; Department of Health and Human

Services (DHHS), Centers for Disease Control and Prevention (CDC), National Institute for Safety and

Health; Nitric Acid, 29 November 2018; available at https://www.cdc.gov/niosh/npg/npgd0447.html

(accessed Sep 2019).

Key of Acronyms:

ACGIH       American Conference of Governmental Industrial    NRC          Nuclear Regulatory Commission

Hygienists

ALI          Annual Limit on Intake                     NTP           National Toxicology Program

CAS          Chemical Abstracts Service                 OSHA         Occupational Safety and Health Administration

CERCLA      Comprehensive Environmental Response,         PEL           Permissible Exposure Limit

Compensation, and Liability Act

CFR         Code of Federal Regulations                RCRA        Resource Conservation and Recovery Act

DOT          Department of Transportation                REL         Recommended Exposure Limit

EC50           Effective Concentration, 50%             RM           Reference Material

EINECS       European Inventory of Existing Commercial       RQ            Reportable Quantity

Chemical Substances

EPCRA       Emergency Planning and Community Right-to-Know  RTECS        Registry of Toxic Effects of Chemical Substances

Act

IARC           International Agency for Research on Cancer      SARA        Superfund Amendments and Reauthorization Act

IATA           International Air Transport Association          SCBA         Self-Contained Breathing Apparatus

IDLH         Immediately Dangerous to Life and Health       SRM          Standard Reference Material

LC50          Lethal Concentration, 50 %                   STEL          Short Term Exposure Level

LD50          Lethal Dose, 50 %                       TLV          Threshold Limit Value

LEL         Lower Explosive Limit                     TPQ          Threshold Planning Quantity

MSDS         Material Safety Data Sheet                  TSCA         Toxic Substances Control Act

NIOSH        National Institute for Occupational Safety and Health TWA        Time Weighted Average

NIST          National Institute of Standards and Technology     UEL         Upper Explosive Limit

n.o.s.         Not Otherwise Specified                  WHMIS      Workplace Hazardous Materials Information System

Disclaimer: The NIST SDS information is specific to the NIST product and is believed to be correct, based upon

our current knowledge. The SDS may not necessarily be all inclusive and should be used only as a guide. NIST

does not guarantee the accuracy or completeness of this information. The only official source for specific values

and uncertainties is the certificate or report.

Users of this SRM should ensure that the SDS in their possession is current. This can be accomplished by contacting

the SRM  Program:  telephone  (301)  975-2200;  e-mail  srmmsds@nist.gov;  or  via  the   Internet   at

https://www.nist.gov/srm.

SRM 3123a                                                                            Page 7 of 7