Indium-115 SDS

Publisher: Neonest AB

Source: https://resds.com/document/indium-115-isotope-sds-in-115-115in-indium-enriched-indium-1eb1fabdff

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Indium,

Enriched Indium

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 14-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Indium, Enriched Indium   https://www.buyisotope.com   Page 1/9

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Indium, Enriched Indium.

Molecular weight:   114.82

Chemical formula:   In

Synonyms:   None.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

STOT RE (Category 1)   H372

Aquatic Chronic (Category 2)   H411

Label elements:

Hazard pictograms:

Signal Words:   Danger.

Page 2

Safety Data Sheet for Indium,

Enriched Indium

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 14-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Indium, Enriched Indium   https://www.buyisotope.com   Page 2/9

Hazard Statements:

H372 Causes damage to organs through prolonged or repeated exposure.

H411 Toxic to aquatic life with long lasting effects.

Precautionary Statements:

P261 Avoid breathing dust/vapours.

P274 Wash hands thoroughly after handling.

P304 + P340 IF INHALED: Remove person to fresh air and keep comfortable for breathing.

P314 Get medical advice/attention if you feel unwell.

P501 Dispose of contents/container in accordance with local and international regulations.

Other hazards:   None known.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Molecular

weight

Concentration

% by weight

Classification

EC1272/2008 EC - No.

Indium

7440-74-6

114.82   >99%   STOT RE 1   H372

Aquatic Chronic 2H411 231-180-0

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

In case of inhalation:   Evacuate the victim to a safe area as soon as possible. Loosen tight clothing such as a collar, tie,

belt or waistband. If breathing is difficult, administer oxygen. If the victim is not breathing, perform mouth-to-mouth

resuscitation. Seek medical attention.

In case of skin contact:   After contact with skin, wash immediately with plenty of water. Gently and thoroughly wash

the contaminated skin with running water and nonabrasive soap. Be particularly careful to clean folds, crevices,

creases and groin. Cover the irritated skin with an emollient. If irritation persists, seek medical attention. Wash

contaminated clothing before reusing.

In case of eye contact:   No known effect on eye contact; rinse with water for a few minutes.

In case of ingestion:   Do not induce vomiting. Loosen tight clothing such as a collar, tie, belt or waistband. If the victim

is not breathing, perform mouth-to-mouth resuscitation. Seek immediate medical attention.

Most important symptoms and effects, both acute and delayed:

Inhalation:   Hazardous in case of inhalation.

Eyes:   Hazardous in case of eye contact.

Skin contact:   Hazardous in case of skin contact.

Ingestion:   Hazardous in case of ingestion.

Indication of any immediate medical attention and special treatment needed:   Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Page 3

Safety Data Sheet for Indium,

Enriched Indium

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 14-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Indium, Enriched Indium   https://www.buyisotope.com   Page 3/9

Section 5: Fire - Fighting Measures

Extinguisher media:

Suitable extinguisher media:   Use water spray or fog. Cool containing vessels with water jet in order to prevent

pressure build-up, autoignition or explosion.

Unsuitable extinguishing media:   None known.

Special hazards arising from the mixture:   Indium/indium oxides.

Advice for fire-fighters:   Wear self-contained breathing apparatus and protective clothing for firefighting.

Further information:   Standard procedure for chemical fires. Use extinguishing measures that are appropriate to local

circumstances and the surrounding environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions:   Use personal protective equipment. Avoid dust formation. Avoid breathing vapours, mist or

gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust. For personal protection see

section 8.

Environmental precautions:   Prevent product from entering drains. Prevent further leakage or spillage if safe to do so.

Local authorities should be advised if significant spillages cannot be contained.

Methods for containment and cleaning up:

Methods for cleaning up:   Stop leak if without risk. Do not touch spilled material. Use water spray curtain to divert

vapor drift. Prevent entry into sewers, basements or confined areas; dike if needed. Eliminate all ignition sources. Call

for assistance on disposal. Be careful that the product is not present at a concentration level above TLV. Check TLV on

the MSDS and with local authorities.

Reference to other sections:

Treat recovered material as described in the section "Disposal considerations".

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling:   Keep away from heat. Keep away from sources of ignition. Ground all equipment containing

material. Do not breathe dust.

Hygiene measures:   Do not eat, drink or smoke when using this product.

Conditions for safe storage, including incompatibilities:

Requirements for storage areas and containers:   Flammable materials should be stored in a separate safety storage

cabinet or room. Keep away from heat. Keep away from sources of ignition. Keep container tightly closed. Keep in a

cool, well-ventilated place. Ground all equipment containing material. Keep container dry. Keep in a cool place.

Section 8: Exposure Controls/ Personal   Protection

Control parameters:

Occupational Exposure Limits:

TWA 0.1 (mg/m 3 ) from OSHA (PEL).

Page 4

Safety Data Sheet for Indium,

Enriched Indium

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 14-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Indium, Enriched Indium   https://www.buyisotope.com   Page 4/9

TWA 0.1 (mg/m 3 ) from ACGIH (Consult local authorities for acceptable exposure limits).

Exposure controls:

Appropriate engineering controls:   Use process enclosures, local exhaust ventilation, or other engineering controls to

keep airborne levels below recommended exposure limits. If user operations generate dust, fume or mist, use

ventilation to keep exposure to airborne contaminants below the exposure limit.

Individual protection measures, such as personal protective equipment:

Eye/face protection:   Use safety glasses. If there is a potential for exposure to particles which could cause eye

discomfort, wear chemical goggles.

Hand protection:   Use chemical resistant gloves. Examples of preferred glove barrier materials include: Butyl rubber,

Neoprene, Nitrile/butadiene rubber, Polyethylene, Ethyl vinyl alcohol laminate, polyvinyl alcohol, Polyvinyl chloride.

Body protection:   Wear protective clothing as appropriate.

Respiratory protection:   Dust respirator. Be sure to use an approved/certified respirator or equivalent.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Solid (powder).

Colour:   Not applicable.

Odour:   No data available.

Odour threshold:   No data available.

Molecular Weight:   114.82

pH (concentration):   No data available.

Melting point/range (° C):   156.17   °C

Boiling point/range (° C):   200 0 °C

Freezing point   (°C):   No data available.

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   No data available.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour   pressure (20 ° C):   No data available.

Vapour density:   No data available.

Relative densi ty (25 ° C):   7.3

Water solubility (g/L ) at 20 ° C:   Insoluble.

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and React ivity

Reactivity:   No dangerous reaction known under conditions of normal use.

Chemical stability:   Stable under normal conditions.

Page 5

Safety Data Sheet for Indium,

Enriched Indium

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 14-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Indium, Enriched Indium   https://www.buyisotope.com   Page 5/9

Possibility of hazardous reactions:   No dangerous reaction known under conditions of normal use.

Conditions to avoid:   None   known .

Incompatible materials:   None known.

Hazardous decomposition products:   None known.

Section 11: Toxicological Information

Information on toxicological effects:

Not classified based on available information.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Not classified based on available information.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Causes damage to organs through prolonged or repeated exposure.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:   No data available.

Persistence and degradability:   Possibly hazardous short-term degradation products are not likely. However, long-

term degradation products may arise.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   Not relevant.

Other adverse effects:   No data available.

Section 13: Disposal Considerations

Waste treatment methods:   Contact a licensed professional waste disposal service to dispose of this material. Observe

all federal, state, and local environmental regulations. Catalysts and expensive metals should be recovered for reuse

or recycling.

Section 14: Transport Information

Page 6

Safety Data Sheet for Indium,

Enriched Indium

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 14-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Indium, Enriched Indium   https://www.buyisotope.com   Page 6/9

DOT:

Proper Shipping Name:   ENVIRONMENTALL HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Indium)

Hazard Class : 9

UN Number:   3077

Packing Group:   III

Hazard Labels:

IMDG:

Proper Shipping Name:   ENVIRONMENTALL HAZARDOUS SUBSTANCE, SOLID, N.O.S. (Indium)

Hazard Class : 9

UN Number:   3077

Packing Group:   III

Marine Pollutant:   Yes.

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Page 7

Safety Data Sheet for Indium,

Enriched Indium

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 14-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Indium, Enriched Indium   https://www.buyisotope.com   Page 7/9

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations:   The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

SARA 302 Components:   No chemicals in this material are subject to the reporting requirements of SARA Title III,

Section 302.

SARA 313 Components:   This material does not contain any chemical components with known CAS numbers that

exceed the threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 Hazards:   Fire Hazard, Acute Health Hazard, Chronic Health Hazard.

Massachusetts Right to Know Components:   Indium / CAS No. 7440-74-6 / Revision Date 1994-04-01

Pennsylvania Right to Know Components:   Indium / CAS No. 7440-74-6 / Revision Date 1994-04-01

New Jersey Right to Know Components:   Indium / CAS No. 7440-74-6 / Revision Date 1994-04-01

California Prop. 65 Components:   This product does not contain any chemicals known to the State of California to

cause cancer, birth defects, or any other reproductive harm.

National regulations:   Follow national regulation for work with chemical agents.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

Page 8

Safety Data Sheet for Indium,

Enriched Indium

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 14-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Indium, Enriched Indium   https://www.buyisotope.com   Page 8/9

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

Page 9

Safety Data Sheet for Indium,

Enriched Indium

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 14-Aug-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Indium, Enriched Indium   https://www.buyisotope.com   Page 9/9

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H372 Causes damage to organs through prolonged or repeated exposure.

H411 Toxic to aquatic life with long lasting effects.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.