Mercury-199 SDS

Publisher: Neonest AB

Source: https://resds.com/document/mercury-199-isotope-sds-hg-199-199hg-mercury-oxide-enriched-mercury-oxide-71e5e86d71

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 1/10

Section 1:   Identification of the Substance/Mixture and of the Company/Undertaking

Product Identifier:

Identification as on the label/Trade name:   Mercury Oxide, Enriched Mercury Oxide.

Molecular weight:   2 16 .59

Chemical formula:   HgO

Synonyms:   Mercury (II) oxide, mercuric oxide red, mercuric oxide yellow.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

24-hour Emergency Contact:

Swedish Poisons Centre

Phone: 112 - Ask for Poisons Information, 112   –   begär Giftinformation .

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Acute Toxicity, Oral (Category 2)   H300

Acute Toxicity, Dermal (Category 1)   H310

Acute Toxicity, Inhalation (Category 2)   H330

STOT RE (Category 2)   H373

Aquatic Acute (Category 1)   H400

Aquatic Chronic (Category 1)   H410

Label elements:

Hazard pictograms:

Page 2

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 2/10

Signal Words:   Danger.

Hazard Statements:

H300 Fatal if swallowed.

H310 Fatal in contact with skin.

H330 Fatal if inhaled.

H373 May cause damage to organs through prolonged or repeated exposure.

H400 Very toxic to aquatic life.

H410 Very toxic to aquatic life with long lasting effects.

Precautionary Statements:

P201 Obtain special instructions before use.

P202 Do not handle until all safety precautions have been read and understood.

P270 Do not eat, drink or smoke when using this product.

P271 Use only outdoors or in a well-ventilated area.

P273 Avoid release to the environment.

P301 + P310 IF SWALLOWED: Immediately call a POISON CENTER/doctor.

P302 + P352 IF ON SKIN: Wash with plenty of water.

P304 + P340 IF INHALED: Remove person to fresh air and keep comfortable for breathing.

P308 + P313 IF exposed or concerned: Get medical advice/attention.

P310 Immediately call a POISON CENTER/doctor.

P391 Collect spillage.

P403 + P233 Store in a well-ventilated place. Keep container tightly closed.

P405 Store locked up.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Concentration

% by weight

Classification

EC1272/2008 EC - No.

Mercury monoxide

21908-53-2

>99.9%

Acute Tox. 2   H300

Acute Tox. 1   H310

Acute Tox. 2   H330

STOT RE 2   H373

Aquatic Acute 1   H400

Aquatic Chronic 1H410

244-654-7

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

Page 3

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 3/10

Inhalation Exposure:   Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give

oxygen. Get medical attention immediately.

Oral Exposure:   Induce vomiting immediately as directed by medical personnel. Never give anything by mouth to an

unconscious person. Get medical attention immediately.

Skin Exposure:   Immediately flush skin with plenty of water for at least 15 minutes while removing contaminated

clothing and shoes. Get medical attention immediately. Wash clothing and shoes thoroughly before reuse.

Eye Contact:   Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids

occasionally. Get medical attention immediately.

Most important symptoms and effects, both acute and delayed:

Inhalation:   Causes irritation to the respiratory tract. Symptoms include sore throat, coughing, pain, tightness in chest,

breathing difficulties, shortness of breath and headache. Pneumonitis may develop. Can be absorbed through

inhalation with symptoms to parallel ingestion.

Ingestion:   Highly Toxic! Average lethal dose for inorganic mercury salts is about 1 gram. May cause burning of the

mouth and pharynx, abdominal pain, vomiting, corrosive ulceration, bloody diarrhoea. May be followed by a rapid and

weak pulse, shallow breathing, paleness, exhaustion, central nervous system problems, tremors and collapse. Delayed

death may occur from renal failure.

Skin Contact:   Causes irritation. Symptoms include redness and pain. May cause burns. May cause sensitization. Can be

absorbed through the skin with symptoms to parallel those of ingestion.

Eye Contact:   Causes irritation and burns to eyes. Symptoms include redness, pain, blurred vision; may cause serious

and permanent eye damage.

Chronic Exposure:   Chronic exposure through any route can produce central nervous system damage. May cause

muscle tremors, personality and behaviour changes, memory loss, metallic taste, loosening of the teeth, digestive

disorders, skin rashes, brain damage and kidney damage. Can cause skin allergies and can accumulate in the body.

Repeated skin contact can cause the skin to turn grey in colour. Not a known reproductive hazard, but related mercury

compounds can damage the developing foetus and can decrease fertility in males and females.

Aggravation of Pre-existing Conditions:   Persons with nervous disorders, or impaired kidney or respiratory function, or

a history of allergies or a known sensitization to mercury, may be more susceptible to the effects of the substance.

Section 5: Fire - Fighting Measures

Extinguisher media:

Fire:   Substance is a strong oxidizer, and its heat of reaction with reducing agents or combustibles may cause ignition.

Upon heating it decomposes to form oxygen, which increases the flammability of combustibles.

Explosion:   Not considered an explosion hazard.

Suitable Extinguishing Media:   Use any means suitable for extinguishing surrounding fire. Do not allow water runoff to

enter sewers or waterways.

Special Information:   In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing

apparatus with full face piece operated in the pressure-demand or other positive-pressure mode.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Ventilate area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8. Avoid dust

formation. Avoid breathing vapours, mist or gas.

Environmental precautions:   Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Discharge into the environment must be avoided.

Page 4

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 4/10

Methods and materials for containment and cleaning up:

Pick up and place in a suitable container for reclamation or disposal in a method that does not generate dust. Do not

sweep. Damp mop any residue.

Section 7: Handling and Storage

Precautions for safe handling:

Handling:   Follow strict hygiene practices. Containers of this material may be hazardous when empty since they retain

product residues (dust, solids); observe all warnings and precautions listed for the product. Avoid contact with skin

and eyes. Avoid formation of dust and aerosols. Provide appropriate exhaust ventilation at places where dust is

formed.

Conditions for safe storage, including any incompatibilities:

Keep in a tightly closed container, stored in a cool, dry, ventilated area. Protect against physical damage and moisture.

Isolate from any source of heat or ignition. Avoid storage on wood floors. Separate from incompatibles, combustibles,

organic or other readily oxidizable materials. Isolate from any source of heat or ignition. Do not use or store on porous

work surfaces (wood, unsealed concrete, etc.). Protect from light.

Section 8: Exposure Controls/ Personal Protection

Control parameters:

Occupational exposure limits:

OSHA Acceptable Ceiling Concentration   Mercury and mercury compounds: 0.1 mg/m 3   (TWA), skin.

ACGIH Threshold Limit Value (TLV)   Inorganic and metallic mercury, as Hg: 0.025 mg/m 3   (TWA) skin, A4 not

classifiable as a human carcinogen.

ACGIH Biological Exposure Indices   Total inorganic mercury in urine (preshift): 35 ug/g creatinine; total

inorganic mercury in blood (end of shift): 15 ug/l.

Exposure controls:

Appropriate engineering controls:   A system of local and/or general exhaust is recommended to keep employee

exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control

the emissions of the contaminant at its source, preventing dispersion of it into the general work area.

Individual protection measures, such as personal protective equipment:

Personal Respirators:   If the exposure limit is exceeded and engineering controls are not feasible, a NIOSH-approved

full facepiece particulate respirator (NIOSH-type N100 filters) may be worn for up to 50 times the exposure limit or

the maximum use concentration specified by the appropriate regulatory agency or respirator supplier, whichever is

lowest. If oil particles (e.g. lubricants, cutting fluids. glycerine, etc.) are present, use a NIOSH-type R or P filter. For

emergencies or instances where the exposure levels are not known, use a full-facepiece positive-pressure, air-supplied

respirator. WARNING: Air-purifying respirators do not protect workers in oxygen deficient atmospheres.

Skin Protection:   Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as

appropriate, to prevent skin contact.

Eye Protection:   Use chemical safety goggles and/or full-face shield where dusting or splashing of solutions is possible.

Maintain eye wash fountain and quick drench facilities in work area.

Page 5

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 5/10

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Solid (powder).

Colour:   Orange-red / orange-yellow.

Odour:   Odourless.

Odour threshold:   No data available.

pH (concentration):   Not applicable.

Melting point/range (° C):   500   °C

Boiling point/range (° C):   Not applicable.

Flash point (° C):   No data available.

Evaporation rate:   0.6

Flammability (solid, gas):   Non-flammable.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour pressure (20   ° C):   No data available.

Vapour density:   No data available.

Relative densi ty (25 ° C):   11.14

Water solubility (g/L ) at 20 ° C:   Insoluble.

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Reactivity & Chemical stability:   Stable under ordinary conditions of use and storage. Decomposes on exposure to light

into mercury and oxygen.

Conditions to avoid:   Light, heat, incompatibles.

Incompatible materials to avoid:   Reacts with reducing materials. Incompatible with chlorine, hydrazine hydrate,

hydrogen peroxide, hydrogen trisulfide, hypophosphorous acid, iodine plus methanol plus ethanol, magnesium,

phosphorus, phospham, sodium potassium alloy, sulphur, acetyl nitrate, butadiene, hydrocarbons, sulphur chloride,

methanethiol.

Hazardous Decomposition Products:   Emits toxic mercury vapours.

Hazardous Polymerization:   This substance does not polymerize.

Se ction 11: Toxicological Information

Information on toxicological effects:

Fatal if swallowed, in contact with skin, or inhaled.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Page 6

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 6/10

Not classified based on available information.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Not classified based on available information.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

May cause damage to organs through prolonged or repeated exposure.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:   Very toxic to aquatic life with long lasting effects. The LC50/96-hour values for fish are less than 1 mg/l.

Persistence and degradability:   No data available.

Bioaccumulative potential:   This material has an experimentally-determined bioconcentration factor (BCF) of greater

than 100. This material is expected to significantly bioaccumulate.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   No data available.

Other adverse effects:   No data available.

Section 13: Disposal Considerations

Product:   Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to an

RCRA-approved waste facility. Processing, use or contamination of this product may change the waste management

options. State and local disposal regulations may differ from federal disposal regulations. Dispose of unused contents

in accordance with federal, state and local requirements.

Contaminated Packaging:   Dispose of container in accordance with federal, state and local requirements.

Section 14: Transport Information

UN number:   1641

UN proper shipping name:   MERCURY OXIDE

Transport hazard class:   Class 6.1

Transport hazard labels:

Packing group:   II

Marine pollutant:   Yes.

Page 7

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 7/10

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use as

amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and mutagens

at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given birth or

are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical agents

at work.

Always applicable.

Page 8

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 8/10

Directive 94/33/EC on the protection of young people at work

Not listed.

Additional Information:

SARA 302 Components:   The following components are subject to reporting levels established by SARA Title III, Section

302: Mercury monoxide / CAS No. 21908-53-2 / Revision Date 2007-07-01.

SARA 313 Components:   This material does not contain any chemical components with known CAS numbers that

exceed the threshold (De Minimis) reporting levels established by SARA Title III, Section 313.

SARA 311/312 Hazards:   Acute Health Hazard, Chronic Health Hazard.

Massachusetts Right to Know Components:   Mercury monoxide / CAS No. 21908-53-2 / Revision Date 2007-07-01

Pennsylvania Right to Know Components:   Mercury monoxide / CAS No. 21908-53-2 / Revision Date 2007-07-01

New Jersey Right to Know Components:   Mercury monoxide / CAS No. 21908-53-2 / Revision Date 2007-07-01

California Prop. 65 Components:   WARNING: This product contains a chemical known to the State of California to

cause birth defects or other reproductive harm: Mercury monoxide / CAS No. 21908-53-2 / Revision Date 2007-07-01

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other Information

List of abbreviations:

ACGIH American Conference of Governmental Industrial Hygienists

ADR European Agreement Concerning the International Carriage of Dangerous Goods by Road

ALARA As Low As Is Reasonably Achievable

AMU Atomic Mass Unit

ANSI American National Standards Institute

BLS Basic Life Support

CAM Continuous Air Monitor

CAS Chemical Abstracts Service (division of the American Chemical Society)

CEN European Committee for Standardization

CERCLA Comprehensive Environmental Response Compensation and Liability Act

CLP Classification, Labelling and Packaging (European Union)

CPR Controlled Products Regulations (Canada)

CWA Clean Water Act (USA)

DAC Derived Air Concentration (USA)

DOE United States Department of Energy (USA)

DOT United States Department of Transportation (USA)

DSL Domestic Substances List (Canada)

EC50 Half Maximal Effective Concentration

EINECS European Inventory of Existing Commercial Chemical Substances

EHS Environmentally Hazardous Substance

ELINCS European List of Notified Chemical Substances

EMS Emergency Response Procedures for Ships Carrying Dangerous Goods

EPA Environmental Protection Agency (USA)

EPCRA Emergency Planning and Community Right-To-Know Act (EPCRA) of 1986

GHS Globally Harmonized System

HMIS Hazardous Materials Identification System (USA)

IARC International Agency for Research on Cancer

IATA International Air Transport Association

Page 9

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 9/10

IBC Intermediate Bulk Containers

ICAO International Civil Aviation Organization

IDLH Immediately Dangerous to Life or Health

IMDG International Maritime Code for Dangerous Goods

LC50 Lethal concentration, 50 percent

LD50 Lethal dose, 50 percent

LDLO Lethal Dose Low

LOEC Lowest-Observed-Effective Concentration

MARPOL International Convention for the Prevention of Pollution from Ships

MSHA Mine Safety and Health Administration (USA)

NCRP National Council on Radiation Protection & Measurements (USA)

NDSL Non-Domestic Substances List (Canada)

NFPA National Fire Protection Association (USA)

NIOSH National Institute for Occupational Safety and Health (USA)

NOEC No Observed Effect Concentration

N.O.S. Not Otherwise Specified

NRC Nuclear Regulatory Commission (USA)

NTP National Toxicology Program (USA)

OSHA Occupational Safety and Health Administration (USA)

PBT Persistent Bioaccumulative and Toxic Chemical

PEL Permissible Exposure Limit

PIH Poisonous by Inhalation Hazard

RCRA Resource Conservation and Recovery Act (USA)

RCT Radiation Control Technician

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals (Europe)

RID Regulations Concerning the International Transport of Dangerous Goods by Rail

RTECS Registry of Toxic Effects of Chemical Substances

SARA Superfund Amendments and Reauthorization Act (USA)

TDG Transportation of Dangerous Goods (Canada)

TIH Toxic by Inhalation Hazard

TLV Threshold Limit Value

TPQ Threshold Planning Quantity

TSCA Toxic Substances Control Act

TWA Time Weighted Average

UN United Nations (Number)

VOC Volatile Organic Compound

vPvB Very Persistent Very Bioaccumulative Chemical

WGK Wassergefährdungsklassen (Germany: Water Hazard Classes)

WHMIS Workplace Hazardous Materials Information System

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

H300 Fatal if swallowed.

H310 Fatal in contact with skin.

H330 Fatal if inhaled.

H373 May cause damage to organs through prolonged or repeated exposure.

H400 Very toxic to aquatic life.

Page 10

Safety Data Sheet for Mercury Oxide,

Enriched Mercury Oxide

According to ISO 11014:2010

First Print Date: 05-Mar-2015

Revision Date: 20-Jan-2020

Version: 1.1.1.

Safety Data Sheet (SDS) for Mercury Oxide, Enriched Mercury Oxide   https://www.buyisotope.com   Page 10/10

H410 Very toxic to aquatic life with long lasting effects.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.