OREAS 146 rare-earth and yttrium certified reference material (covered by OREAS RADA SDS) SDS

Publisher: Ore Research & Exploration Pty Ltd (OREAS)

Source: https://resds.com/document/oreas-rada-reference-materials-including-oreas-146-en-9a70721e67

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Page 1 of 6

Safety Data Sheet

according to WHS Regulations

Print date: 05.06.2024                                                             Revision date: 31.05.2024

1 Identification

Product Name: OREAS (RADA)

Other Means of Identification: Mixture

Other Name:

All products are branded 'OREAS' followed by a number ± letter

100a, 101a, 101b, 102a, 104, 105, 106, 120, 121, 122, 123, 124, 146

Recommended Use of the Chemical and Restriction on Use: Reference material

Details of Manufacturer or Importer:

Ore Research & Exploration Pty Ltd

37A Hosie Street

Bayswater North VIC 3153

Phone Number: 03 9729 0333

Emergency telephone number: 03 9729 0333

2 Hazard(s) Identification

Hazardous Nature:

Classified as Hazardous according to the Globally Harmonised System of Classification and Labelling of

Chemicals (GHS) and Safe Work Australia criteria.

Not classified as Dangerous Goods according to the Australian Code for the Transport of Dangerous Goods by

Road and Rail (7th edition), IATA and IMDG/IMSBC.

Skull and crossbones

Acute Toxicity (Oral) 3     H301 Toxic if swallowed.

Health hazard

Carcinogenicity 1A         H350i May cause cancer by inhalation.

STOT RE 1             H372 Causes damage to organs through prolonged or repeated exposure.

Acute Toxicity (Inhalation) 4 H332 Harmful if inhaled.

Aquatic Chronic 3        H412 Harmful to aquatic life with long lasting effects.

Signal Word Danger

Hazard Statements

H301 Toxic if swallowed.

H332 Harmful if inhaled.

H350i May cause cancer by inhalation.

H372 Causes damage to organs through prolonged or repeated exposure.

H412 Harmful to aquatic life with long lasting effects.

Precautionary Statements

P201      Obtain special instructions before use.

P202     Do not handle until all safety precautions have been read and understood.

P260     Do not breathe dust/fume/gas/mist/vapours/spray.

P264     Wash thoroughly after handling.

P270     Do not eat, drink or smoke when using this product.

(Contd. on page 2)

58.0.1

Page 2

Page 2 of 6

Safety Data Sheet

according to WHS Regulations

Print date: 05.06.2024                                                             Revision date: 31.05.2024

Product Name: OREAS (RADA)

(Contd. of page 1)

P271     Use only outdoors or in a well-ventilated area.

P273      Avoid release to the environment.

P280     Wear protective gloves/protective clothing/eye protection/face protection.

P301+P310 IF SWALLOWED: Immediately call a POISON CENTER/ doctor.

P321       Specific treatment (see on this label).

P330      Rinse mouth.

P304+P340 IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

P308+P313 IF exposed or concerned: Get medical advice/attention.

P312       Call a POISON CENTER/doctor if you feel unwell.

P314      Get medical advice/attention if you feel unwell.

P405       Store locked up.

P501      Dispose of contents/container in accordance with local/regional/national/international regulations.

3 Composition and Information on Ingredients

Chemical Characterization: Mixtures

Description: Mixture of substances listed below with nonhazardous additions.

Hazardous Components:

CAS: 7631-86-9 Silica                                                            30-82%

Carcinogenicity 1A, H350i; STOT RE 1, H372

CAS: 7440-50-8 Copper                                                              0.01-<3%

Aquatic Chronic 2, H411

CAS: 7440-61-1 Uranium                                                           <0.2%

Acute Toxicity (Oral) 1, H300; Acute Toxicity (Inhalation) 1, H330;  STOT

RE 2, H373

Non Hazardous Components:

CAS: 1309-37-1   Iron(III) oxide                                                       1-10%

CAS: 12001-26-2 Potassium Aluminum Silicate                                          1-10%

4 First Aid Measures

Inhalation: If inhaled, remove to fresh air. Seek medical attention if breathing problems develop.

Skin Contact:

In case of skin contact, immediately remove contaminated clothing and wash affected areas with water and

soap. Seek medical attention if irritation occurs.

Eye Contact:

In case of eye contact, rinse with water for several minutes, including under eyelids. Remove contact lenses, if

present and easy to do. Continue rinsing for at least 15 minutes. Seek medical attention if irritation persists.

Ingestion:

If swallowed, do not induce vomiting. Immediately rinse mouth with water. Never give anything by mouth to an

unconscious person. Seek immediate medical attention.

Symptoms Caused by Exposure:

Inhalation: Harmful if inhaled. May cause cancer by inhalation. May cause respiratory irritation.

Skin Contact: May cause skin irritation.

Eye Contact: May cause eye irritation.

Ingestion: Toxic if swallowed. May cause gastrointestinal irritation, nausea, diarrhoea and vomiting.

5 Fire Fighting Measures

Suitable Extinguishing Media: Use fire extinguishing methods suitable to surrounding conditions.

Specific Hazards Arising from the Chemical:

Hazardous combustion products include toxic gases.

Product is not flammable.

Containers close to fire should be removed only if safe to do so. Use water spray to cool fire exposed

containers.

(Contd. on page 3)

58.0.1

Page 3

Page 3 of 6

Safety Data Sheet

according to WHS Regulations

Print date: 05.06.2024                                                             Revision date: 31.05.2024

Product Name: OREAS (RADA)

(Contd. of page 2)

Prevent run-off from fire fighting entering drains or water courses.

Special Protective Equipment and Precautions for Fire Fighters:

When fighting a major fire wear self-contained breathing apparatus and protective equipment.

6 Accidental Release Measures

Personal Precautions, Protective Equipment and Emergency Procedures:

Wear approved dust/particulate filter respirator and full protective clothing. Evacuate all non-essential

personnel from affected area. Do not breathe dust. Ensure adequate ventilation. Avoid generating dust.

Environmental Precautions:

In the event of a major spill, prevent spillage from entering drains or water courses.

Inform respective authorities in case of seepage into water course or sewage system.

Methods and Materials for Containment and Cleaning Up:

Stop leak if safe to do so and sweep granules into a pile and shovel into drums for subsequent disposal. Avoid

generating dust. Provide adequate ventilation. Decontaminate spill area with detergent and water.

7 Handling and Storage

Precautions for Safe Handling:

Use of safe work practices are recommended to avoid eye or skin contact and inhalation of dust.

Food, beverages and tobacco products should not be stored or consumed where this material is in use. Always

wash hands before smoking, eating, drinking or using the toilet. Wash contaminated clothing and other

protective equipment before storage or re-use. Provide eyewash fountains and safety showers in close

proximity to points of potential exposure.

Conditions for Safe Storage:

Store in a cool, dry and well ventilated area. Keep container tightly closed when not in use. Keep away from

strong oxidising agents such as fluorine, bromine trifluoride, chlorine trifluoride, manganese trifluoride, oxygen

difluoride, and ozone; acids such as hydrofluoric acid, and hydrochloric acid; strong alkalis; vinyl acetate, and

xenon hexafluoride.

8 Exposure Controls and Personal Protection

Exposure Standards:

CAS: 7631-86-9 Silica

WES TWA: 2 mg/m³

CAS: 1309-37-1 Iron(III) oxide

WES TWA: 5 mg/m³

CAS: 12001-26-2 Potassium Aluminum Silicate

WES TWA: 2.5 mg/m³

CAS: 7440-50-8 Copper

WES TWA: 1* 0.2** mg/m³

*dust & mists (as Cu) **fume

CAS: 7440-61-1 Uranium

WES STEL: 0.6 mg/m³

TWA: 0.2 mg/m³

as U

Engineering Controls:

Ensure adequate ventilation of the working area, keeping airborne concentrations below occupational exposure

standards.

Respiratory Protection:

Where an inhalation risk exists, wear approved particulate respirator (filter type P). At high dust levels, wear a

powered air purifying respirator (PAPR) with P3 filter or an air-line respirator or a full-face P3 (particulate)

respirator. See Australian/New Zealand Standards AS/NZS 1715 and 1716 for more information.

(Contd. on page 4)

58.0.1

Page 4

Page 4 of 6

Safety Data Sheet

according to WHS Regulations

Print date: 05.06.2024                                                             Revision date: 31.05.2024

Product Name: OREAS (RADA)

(Contd. of page 3)

Skin Protection:

Protective gloves. See Australian/New Zealand Standard AS/NZS 2161 for more information.

When selecting gloves for use against certain chemicals, the degradation resistance, permeation rate and

permeation breakthrough time should be considered.

Occupational protective clothing (depending on conditions in which it has to be used, in particular as regards

the period for which it is worn, which shall be determined on the basis of the seriousness of the risk, the

frequency of exposure to the risk, the characteristics of the workstation of each worker and the performance of

the protective clothing). See Australian/New Zealand Standard AS/NZS 4501 for more information.

Eye and Face Protection:

Eye and face protectors for protection against dust. See Australian/New Zealand Standard AS/NZS 1337 for

more information.

9 Physical and Chemical Properties

Appearance:

Form:                          Powder

Colour:                            Pale grey to tan to dark grey

Odour:                         None

Odour Threshold:                No information available

pH-Value:                           Not applicable

Melting point/freezing point:        No information available

Initial Boiling Point/Boiling Range:   No information available

No information available

Flash Point:                         Not applicable

Flammability (solid, gas):          No information available

Auto-ignition Temperature:         No information available

Decomposition Temperature:       No information available

Explosion Limits:

Lower:                      No information available

Upper:                       No information available

Vapour Pressure:                    Not applicable

Density:                       No information available

Evaporation Rate:                   Not applicable

Solubility in Water:                     Insoluble

Partition Coefficient (n-octanol/water): No information available

Viscosity:                      No information available

10 Stability and Reactivity

Possibility of Hazardous Reactions: No dangerous reactions known under conditions of normal use.

Chemical Stability: Stable at ambient temperature and under normal conditions of storage and use.

Conditions to Avoid: No further relevant information available.

Incompatible Materials:

Strong oxidising agents such as fluorine, bromine trifluoride, chlorine trifluoride, manganese trifluoride, oxygen

difluoride, and ozone; acids such as hydrofluoric acid, and hydrochloric acid; strong alkalis; vinyl acetate, and

xenon hexafluoride.

Hazardous Decomposition Products: Toxic gases.

(Contd. on page 5)

58.0.1

Page 5

Page 5 of 6

Safety Data Sheet

according to WHS Regulations

Print date: 05.06.2024                                                             Revision date: 31.05.2024

Product Name: OREAS (RADA)

(Contd. of page 4)

11 Toxicological Information

Toxicity:

LD50/LC50 Values:

CAS: 7631-86-9 Silica

Oral LD50 10,000 mg/kg (Rattus norvegicus (rat))

CAS: 1309-37-1 Iron(III) oxide

Oral LD50 >15,000 mg/kg (Rattus norvegicus (rat))

CAS: 7440-50-8 Copper

Oral LD50 >2,000 mg/kg (Rattus norvegicus (rat))

Acute Health Effects

Inhalation: Harmful if inhaled. May cause cancer by inhalation. May cause respiratory irritation.

Skin: May cause skin irritation.

Eye: May cause eye irritation.

Ingestion: Toxic if swallowed. May cause gastrointestinal irritation, nausea, diarrhoea and vomiting.

Skin Corrosion / Irritation: Based on classification principles, the classification criteria are not met.

Serious Eye Damage / Irritation: Based on classification principles, the classification criteria are not met.

Respiratory or Skin Sensitisation: Based on classification principles, the classification criteria are not met.

Germ Cell Mutagenicity: Based on classification principles, the classification criteria are not met.

Carcinogenicity:

Crystalline silica and uranium are classified by IARC as Group 1 - Carcinogenic to humans.

May cause cancer by inhalation.

Reproductive Toxicity: Based on classification principles, the classification criteria are not met.

Specific Target Organ Toxicity (STOT) - Single Exposure:

Based on classification principles, the classification criteria are not met.

Specific Target Organ Toxicity (STOT) - Repeated Exposure:

Causes damage to organs through prolonged or repeated exposure.

Aspiration Hazard: Based on classification principles, the classification criteria are not met.

Chronic Health Effects: No data associated with long term health effects.

Existing Conditions Aggravated by Exposure: No data available.

12 Ecological Information

Ecotoxicity:

Aquatic toxicity:

Harmful to aquatic life with long lasting effects.

CAS: 7440-50-8 Copper

EC50/48 h 0.792 mg/l (Daphnia magna (water flea))

EC50/72 h 0.333 mg/l (Algae)

LC50/96 h 0.0068-0.0156 mg/l (Pimephales promelas (fathead minnow))

Persistence and Degradability: No data available on finished product.

Bioaccumulative Potential: No data available on finished product.

Mobility in Soil: No data available on finished product.

(Contd. on page 6)

58.0.1

Page 6

Page 6 of 6

Safety Data Sheet

according to WHS Regulations

Print date: 05.06.2024                                                             Revision date: 31.05.2024

Product Name: OREAS (RADA)

(Contd. of page 5)

Other adverse effects: No further relevant information available.

13 Disposal Considerations

Disposal Methods and Containers: Dispose according to applicable local and state government regulations.

Special Precautions for Landfill or Incineration:

Please consult your state Land Waste Management Authority for more information.

14 Transport Information

UN Number            Not regulated

Proper Shipping Name  Not regulated

Dangerous Goods Class Not regulated

Packing Group:         Not regulated

15 Regulatory Information

Australian Inventory of Industrial Chemicals: All ingredients are listed.

Standard for the Uniform Scheduling of Medicines and Poisons (SUSMP) - Poison Schedule:

Not a scheduled poison.

16 Other Information

Date of Preparation or Last Revision: 31.05.2024

Prepared by: MSDS.COM.AU Pty Ltd                    www.msds.com.au

Abbreviations and acronyms:

GHS: Globally Harmonised System of Classification and Labelling of Chemicals

CAS: Chemical Abstracts Service (division of the American Chemical Society)

LC50: Lethal concentration, 50 percent

LD50: Lethal dose, 50 percent

IARC: International Agency for Research on Cancer

STEL: Short Term Exposure Limit

TWA: Time Weighted Average

NES: National Exposure Standard (Safe Work Australia - Workplace Exposure Standards For Airborne Contaminants)

Acute Toxicity (Oral) 1: Acute toxicity – Category 1

Acute Toxicity (Oral) 3: Acute toxicity – Category 3

Acute Toxicity (Inhalation) 4: Acute toxicity – Category 4

Carcinogenicity 1A: Carcinogenicity – Category 1Ai

STOT RE 1: Specific target organ toxicity (repeated exposure) – Category 1

STOT RE 2: Specific target organ toxicity (repeated exposure) – Category 2

Aquatic Chronic 2: Hazardous to the aquatic environment, long-term (Chronic). Category 2

Aquatic Chronic 3: Hazardous to the aquatic environment, long-term (Chronic). Category 3

Disclaimer

This SDS is prepared in accord with the Safe Work Australia document “Code of Practice for the Preparation

of Safety Data Sheets for Hazardous Chemicals - July 2020”.

The information contained in this safety data sheet is provided in good faith and is believed to be accurate at

the date of issuance. Ore Research & Exploration Pty Ltd makes no representation of the accuracy or

comprehensiveness of the information and to the full extent allowed by law excludes all liability for any loss or

damage related to the supply or use of the information in this material safety data sheet. MSDS.COM.AU Pty

Ltd is not in a position to warrant the accuracy of the data herein. The user is cautioned to make their own

determinations as to the suitability of the information provided to the particular circumstances in which the

product is used.

58.0.1