Samarium (II) iodide, 0.1M in THF ALFAA22360 SDS

Publisher: Avocado Research Chemicals Ltd. (Part of Thermo Fisher Scientific)

Source: https://resds.com/document/samarium-ii-iodide-0-1m-in-thf-sds-avocado-research-chemicals-ltd-part-of-thermo-fisher-en-b9255a6370

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Page 1 / 9             SAFETY DATA SHEET

Creation Date 20-Aug-2009

Revision Date 03-Aug-2026

Version 5

ALFAA22360

Samarium (II) iodide, 0.1M in THF

SECTION 1. IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

产品说明:产品说明:产品说明:产品说明:                  碘化钐(II)碘化钐(II)碘化钐(II)碘化钐(II)

Product Description:            Samarium (II) iodide, 0.1M in THF

Cat No. :                        22360

Molecular Formula                    I2 Sm

Supplier                         Avocado Research Chemicals Ltd.

(Part of Thermo Fisher Scientific)

Shore Road, Heysham

Lancashire, LA3 2XY,

United Kingdom

Office Tel: +44 (0) 1524 850506

Office Fax: +44 (0) 1524 850608

Emergency Telephone Number     For information US call: 001-800-227-6701 / Europe call: +32 14 57 52 11

Emergency Number US:001-201-796-7100 / Europe: +32 14 57 52 99

CHEMTREC Tel. No. US:001-800-424-9300 / Europe:001-703-527-3887

E-mail address                    begel.sdsdesk@thermofisher.com

Recommended Use                 Laboratory chemicals.

Uses advised against           No Information available

SECTION 2. HAZARD IDENTIFICATION

Physical State                      Appearance                          Odor

Liquid                              Clear Colorless                 No information available

Emergency Overview

Highly flammable liquid and vapor. Causes serious eye irritation. Suspected of causing cancer. May cause respiratory irritation.

Harmful if swallowed. May cause drowsiness and dizziness. May form explosive peroxides. Moisture sensitive. Air sensitive.

Classification of the substance or mixture

Flammable liquids.                                                                Category 2

Acute Oral Toxicity                                                                Category 4

Serious Eye Damage/Eye Irritation                                                  Category 2

Carcinogenicity                                                                   Category 2

Specific target organ toxicity - (single exposure)                                       Category 3

Label Elements

______________________________________________________________________________________________

Page 1

Page 2

ALFAA22360               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 2 / 9

Samarium (II) iodide, 0.1M in THF

______________________________________________________________________________________________

Signal Word                    Danger

Hazard Statements

H225 - Highly flammable liquid and vapor

H319 - Causes serious eye irritation

H351 - Suspected of causing cancer

H335 - May cause respiratory irritation

H302 - Harmful if swallowed

H336 - May cause drowsiness or dizziness

Precautionary Statements

Prevention

P201 - Obtain special instructions before use

P202 - Do not handle until all safety precautions have been read and understood

P210 - Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking

P240 - Ground and bond container and receiving equipment

P242 - Use non-sparking tools

P243 - Take action to prevent static discharges

P260 - Do not breathe dust/fume/gas/mist/vapors/spray

P264 - Wash face, hands and any exposed skin thoroughly after handling

P270 - Do not eat, drink or smoke when using this product

P271 - Use only outdoors or in a well-ventilated area

P280 - Wear eye protection/ face protection

Response

P301 + P312 - IF SWALLOWED: Call a POISON CENTER or doctor if you feel unwell

P303 + P361 + P353 - IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water or shower

P304 + P340 - IF INHALED: Remove person to fresh air and keep comfortable for breathing

P305 + P351 + P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to

do. Continue rinsing

P308 + P313 - IF exposed or concerned: Get medical advice/attention

P330 - Rinse mouth

P370 + P378 - In case of fire: Use dry sand, dry chemical or alcohol-resistant foam to extinguish

Storage

P403 + P233 - Store in a well-ventilated place. Keep container tightly closed

Disposal

P501 - Dispose of contents/ container to an approved waste disposal plant

Physical and Chemical Hazards

Vapors may cause flash fire or explosion. Highly flammable. May form explosive peroxides.

Health Hazards

Causes serious eye irritation. Suspected of causing cancer. May cause respiratory irritation. Harmful if swallowed. May cause

drowsiness or dizziness.

Environmental hazards

Contains no substances known to be hazardous to the environment or not degradable in waste water treatment plants.  . Will likely

be mobile in the environment due to its water solubility. The product is water soluble, and may spread in water systems.

Toxic to terrestrial vertebrates. This product does not contain any known or suspected endocrine disruptors.

SECTION 3. COMPOSITION/INFORMATION ON INGREDIENTS

Component                              CAS No                Weight %

Tetrahydrofuran                                         109-99-9                   96

Samarium iodide (SmI2)                                   32248-43-4                  4

SECTION 4. FIRST AID MEASURES

General Advice

If symptoms persist, call a physician.

Eye Contact

______________________________________________________________________________________________

Page 2

Page 3

ALFAA22360               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 3 / 9

Samarium (II) iodide, 0.1M in THF

______________________________________________________________________________________________

Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Get medical attention.

Skin Contact

Wash off immediately with plenty of water for at least 15 minutes. If skin irritation persists, call a physician.

Inhalation

Remove to fresh air. If not breathing, give artificial respiration. Get medical attention if symptoms occur.

Ingestion

Clean mouth with water and drink afterwards plenty of water.

Most important symptoms and effects

None reasonably foreseeable. Symptoms of overexposure may be headache, dizziness, tiredness, nausea and vomiting:

Inhalation of high vapor concentrations may cause symptoms like headache, dizziness, tiredness, nausea and vomiting: Causes

central nervous system depression

Self-Protection of the First Aider

Ensure that medical personnel are aware of the material(s) involved, take precautions to protect themselves and prevent spread of

contamination.

Notes to Physician

Treat symptomatically. Symptoms may be delayed.

SECTION 5. FIRE-FIGHTING MEASURES

Suitable Extinguishing Media

Water spray, carbon dioxide (CO2), dry chemical, alcohol-resistant foam. Water mist may be used to cool closed containers.

Extinguishing media which must not be used for safety reasons

No information available.

Specific Hazards Arising from the Chemical

Flammable. Containers may explode when heated. Vapors may form explosive mixtures with air. Vapors may travel to source of

ignition and flash back. May form explosive peroxides.

Protective Equipment and Precautions for Firefighters

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full

protective gear.

SECTION 6. ACCIDENTAL RELEASE MEASURES

Personal Precautions

Ensure adequate ventilation. Use personal protective equipment as required. Remove all sources of ignition. Take precautionary

measures against static discharges.

Environmental Precautions

Should not be released into the environment.

Methods for Containment and Clean Up

Soak up with inert absorbent material. Keep in suitable, closed containers for disposal. Remove all sources of ignition. Use

spark-proof tools and explosion-proof equipment.

Refer to protective measures listed in Sections 8 and 13.

SECTION 7. HANDLING AND STORAGE

Handling

______________________________________________________________________________________________

Page 3

Page 4

ALFAA22360               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 4 / 9

Samarium (II) iodide, 0.1M in THF

______________________________________________________________________________________________

Ensure adequate ventilation. Wear personal protective equipment/face protection. Avoid ingestion and inhalation. Do not get in

eyes, on skin, or on clothing. If peroxide formation is suspected, do not open or move container. Keep away from open flames, hot

surfaces and sources of ignition. Use only non-sparking tools. To avoid ignition of vapors by static electricity discharge, all metal

parts of the equipment must be grounded. Take precautionary measures against static discharges.

Storage

Keep containers tightly closed in a dry, cool and well-ventilated place. Keep away from heat, sparks and flame. Shelf life 12

months. May form explosive peroxides on prolonged storage. Containers should be dated when opened and tested periodically for

the presence of peroxides. Should crystals form in a peroxidizable liquid, peroxidation may have occurred and the product should

be considered extremely dangerous. In this instance, the container should only be opened remotely by professionals.

Specific Use(s)

Use in laboratories

SECTION 8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Control Parameters

Component                China              ACGIH TLV           OSHA PEL              NIOSH

Tetrahydrofuran         TWA: 300 mg/m3          TWA: 50 ppm         (Vacated) TWA: 200 ppm       IDLH: 2000 ppm

STEL: 100 ppm       (Vacated) TWA: 590 mg/m3      TWA: 200 ppm

Skin             (Vacated) STEL: 250 ppm      TWA: 590 mg/m3

(Vacated) STEL: 735 mg/m3      STEL: 250 ppm

TWA: 200 ppm          STEL: 735 mg/m3

TWA: 590 mg/m3

Component            Hong Kong                Thailand            European Union       The United Kingdom

Tetrahydrofuran          TWA: 200 ppm          TWA: 200 ppm         TWA: 50 ppm (8h)       STEL: 100 ppm 15 min

TWA: 590 mg/m3                            TWA: 150 mg/m3 (8h)     STEL: 300 mg/m3 15 min

STEL: 250 ppm                               STEL: 100 ppm (15min)      TWA: 50 ppm 8 hr

STEL: 737 mg/m3                              STEL: 300 mg/m3 (15min)    TWA: 150 mg/m3 8 hr

Skin                       Skin

Legend

ACGIH - American Conference of Governmental Industrial Hygienists

OSHA - Occupational Safety and Health Administration

NIOSH: NIOSH - National Institute for Occupational Safety and Health

Monitoring methods

BS EN 14042:2003 Title Identifier: Workplace atmospheres. Guide for the application and use of procedures for the assessment of

exposure to chemical and biological agents. MDHS70 General methods for sampling airborne gases and vapours MDHS 88

Volatile organic compounds in air. Laboratory method using diffusive samplers, solvent desorption and gas chromatography MDHS

96 Volatile organic compounds in air - Laboratory method using pumped solid sorbent tubes, solvent desorption and gas

chromatography

Exposure Controls

Engineering Measures

Ensure that eyewash stations and safety showers are close to the workstation location. Ensure adequate ventilation, especially in

confined areas. Use explosion-proof electrical/ventilating/lighting equipment. Wherever possible, engineering control measures

such as the isolation or enclosure of the process, the introduction of process or equipment changes to minimise release or contact,

and the use of properly designed ventilation systems, should be adopted to control hazardous materials at source.

Personal protective equipment

Eye Protection                Goggles (European standard - EN 166)

Hand Protection                  Protective gloves

Glove material     Breakthrough time  Glove thickness   EU standard             Glove comments

Butyl rubber      See manufacturers               -          EN 374             (minimum requirement)

recommendations

Neoprene gloves

Inspect gloves before use.

______________________________________________________________________________________________

Page 4

Page 5

ALFAA22360               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 5 / 9

Samarium (II) iodide, 0.1M in THF

______________________________________________________________________________________________

Please observe the instructions regarding permeability and breakthrough time which are provided by the supplier of the gloves.

(Refer to manufacturer/supplier for information)

Ensure gloves are suitable for the task: Chemical compatability, Dexterity, Operational conditions, User susceptibility, e.g.

sensitisation effects, also take into consideration the specific local conditions under which the product is used, such as the danger

of cuts, abrasion.

Remove gloves with care avoiding skin contamination.

Skin and body protection       Long sleeved clothing

Respiratory Protection       When workers are facing concentrations above the exposure limit they must use

appropriate certified respirators.

To protect the wearer, respiratory protective equipment must be the correct fit and be used

and maintained properly

Large scale/emergency use    Use a NIOSH/MSHA or European Standard EN 136 approved respirator if exposure limits

are exceeded or if irritation or other symptoms are experienced

Recommended Filter type: low boiling organic solvent Type AX Brown conforming to

EN371 or Organic gases and vapours filter Type A Brown conforming to EN14387

Small scale/Laboratory use     Use a NIOSH/MSHA or European Standard EN 149:2001 approved respirator if exposure

limits are exceeded or if irritation or other symptoms are experienced.

Recommended half mask:- Valve filtering: EN405; or; Half mask: EN140; plus filter, EN

141

When RPE is used a face piece Fit Test should be conducted

Hygiene Measures                Handle in accordance with good industrial hygiene and safety practice.

Environmental exposure controls  No information available.

SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

Appearance                        Clear Colorless

Physical State                        Liquid

Odor                       No information available

Odor Threshold               No data available

pH                         No information available

Melting Point/Range            No data available

Softening Point                No data available

Boiling Point/Range            No information available

Flash Point                          -17 °C  / 1.4 °F                Method - No information available

Evaporation Rate              No data available

Flammability (solid,gas)            Not applicable                         Liquid

Explosion Limits               No data available

Vapor Pressure                No data available

Vapor Density                 No data available                         (Air = 1.0)

Specific Gravity  / Density          0.9222

Bulk Density                      Not applicable                         Liquid

Water Solubility                      Miscible

Solubility in other solvents       No information available

Partition Coefficient (n-octanol/water)

Component                        log Pow

Tetrahydrofuran                     0.45

Autoignition Temperature        No data available

Decomposition Temperature      No data available

Viscosity                    No data available

Explosive Properties                                               Vapors may form explosive mixtures with air

Oxidizing Properties            No information available

Molecular Formula                    I2 Sm

______________________________________________________________________________________________

Page 5

Page 6

ALFAA22360               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 6 / 9

Samarium (II) iodide, 0.1M in THF

______________________________________________________________________________________________

Molecular Weight                  404.17

SECTION 10. STABILITY AND REACTIVITY

Stability                     May form explosive peroxides. Moisture sensitive. Air sensitive.

Hazardous Reactions            None under normal processing.

Hazardous Polymerization         Hazardous polymerization may occur.

Conditions to Avoid                 Incompatible products. Excess heat. Keep away from open flames, hot surfaces and

sources of ignition. Exposure to moist air or water. Exposure to air.

Materials to avoid                  Strong oxidizing agents.

Hazardous Decomposition Products Carbon monoxide (CO). Carbon dioxide (CO2).

SECTION 11. TOXICOLOGICAL INFORMATION

Product Information

(a) acute toxicity;

Toxicology data for the components

Component                    LD50 Oral                LD50 Dermal             LC50 Inhalation

Tetrahydrofuran                 1650 mg/kg ( Rat )          > 2000 mg/kg (Rabbit)          180 mg/L  ( Rat ) 1 h

53.9 mg/L  ( Rat ) 4 h

(b) skin corrosion/irritation;        Based on available data, the classification criteria are not met

(c) serious eye damage/irritation;   Category 2

(d) respiratory or skin sensitization;

Respiratory                  Based on available data, the classification criteria are not met

Skin                        Based on available data, the classification criteria are not met

Component                      Test method                  Test species                 Study result

Tetrahydrofuran                 Local Lymph Node Assay             mouse                       non-sensitising

109-99-9 ( 96 )           OECD Test Guideline 429

No information available

(e) germ cell mutagenicity;        Based on available data, the classification criteria are not met

Component                      Test method                  Test species                 Study result

Tetrahydrofuran           OECD Test Guideline 476                     in vivo                         negative

109-99-9 ( 96 )                Gene cell mutation               Mammalian

------------------------------------------

OECD Test Guideline 473       ------------------------------------------    ------------------------------------------

Chromosomal aberration assay                 in vitro                         negative

Mammalian

Mutagenic effects have occured in microorganisms

(f) carcinogenicity;                 Category 2

Limited evidence of a carcinogenic effect The table below indicates whether each agency

has listed any ingredient as a carcinogen

Component               EU               UK               Germany               IARC

Tetrahydrofuran                                                                                  Group 2B

______________________________________________________________________________________________

Page 6

Page 7

ALFAA22360               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 7 / 9

Samarium (II) iodide, 0.1M in THF

______________________________________________________________________________________________

(g) reproductive toxicity;          Based on available data, the classification criteria are not met

Component                      Test method             Test species / Duration            Study result

Tetrahydrofuran           OECD Test Guideline 416           Rat 2 Generation          NOAEL = 3,000 ppm

109-99-9 ( 96 )

(h) STOT-single exposure;          Category 3

Results / Target organs          Respiratory system

Central nervous system (CNS)

(i) STOT-repeated exposure;       Based on available data, the classification criteria are not met

Target Organs               None known.

(j) aspiration hazard;             Based on available data, the classification criteria are not met

Symptoms  / effects,both acute and Symptoms of overexposure may be headache, dizziness, tiredness, nausea and vomiting:

delayed                                Inhalation of high vapor concentrations may cause symptoms like headache, dizziness,

tiredness, nausea and vomiting: Causes central nervous system depression

SECTION 12. ECOLOGICAL INFORMATION

Ecotoxicity effects             Do not empty into drains. Do not flush into surface water or sanitary sewer system. Do not

allow material to contaminate ground water system.

Component                Freshwater Fish         Water Flea        Freshwater Algae         Microtox

Tetrahydrofuran            2160 mg/l LC50 = 96 h  EC50 48 h 3485 mg/l

Pimephales promelas  EC50: >10000 mg/L/24h

Leuciscus idus: LC50:

2820 mg/L/48h

Persistence and Degradability

Persistence                     Persistence is unlikely, Miscible with water, based on information available.

Bioaccumulative Potential          Bioaccumulation is unlikely

Component                                 log Pow                           Bioconcentration factor (BCF)

Tetrahydrofuran                                  0.45                            No data available

Mobility in soil                  The product is water soluble, and may spread in water systems Will likely be mobile in the

environment due to its water solubility Highly mobile in soils

Endocrine Disruptor Information

Component            EU - Endocrine Disrupters   EU - Endocrine Disruptors -   Japan - Endocrine Disruptor

Candidate List            Evaluated Substances              Information

Tetrahydrofuran                 Group III Chemical

Persistent Organic Pollutant        This product does not contain any known or suspected substance

Ozone Depletion Potential           This product does not contain any known or suspected substance

SECTION 13. DISPOSAL CONSIDERATIONS

Waste from Residues/Unused      Waste is classified as hazardous. Dispose of in accordance with the European Directives

Products                        on waste and hazardous waste. Dispose of in accordance with local regulations.

______________________________________________________________________________________________

Page 7

Page 8

ALFAA22360               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 8 / 9

Samarium (II) iodide, 0.1M in THF

______________________________________________________________________________________________

Contaminated Packaging           Dispose of this container to hazardous or special waste collection point. Empty containers

retain product residue, (liquid and/or vapor), and can be dangerous. Keep product and

empty container away from heat and sources of ignition.

Other Information              Do not flush to sewer. Waste codes should be assigned by the user based on the

application for which the product was used. Can be landfilled or incinerated, when in

compliance with local regulations.

SECTION 14. TRANSPORT INFORMATION

Road and Rail Transport

UN-No                       UN2056

Proper Shipping Name          TETRAHYDROFURAN

Hazard Class                    3

Packing Group                                     II

IMDG/IMO

UN-No                       UN2056

Proper Shipping Name          TETRAHYDROFURAN

Hazard Class                    3

Packing Group                                     II

IATA

UN-No                       UN2056

Proper Shipping Name          TETRAHYDROFURAN

Hazard Class                    3

Packing Group                                     II

Special Precautions for User      No special precautions required

SECTION 15. REGULATORY INFORMATION

International Inventories

X = listed, China (IECSC), Europe (EINECS/ELINCS/NLP), U.S.A. (TSCA), Canada (DSL/NDSL), Philippines (PICCS), Japan (ENCS), Japan

(ISHL), Australia (AICS), Korea (KECL).

Component        The        List of    IECSC   EINECS   TSCA   DSL   PICCS  ENCS    ISHL    AICS    KECL

Inventory of dangerous

Hazardous  goods GB

Chemicals   12268 -

(2015      2012

Edition)

Tetrahydrofuran      X        X       X      203-726-8    X      X      X      X      X      X     KE-33454

National Regulations

SECTION 16. OTHER INFORMATION

Prepared By                         Health, Safety and Environmental Department

Creation Date                     20-Aug-2009

Revision Date                     03-Aug-2026

______________________________________________________________________________________________

Page 8

Page 9

ALFAA22360               SAFETY DATA SHEET                    Revision Date 03-Aug-2026Page 9 / 9

Samarium (II) iodide, 0.1M in THF

______________________________________________________________________________________________

Revision Summary                Not applicable.

Training Advice

Chemical hazard awareness training, incorporating labelling, Safety Data Sheets (SDS), Personal Protective Equipment (PPE) and

hygiene.

Use of personal protective equipment, covering appropriate selection, compatibility, breakthrough thresholds, care, maintenance, fit

and standards.

First aid for chemical exposure, including the use of eye wash and safety showers.

Fire prevention and fighting, identifying hazards and risks, static electricity, explosive atmospheres posed by vapours and dusts.

Chemical incident response training.

Legend

CAS - Chemical Abstracts Service                          TSCA - United States Toxic Substances Control Act Section 8(b)

Inventory

EINECS/ELINCS - European Inventory of Existing Commercial Chemical DSL/NDSL - Canadian Domestic Substances List/Non-Domestic

Substances/EU List of Notified Chemical Substances                  Substances List

PICCS - Philippines Inventory of Chemicals and Chemical Substances  ENCS - Japanese Existing and New Chemical Substances

IECSC - Chinese Inventory of Existing Chemical Substances          AICS - Australian Inventory of Chemical Substances

KECL - Korean Existing and Evaluated Chemical Substances         NZIoC - New Zealand Inventory of Chemicals

WEL - Workplace Exposure Limit                        TWA - Time Weighted Average

ACGIH - American Conference of Governmental Industrial Hygienists   IARC - International Agency for Research on Cancer

DNEL - Derived No Effect Level                           PNEC - Predicted No Effect Concentration

RPE - Respiratory Protective Equipment                        LD50 - Lethal Dose 50%

LC50 - Lethal Concentration 50%                            EC50 - Effective Concentration 50%

NOEC - No Observed Effect Concentration                 POW - Partition coefficient Octanol:Water

PBT - Persistent, Bioaccumulative, Toxic                       vPvB - very Persistent, very Bioaccumulative

ICAO/IATA - International Civil Aviation Organization/International Air   IMO/IMDG - International Maritime Organization/International Maritime

Transport Association                                          Dangerous Goods Code

ADR - European Agreement Concerning the International Carriage of   MARPOL - International Convention for the Prevention of Pollution from

Dangerous Goods by Road                                         Ships

OECD - Organisation for Economic Co-operation and Development    ATE - Acute Toxicity Estimate

BCF - Bioconcentration factor                           VOC - (Volatile Organic Compound)

Key literature references and sources for data

https://echa.europa.eu/information-on-chemicals

Suppliers safety data sheet, Chemadvisor - LOLI, Merck index, RTECS

Physical hazards              On basis of test data

Health Hazards                       Calculation method

Environmental hazards              Calculation method

Disclaimer

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the

date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage,

transportation, disposal and release and is not to be considered a warranty or quality specification. The information

relates only to the specific material designated and may not be valid for such material used in combination with any other

materials or in any process, unless specified in the text

End of Safety Data Sheet

______________________________________________________________________________________________

Page 9