Xenon-130 SDS

Publisher: Neonest AB

Source: https://resds.com/document/xenon-130-isotope-sds-xe-130-130xe-xenon-enriched-xenon-inert-gas-09040c7cb3

Text extracted from the original PDF. Tables, symbols and OCR may differ. Check the original PDF for safety-critical details.

Page 1

Safety Data Sheet for Xenon,

Enriched Xenon (inert gas)

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 5-Nov-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Xenon, Enriched Xenon (inert gas)   https://www.buyisotope.com   Page 1/7

Section 1: Chemical Product and Company Identification

Product identifier:

Identification as on the label/Trade name:   Xenon, Enriched Xenon (inert gas)

Molecular weight:   131.293

Chemical formula:   Xe

Synonyms : None.

Details of the supplier of the Safety Data Sheet:

Neonest AB

Storgatan 70C, Solna

SE-17152

Sweden

Contact details:

+46-76-219-9731

Emergency telephone numbers:

24-hour Emergency Contact:

Swedish Poisons Center phone: 112   –   “ ask for Poisons Information ”.   "112   –   begär Giftinformation"

Other International Contacts:

CHEMTREC 24-hour: +1-703-741-5500 (US + Worldwide)

NHS: 111 (UK)

Charite: +49 30 450 531 000 (Netherlands)

INTCF: +34 917689800 (Spain)

CapTv: +33 1 40 05 48 48 (France)

Section 2: Hazards Identification

Classification of the substances or mixture:

The mixture is classified according to:   Regulation EC 1272/2008 [EU-GHS/CLP]

Hazard classes/Hazard categories:   Hazard statement:

Pressurized Gas   H280

Label elements:

Hazard pictograms:

Signal Words:   Warning.

Hazard Statements:

H280 Contains gas under pressure; may explode if heated.

Page 2

Safety Data Sheet for Xenon,

Enriched Xenon (inert gas)

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 5-Nov-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Xenon, Enriched Xenon (inert gas)   https://www.buyisotope.com   Page 2/7

Precautionary Statements:

None.

Other hazards:   None known.

Section 3: Composition/Information on Ingredients

Substance/Mixture:   Substance.

Ingredients:

Substance name   (IUPAC/EC)   CAS - No.   Molecular

weight

Concentration

% by weight

Classification

EC1272/2008 EC - No.

Xenon

7440-63-3

131.293   99-100%   Press. Gas (Comp.) H280

231-172-7

For explanation of abbreviations see Section 16.

Section 4: First - Aid Measures

Description of first aid measures:

In case of inhalation:   Move to fresh air. If unconscious, place in recovery position and seek medical advice. If

symptoms persist, call a physician.

In case of skin contact:   If on skin, rinse well with warm water.

In case of eye contact:   Immediately flush eye(s) with plenty of water. Remove contact lenses. Protect unharmed eye.

Keep eye wide open while rinsing. If eye irritation persists, consult a specialist.

Most important symptoms and effects, both acute and delayed:

None known.

Indication of any immediate medical attention and special treatment needed:   Treat symptomatically. Show this

safety data sheet to a physician or emergency room.

Section 5: Fire - Fi ghting Measures

Extinguisher media:

Suitable extinguisher media:   Use extinguishing measures that are appropriate to local circumstances and the

surrounding environment.

Unsuitable extinguishing media:   High volume water jet.

Special hazards arising from the mixture:   Heat from fire can build pressure in the cylinder and cause it to rupture.

Advice for fire-fighters:   Wear self-contained breathing apparatus for firefighting if necessary.

Further information:   Standard procedure for chemical fires. Use extinguishing measures that are appropriate to local

circumstances and the surrounding environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Personal precautions:   Use personal protective equipment. Refer to protective measures listed in Sections 7 and 8.

Page 3

Safety Data Sheet for Xenon,

Enriched Xenon (inert gas)

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 5-Nov-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Xenon, Enriched Xenon (inert gas)   https://www.buyisotope.com   Page 3/7

Environmental precautions:   Prevent product from entering drains. Prevent further leakage or spillage if safe to do so.

Local authorities should be advised if significant spillages cannot be contained.

Methods for containment and cleaning up:

Methods for cleaning up:   Shut off the flow if safe to do so. Ventilate the area.

Reference to other sections:

Treat recovered material as described in the section "Disposal considerations".

Section 7: Handling and Storage

Precautions for safe handling:

Advice on safe handling:   Protect cylinders from damage. Do not drag, roll, or drop cylinders. Check for leaks using

soapy water, do not use a flame. Never attempt to lift a cylinder by the cap. Never insert an object (e.g. screwdriver)

into cap openings, as this may damage the valve and cause a leak. Open valve slowly. If the valve is hard to open,

discontinue use and contact your supplier.

Hygiene measures:   Do not eat, drink or smoke when using this product.

Conditions for safe storage, including incompatibilities:

Requirements for storage areas and containers:   Store and use with adequate ventilation. Firmly secure cylinders

upright to keep them from falling or being knocked over. Screw valve protection cap firmly in place. Store in

temperatures not exceeding 52 °C .

Section 8: Exposure Controls/ Personal Protection

Control parameters:

Occupational exposure limits:   Contains no substances with occupational exposure limit values.

Exposure controls:

Appropriate engineering controls:   No data available.

Individual protection measures, such as personal protective equipment:

Eye protection:   No personal eye protective equipment normally required.

Hand protection:   No personal hand protective equipment normally required. If discomfort is experienced, wear

protective gloves.

Respiratory protection:   No personal respiratory protective equipment normally required.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Appearance (form):   Gas.

Colour:   Colourless.

Odour:   Odourless.

Odour threshold:   No data available.

Molecular Weight:   131.293

pH (concentration):   No data available.

Page 4

Safety Data Sheet for Xenon,

Enriched Xenon (inert gas)

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 5-Nov-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Xenon, Enriched Xenon (inert gas)   https://www.buyisotope.com   Page 4/7

Melting point/range (° C):   No data available.

Boiling point/range   (° C):   -108.11   °C

Freezing point   (°C):   - 111°C

Flash point (° C):   No data available.

Evaporation rate:   No data available.

Flammability (solid, gas):   No data available.

Ignition temperature (° C):   No data available.

Upper/lower flammability/explosive limits:   No data available.

Vapour   pressure (20 ° C):   No data available.

Vapour density:   0.00547 g/ml @ 21.1   °C

Relative densi ty (25 ° C):   No data available.

Water solubility (g/L ) at 20 ° C:   No data available.

n-Octanol/Water partition coefficient : No data available.

Auto-ignition temperature:   No data available.

Decomposition temperature:   No data available.

Viscosity, dynamic (mPa s):   No data available.

Explosive properties:   The substance or mixture is not classified as explosive.

Oxidising properties:   The substance or mixture is not classified as oxidizing.

Section 10: Stability and Reactivity

Reactivity:   No dangerous reaction known under conditions of normal use.

Chemical stability:   Stable under normal conditions.

Possibility of hazardous reactions:   No dangerous reaction known under conditions of normal use.

Conditions to avoid:   No data available .

Incompatible materials:   None.

Hazardous decomposition products:   No decomposition if stored and applied as directed.

Section 11: Toxicological Information

Information on toxicological effects:

Not classified based on available information.

Classification according to GHS (1272/2008/EG, CLP)

Skin corrosion/irritation:

Not classified based on available information.

Serious eye damage/eye irritation:

Not classified based on available information.

Respiratory or skin sensitisation:

Not classified based on available information.

Germ cell mutagenicity:

Not classified based on available information.

Carcinogenicity:

Not classified based on available information.

Reproductive toxicity:

Not classified based on available information.

Specific target organ toxicity   –   single exposure (STOT):

Page 5

Safety Data Sheet for Xenon,

Enriched Xenon (inert gas)

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 5-Nov-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Xenon, Enriched Xenon (inert gas)   https://www.buyisotope.com   Page 5/7

Not classified based on available information.

Specific target organ toxicity (STOT)   –   repeated exposure:

Not classified based on available information.

Aspiration toxicity:

Not classified based on available information.

Section 12: Ecological Information

Toxicity:   No data available.

Persistence and degradability:   No data available.

Bioaccumulative potential:   No data available.

Mobility in soil:   No data available.

Results of PBT& vPvB assessment:   Not relevant.

Other adverse effects:   No data available.

Section 13: Disposal Considerations

Waste treatment methods:   Dispose of in accordance with municipal, provincial and national regulations.

Product/packaging disposal:   Do not re-use empty containers.

Section 14: Transport Information

UN Number:   2036

UN Proper Shipping Name:   XENON

Transport Hazard Classes : Class 2.2

Hazard labels:

Section 15: Regulatory Information

EU regulations:

Regulation (EC) No. 1005/2009 on substances that deplete the ozone layer, Annex I and II, as amended

Not listed.

Regulation (EC) No. 850/2004 on persistent organic pollutants, Annex I

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 1 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 2 as

amended

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex I, Part 3 as

amended

Page 6

Safety Data Sheet for Xenon,

Enriched Xenon (inert gas)

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 5-Nov-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Xenon, Enriched Xenon (inert gas)   https://www.buyisotope.com   Page 6/7

Not listed.

Regulation (EU) No. 649/2012 concerning the export and import of dangerous chemicals, Annex V as

amended

Not listed.

Regulation (EC) No. 166/2006 Annex II Pollutant Release and Transfer Registry

Not listed.

Regulation (EC) No. 1907/2006, REACH Article 59(10) Candidate List as currently published by ECHA

Not listed.

Authorisations:

Regulation (EC) No. 1907/2006, REACH Annex XIV Substances subject to authorization, as amended

Not listed.

Restrictions on use:

Regulation (EC) No. 1907/2006 Annex XVII Substances subject to restriction on marketing and use

Not regulated.

Regulation (EC) No. 1907/2006, REACH Annex XVII Substances subject to restriction on marketing and use

as amended

Not listed.

Directive 2004/37/EC on the protection of workers from the risks related to exposure to carcinogens and

mutagens at work

Not regulated.

Directive 92/85/EEC: on the safety and health of pregnant workers and workers who have recently given

birth or are breastfeeding.

Not regulated.

Other EU regulations:

Directive 2012/18/EU on major accident hazards involving dangerous substances

Not listed.

Directive 98/24/EC on the protection of the health and safety of workers from the risks related to chemical

agents at work.

Always applicable.

Directive 94/33/EC on the protection of young people at work

Not listed.

Other regulations:   The product is classified and labelled in accordance with EC directives or respective national laws.

This Safety Data Sheet complies with the requirements of Regulation (EC) No 1907/2006, as amended.

National regulations:   Follow national regulation for work with chemical agents.

Chemical safety assessment:   No Chemical Safety Assessment has been carried out.

Section 16: Other I nformation

List of abbreviations:

Not available.

References:

Not available.

Full text of any H-statements not written out in full under Sections 2 to 15:

Page 7

Safety Data Sheet for Xenon,

Enriched Xenon (inert gas)

According to ISO 11014:2010

First Print Date: 5-Mar-2015

Revision Date: 5-Nov-2019

Version: 1.1.1.

Safety Data Sheet (SDS) for Xenon, Enriched Xenon (inert gas)   https://www.buyisotope.com   Page 7/7

H280 Contains gas under pressure; may explode if heated.

Revision information:

None.

Training information:

Follow training instructions when handling this material.

Further Information:

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at

the date of its publication. The information given is designed only as a guidance for safe handling, use, processing,

storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The

information relates only to the specific material designated and may not be valid for such material used in

combination with any other materials or in any process, unless specified in the text.